3GPP TSG-SA WG2 Meeting #133	S2-1905576

13 - 17 May 2019, USA,

Source:	Orange

Title:	NF instance discovery

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes to provide guidelines on the discovery of NFs serving a given UE or group of UEs.

# Discussion

This discussion addresses the following topics:

- Discovery of UDM instance serving a given UE

- Discovery of AMF, SMF and MCF instances service an given UE

- Discovery of NEF and AF instances serving a given UE

A Discovery for the case of a single UE

The question is complexified due to:

- Multiple instances of the same NF

- NWDAF does not have a priori the same level of information then UEs or NFs directly in contact with the UEs.

Several techniques can be envisioned or even combined:

- Notification of a new or existing event (meaning “NF-x is serving this UE” in each NF): possible but with impact on signalling load if no service is requested for the UE to the NWDAF

- Use of stored information  in a repository instance: this is possible for AMF and SMF instances in UDM, and for PCF instances in BSF

- Use of stored information in NRF (ranges of SUPIs, group ID, routing indicator), for example to get acces to UDM instance or BSF instance

- Neighbour to neighbour explicit information passing (e.g. PCF => SMF => AMF) and finally in the NWDAF, typically the routing indicator (or the list of service instances)

- Broadcast one-shot subscription (equivalent of a request-response) of NWDAF to a subset of preselected NF instances in complement of S2, in order to determine the right instance.

- NWDAF consumer passes its own instance ID (NF Service Advertisement URI) as a first key (with the assumption that the consumer NF does serve the UE).

We propose to combine S1/S2/S3 and have S4 as a back-up solution.

Besides, document 23.501 and 23.502 should be updated.

B Discovery for the case of a groups of UE

The question of discovery of groups of UEs is left for further study.

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

### 6.2.2	Data Collection from NFs/AFs

#### 6.2.2.1	General

The Data Collection from NFs/AFs is used by NWDAF to subscribe/unsubscribe at any NF or AF to be notified for data on a set of events.

The Data Collection from NFs/AFs is based on the services of AMF, SMF, UDM, PCF, NRF and AF (possibly via NEF):

-	Event Exposure Service offered by each NF or AF as defined in TS 23.502 [3] clause 4.15 and clause 5.2.

-	other NF services (e.g. Nnrf_NFDiscovery and Nnrf_NFManagement in NRF as defined in TS 23.502 [3] clause 4.17)

This data collection service is used directly in order to retrieve behaviour data for individual UEs or groups of UEs (e.g. UE reachability), and also to retrieve global UE information (e.g. Number of UEs present in a geographical area).

Editor's note:	The Exposition Framework is likely to be extended depending on the analysis of use cases, with either:

-	new event-IDs in order to retrieve metrics  covering UE populations, not covered by OAM services, as described in TR 23.791 [13] (e.g. metrics per area on CM/RM states).

-	extensions of existing event-IDs and associated reports (e.g. for AMF  the "Number of UEs present in a geographical area" in order to obtain finer grain information (cells) than TA).

Table 6.2.2.1-1: NF Services consumed by NWDAF for data collection

| NF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- |
| AMF | Namf_EventExposure | 5.2.2.3 |
| SMF | Nsmf_EventExposure | 5.2.8.3 |
| PCF | Npcf_EventExposure | 5.2.5.7 |
| UDM | Nudm_EventExposure | 5.2.3.5 |
| NEF | Nnef_EventExposure | 5.2.6.2 |
| NRF | Nnrf_NFDiscovery | 5.2.7.3 |
|  | Nnrf_NFManagement | 5.2.7.2 |

NOTE:	How NWDAF collects data from UPF is not defined in this Release of the specification.

To retrieve data related to a specific UE, the NWDAF shall first discover which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE.

Table 6.2.2.1-2: NF Services consumed by NWDAF for NF discovery

| NF instance to discover whish serves the UE |  | NF to be contacted by NWDAF |  | Service |  | Reference in TS 23.502 [3] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UDM | NRF |  | Nnrf_NFDiscovery |  | 5.2.7.3 |  |
| AMF |  | UDM |  | Nudm_SubscriberDataManagement |  | 5.2.3.3 |  |
| SMF |  | UDM |  | Nudm_SubscriberDataManagement |  | 5.2.3.3 |  |
| BSF | NRF |  | Nnrf_NFDiscovery |  | 5.2.7.3 |  |
| PCF |  | BSF |  | Nbsf_Management |  | 5.2.13.2 |  |
| NEF |  | FFS |  |  |  |  |  |
| AF | FFS |  |  |  |  |  |

Editor’s note: the question of discovering  NEF and AF is left for further study (FFS).

When an NF instance that serves an UE requests a service to the NWDAF, it shall provide the Routing Indicator to the NWDAF in the request or subscription.

The UDM instance should  be discovered using NRF thanks to optional request parameters (e.g. SUPI ranges, NF group ID, routing indicator) as stated in clause 4.17.4 of  3GPP TS 23.502 [3].

The AMF, SMF instances should be discovered using an UDM request providing the UE ID.

The BSF instance should be discovered using NRF thanks to optional request parameters (e.g. DNN list, IP domain list, IPv4 address range) as stated in clause 4.17.4 of 3GPP TS 23.502 [3].

Editor’s note: the question of providing or provisioning these parameters to the NWDAF is left for further study.

The PCF instance should be discovered using a BSF request providing the UE addresses and UE ID.

Editor’s note: the question of providing or provisioning these parameters to the NWDAF is left for further study.

Editor’s note The question of discovery of groups of UEs is left for further study.

* * * Second change * * * *

### 6.1.3	Contents of Analytics Exposure

The NFs/AFs/ OAM subscribing to (using Nnwdaf_ AnalyticsSubscription) or requesting (using Nnwdaf_AnalyticsInfo) analytics to NWDAF may provide:

-	Analytics ID: one or multiple Analytics ID(s)

NOTE: Analytics ID(s) are also used by AFs subscribing to analytics information via NEF. NEF controls the mapping of Analytics IDs allowed to be exposed to AFs. Details in Clause 6.1.1.2 and 6.1.2.2.

-	Analytics Filter Information. This set of parameter types and values enables to select which type of analytics information is requested (e.g. subset of all available analytics produced by NWDAF for the given Analytics ID value).

-	Target of Analytics Reporting: the object targeted by analytics. The target of analytics reporting indicates entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

- 	Routing Indicator, as defined in 3GPP TS 23.502 [3]

-	Analytics Reporting Information with the following parameters:

-	Parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	Observation period: time interval [start..end], either in the past or in the future, expressed as positive or negative offsets to the present. An interval in the past is a request for statistics. An interval in the future is a request for predictions. A default value means statistics in the recent past, with a time range defined by the NWDAF.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	Time when analytics are needed (if applicable).

NOTE: The feasability of the parameter "Time when analytics are needed" will be checked by stage 3.

* * * End of changes * * * *