SA WG2 Meeting #133	S2-1905772

# May 13 - 17, 2019, Reno, NV, USA

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.725 | CR | 0038 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Key Issue #7: “Automatic GBR service recovery” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The response LS from RAN 3 in R3-191991 indicates that RAN 3 have not understood the need for a solution for this key issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The key issue is expanded to explain in more detail the problems that are likely to occur if the RAN does not adopt a different approach to admission control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reliability and availability of GBR services on 5GCore will not meet the requirements in TS 22.261. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>>>>>>>>>>>>>>>>>>>>>>>>>> Start of Changes <<<<<<<<<<<<<<<<<<<<<<<<

## 5.7	Key Issue #7: Automatic GBR service recovery after handover

### 5.7.1	General description

When a machine needs a Guaranteed Bit Rate service, it is likely to need that quality level in order to do its job. Hence if the QoS level cannot be maintained (e.g. handover into a congested cell; temporary cell overload due to higher priority services; etc), the QoS level should be restored as soon as possible, and, without a storm of signalling messages.

Release 15 5GC has partially solved this issue by introducing a "notification" mechanism that allows the core network to request the RAN to not release the radio bearer/QoS Flow when the QoS guarantee cannot be met, but, instead to notify the Core Network when the QoS drops, AND, when the RAN has restored the QoS level. This allows the controller of the machine (e.g. car/train) to adapt its behaviour (e.g. reduce speed) and ensures that the RAN continues to try to restore the QoS level while the device is in that cell.

However, at Xn or N2 handover, the target RAN node applies admission control and if it cannot support the required GBR QoS, the target RAN node does not establish that QoS Flow. In such a case, if, e.g. due to movement of the UE, the source RAN node has no choice but to handover to that target RAN node, then the target RAN node will complete the handover but will NOT notify the CN if and when the GBR QoS can be supplied to that UE.

As the machine needs to have its GBR service restored as soon as possible, the CN needs to repeatedly attempt to re-establish the GBR service. These re-attempts involve a considerable number of signalling messages, and are sent without any awareness of RAN congestion or potential link quality.

This is an inadequate solution for any mobile "non-human device" that needs to maintain a GBR data link during mobility (e.g. a car, a train, robots moving around a factory).

An additional problem associated with these retry attempts is that cyclic retries and “preemption invulnerability” leads to random prioritisation.

### 5.7.2	Cyclic retries leading to random prioritisation

For a machine related GBR bearer, in the Allocation/Retention Priority information, the Pre-emption Vulnerability is likely to be set to “not pre-emptable”.

The following figure 5.7.2-1 shows how the existing R15 specified handover behaviour can lead to undesirable consequences when cells are congested.

Figure 5.7.2-1: flow re-establishment following reduction in congestion

1a) 	UE 1 has a low priority, non-preemptable GBR flow established with its h-SMF.

1b) 	UE 2 has a high priority, non-preemptable GBR flow established with its h-SMF.

2)		Congestion occurs in a RAN node that is the target of handover requests.

3)		UE 1 needs to be handed over to the target RAN node.

4) 	The target RAN node admits all the UE’s flows except for the GBR flow.

5)		The signalling caused by the handover implicitly informs the h-SMF that the GBR flow has been released.

6)		UE 2 needs to be handed over to the target RAN node; the target RAN node admits all the UE’s flows except for the GBR flow; and the signalling caused by the handover implicitly informs the h-SMF that the GBR flow has been released.

7)		Both h-SMFs repeatedly try to re-establish their UE’s GBR flows. As the machines need the GBR flow to fulfil their ‘mission’, the retry mechanism can be expected to be aggressive.

8) 		Congestion reduces so that one more GBR flow can be admitted by the target RAN node.

9) 		By random chance, the next request received is for the Low priority GBR flow. The Low priority flow admitted. The cell is now “full” for GBR requests.

10)		a request for the high priority GBR flow is received, but is blocked because the low priority flow is non-preemptable.

The above type of behaviour is not restricted to handover situations but will also occur at “initial admission control for GBR-flows”.

************* end of changes ****************