3GPP TSG- SA WG2 Meeting #133	S2-1906746

# Reno, NV, USA, 13-17 May 2019											      (was 1906640, 1905399)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1393 | rev | 2 | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Context Transfer Support via CTSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Deutsche Telekom, Telecom Italia, Sprint, AT&T, Verizon, Telstra, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#132, Service Context Transfer procedures (S2-1904843) were apporved in TS23.502. CTSF shall support Service Context Transfer with any NF/NF service. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initial Rev - To add the new service in UDSF service Rev 1 - Use of CTSF for context data transfer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service Context Transfer is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.X(NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * First change * * * *

### 5.2.X	CTSF Services

#### 5.2.X.1	General

The following table illustrates the CTSF Services.

Table 5.2.X.1-1: NF services provided by CTSF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nctsf_ServiceContext | Query | Request/Response | Any NF |
|  | Push | Request/Response | Any NF |
|  | Delete | Request/Response | Any NF |
|  | Update | Request/Response | Any NF |

The following table shows the context data that may be transfered via the CTSF. The Data keys uniquely identify the context data that may be stored and retrieved from the CTSF.

:

Table 5.2.12.1-X: Context Data stored in the CTSF

| Category | Services | Description | Data key |
| --- | --- | --- | --- |
| SMContext (Node1) | SMF Context Transfer | Identifeis a Session Management Context that may be transfered from a Source to a target SMF via the CTSF | SUPI, PDU Session ID |
| Note 1: See Table 5.2.8.2.10-1 for single SM Context |  |  |  |

#### 5.2.X.2	Nctsf_ServiceContext service

#### 5.2.X.2.1	General

The Nctsf_ServiceContext service may be used by NF Consumers to store, update and delete its context data in the CTSF which can be retrieved by another NF Consumer of same type. The context data are identified by specific Data keys as defined in Table 5.2.12.1-X.

##### 5.2.X.2.2	Nctsf_ServiceContext_Query service operation

Service operation name: Nctsf_ServiceContext_Query.

Description: NF Service Consumer can query Service Context from CTSF.

Inputs, Required: Data Key(s)

Inputs, Optional: None.

Outputs, Required: Requested Service Context.

Outputs, Optional: None.

##### 5.2.X.2.2	Nctsf_ServiceContext_Push service operation

Service operation name: Nctsf_ServiceContext_Push.

Description: NF service consumer can push its Service Context into the CTSF.

Inputs, Required: Data Key(s), Service Context.

Inputs, Optional: None

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.X.2.3	Nctsf_ServiceContext_Delete service operation

Service operation name: Nctsf_ServiceContext _Delete.

Description: NF service consumer can delete Service Context in the CTSF.

Inputs, Required: Data Key(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.X.2.4	Nctsf_ServiceContext _Update service operation

Service operation name: Nctsf_ServiceContext _Update.

Description: NF service consumer can update Service Context in the CTSF.

Inputs, Required: Data Key(s), Service Context.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

* * * * End of Changes * * * *