3GPP TSG-SA2 Meeting #133	S2-1904952

# May 13 - 17, 2019, Reno, NV, USA

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1312 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing procedure for PLMN assigned Radio Capability ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2019-04-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the conlusion in 23.743, introducing support for PLMN assigned Radio Capability ID |  |  |  |  |  |  |  |  |
| Summary of change: |  | introduing producedure for allcation of PLMN assigned UE Radio Capability ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PLMN assigned Radio Capability ID is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 4.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

4.2.X Procedures for PLMN assigned UE Radio Capability ID

4.2.X.1 Allocation of PLMN assigned UE Radio Capability ID

Figure 4.2.X.1-1: Allocation of PLMN assigned UE Radio Capability ID

1. 	The (R)AN retrieves the UE radio capabilities set from the UE by RRC UE Capability Enquiry, if

- there is neither UE Radio Capability ID nor UE radio capability information, in the Initial Context Setup Request message which includes the Registration Accept in step 21 of figure 4.2.2.2.2-1, or in the N2 Request in step12 of figure 4.2.3.2-1;

- the filter configured at  (R)AN is different from the filter in the received UE radio capability information, etc.

2. 	The UE provides the (R)AN with its UE radio capabilities by sending the RRC UE Capability Information.

3. 	The (R)AN provides the new AMF with the UE radio capabilities set by sending the N2 UE Capability Information Indication message.

4. 	If the UE supports RACS, the new AMF retrieves the PLMN assigned UE Capbiliy ID using Nucmf_UECapabilityManagement_Retrieve service operation. The service operation includes the UE radio capabilities set received from the (R)AN. If the UE doesn’t support RACS, the new AMF may also retrieve the PLMN assigned UE Radio Capability ID using Nucmf_UECapabilityManagement_Retrieve service operation.

Upon reception of the UE radio capabilities set, the UCMF returns to the new AMF the PLMN assigned UE Radio Capability ID corresponding to the received UE radio capabilities set. The UCMF may firstly allocate a PLMN assigned UE Radio Capability ID for the received UE radio capabilities set if there is no existing UE Radio Capability ID corresponding to the received UE radio capabilities set. For the new allocated PLMN assigned UE Radio Capability ID, the UCMF shall store the mapping between the PLMN assigned UE Radio Capability ID and the UE radio capabilities set.

5. 	Upon reception of the PLMN assigned UE Radio Capability ID, if the UE supports RACS, the new AMF sends to the (R)AN the N2 request, including NAS PDU and the PLMN assigned UE Radio Capability ID. The NAS PDU is UE Configuration Update Request includsing the PLMN assigned UE Radio Capability ID. If the UE doesn't support RACS, the new AMF may sends the PLMN assigned UE Radio Capability ID to the (R)AN without NAS PDU to notify the UE.

6. 	The (R)AN sends the received NAS PDU to the UE.

7. 	The UE responses the new AMF the UE Configuration Update Complete message.

<<< End of changes >>>