SA WG2 Meeting #133	S2-1906474

13 – 17 May, 2019, Reno, Nevada, USA	(was S2-1905455)

Source:	CATT

Title:	Solve FFS related to NI-LR Procedures when UE is served by the different PLMNs via 3GPP access and non-3GPP access

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: This paper proposes to solve the FFS in clause 6.9.4.

1.	Discussion

This pCR is proposed to solve the following editor’s note in clause 6.9.4 in TS 23.273:

Editor's note:	How the progress of the study (e.g. network exposure for eLCS, MO-LR Procedure, etc) impact the procedures to support Non-3GPP access are FFS.

When UE is served by the different PLMNs via 3GPP access and non-3GPP access, the UE selects one access to register to the 5GC for emergency services or request the establishment of a PDU Session related to an applicable regulatory service (e.g., emergency session initiation). In order to avoid LMF to perform access selection in NI-LR procedure, it is proposed that AMF provides access type selected by UE to LMF. LMF shall use the access type provided by AMF for positioning.

2.	Proposal

It is proposed to update TS 23.273 as follows.

*** First change ***

6.9.4	NI-LR Procedures when UE is served by the Different PLMNs via 3GPP access and non-3GPP access

When UE is served by the different PLMNs via 3GPP access and non-3GPP access, the UE selects one access to register to the 5GC for emergency services as defined in TS 23.167 [10], or request the establishment of a PDU Session related to an emergency session.

The NI-LR procedures are the same as the 5GC-NI-LR Procedure (described in clause 6.10.1) with the difference:

-	If the procedures are performed in non-3GPP access, NG-RAN in Figure 6.10.1 is corresponding to N3IWF/TNGF and the UE Positioning in step 3 in Figure 6.10.1 is corresponding to step 3d or 3e in Figure 6.9.1-1.

-	In step 2 in 5GC-NI-LR Procedure in clause 6.10.1, AMF also provides access type to LMF. LMF shall use the access type provided by AMF for positioning.

*** End of change ***