3GPP TSG-SA WG2 Meeting #134	S2-1907412

Sapporo, Japan, 24 - 28th June 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1587 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PMF message delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  |  | Date: |  | 2019-06-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed that when the UE goes to CM-IDLE on an access, no PMF-Echo Request messages are sent on this access. However it doesn’t cover the case that when the UE is still in CM-CONNECTD mode but the user plane resource of the PDU session is released on an access, the PMF-Echo shall also not sent on this access. How to stop sending the PMF-Echo Request message in the PMF in UPF is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify when and how to stop sending the PMF Echo Request message on one access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how and when to stop sending the PMF-Echo Request message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.32.5.2	Round Trip Time Measurements

RTT measurements can be conducted by the UE and UPF independently. There is no measurement reporting from one side to the other.

The estimation of the RTT by the UE and by the UPF is based on the following mechanism:

1.	The PMF in the UE sends over the user plane PMF-Echo Request messages to the PMF in the UPF, and the PMF in the UPF responds to each one with a PMF-Echo Response message. Similarly, the PMF in the UPF sends over the user plane PMF-Echo Request messages to the PMF in the UE, and the PMF in the UE responds to each one with a PMF-Echo Response message.

2.	In the case of a MA PDU Session of IP type:

-	The PMF in the UE sends PMF messages to the PMF in the UPF over IP. The destination IP address is the IP address contained in the Measurement Assistance Information and the destination port is one of the two ports contained in the Measurement Assistance Information. One port is used for sending PMF messages to UPF over 3GPP access and the other port is used for sending PMF messages to UPF over non-3GPP access. The source IP address is the IP address assigned to UE for the MA PDU Session and the source port is a port that is dynamically allocated by the UE for PMF communication. This source port in the UE remains the same for the entire lifetime of the MA PDU Session.

-	The PMF in the UPF sends PMF messages to the PMF in the UE over IP. The source IP address is the same IP address as the one provided in the Measurement Assistance Information and the source port is one of the two ports as provided in the Measurement Assistance Information. One port is used for sending PMF messages to UE over 3GPP access and the other port is used for sending PMF messages to UE over the non-3GPP access. The destination IP address is the IP address assigned to UE for the MA PDU Session and the destination port is the dynamically allocated port in the UE, which is contained in all PMF messages received from the UE.

Editor's note:	Whether the PMF messages are exchanged over TCP/IP or over UDP/IP will be specified by Stage 3.

3.	In the case of a MA PDU Session of Ethernet type:

-	The PMF in the UE sends PMF messages to the PMF in the UPF over Ethernet. The Ethertype is the Ethertype contained in the Measurement Assistance Information and the destination MAC address is one of the two MAC addresses contained in the Measurement Assistance Information. One MAC address is used for sending PMF messages to UPF over 3GPP access and the other MAC address is used for sending PMF messages to UPF over non-3GPP access. The source MAC address is a MAC address of the UE, which remains the same for the entire lifetime of the MA PDU Session.

-	The PMF in the UPF sends PMF messages to the PMF in the UE over Ethernet. The Ethertype is the same Ethertype as the one provided in the Measurement Assistance Information and the source MAC address is one of the two MAC addresses as provided in the Measurement Assistance Information. One MAC address is used for sending PMF messages to UE over 3GPP access and the other MAC address is used for sending PMF messages to UE over non-3GPP access. The destination MAC address is the MAC address of the UE, which is contained in all PMF messages received from the UE.

4.	When the user plane resource is released on an access, no PMF-Echo Request messages are sent on this access. When the user plane resource is released on an access, the SMF is updated and it notifies the PMF in the UPF to stop sending the PMF-Echo Request on this access.

5.	The PMF messages exchanged between the UE and UPF shall use the default QoS flow over both 3GPP and non-3GPP accesses.

6.	The UE and the UPF derive an estimation of the average RTT over an access type by averaging the RTT measurements obtained over this access.

.

* * * * End of Changes * * * *