3GPP TSG-SA WG2 Meeting #134	S2-1907808

# Sapporo, Japan, June 24 – 28, 2019											  (revision of S2-1907574)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1611 | rev | 1 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EPS interworking for MA PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EPS interworking is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a description on EPS interworking for MA PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EPS interworking is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.22.6	EPS Interworking

4.22.6.1.	General

This clause includes procedures for interworking with EPS based on N26 interface.

4.22.6.2. Impacts to EPS interworking procedures

4.22.6.2.1	5GS to EPS handover using N26 interface

Based on the signaling flow in Figure 4.11.1.2.1-1, the procedure is performed with the following differences and modifications:

-	In step 2, this step is also performed with all the PGW-C+SMFs corresponding to MA PDU Sessions and have EBI(s) allocated to them.

-	In step 12e, the AMF also requests the release of MA PDU Session, not expected to be transferred to EPC, i.e. no EBI(s) allocated to them.

-	In step 16, if the MA PDU Session is established in non-3GPP access and the MA PDU Session is moved to EPS, the SMF triggers MA PDU Session Release procedure over non-3GPP access.

4.22.6.2.2	5GS to EPS idle mode mobility using N26 interface

Based on the signaling flow in Figure 4.11.1.3.2-1, the procedure is performed with the following differences and modifications:

-	In step 5a, this step is also performed with all the PGW-C+SMFs corresponding to MA PDU Sessions and have EBI(s) allocated to them.

-	In step 12, if the MA PDU Session is established in non-3GPP access and the MA PDU Session is moved to EPS, the SMF triggers MA PDU Session Release procedure over non-3GPP access.

-	In step 15a, the AMF also requests the release of MA PDU Session, not expected to be transferred to EPC, i.e. no EBI(s) allocated to them.

4.22.6.2.3	EPS bearer ID allocation

Based on the signaling flow in Figure 4.11.1.4.1-1, additionally for MA PDU Session, with the following differences and clarifications:

-	In step 1, the following procedures and relevant steps are also initiated during the UE Requested MA PDU Session Establishment, the UE Requested PDU Session Establishment with Network Modification to MA PDU Session and the UE or network requested MA PDU Session Modification procedures.

-	In step 2, if the MA PDU Session is established over 3GPP access and other existing conditions satisfies EPS interworking, the SMF requests EBI allocation for the QoS flow(s) of the MA PDU Session.

4.22.6.2.4	EPS bearer ID revocation

Based on the clause 4.11.1.4.3, additionally the following procedures are updated to revoke the EPS bearer ID(s) assigned to the QoS flow(s) in MA PDU Session:

-	UE or network requested MA PDU Session Release (non-roaming and roaming with local breakout) in clause 4.22.10.2.

-	UE or network requested MA PDU Session Release (home-routed roaming) in clause 4.22.10.3.

-	UE or network requested MA PDU Session Modification (non-roaming and roaming with local breakout) in clause 4.22.8.2.

-	UE or network requested MA PDU Session Modification (home-routed roaming) in clause 4.22.8.3.

-	When the MA PDU Session is released over 3GPP access, the UE and the SMF locally release the EBI(s) for the MA PDU Session. The SMF notifies the AMF of the released EBI(s) by sending Nsmf_PDUSession_SMContextStatusNotify service operation if the MA PDU Session is established in the same PLMN. If the MA PDU Session is established in different PLMNs, the SMF notifies the release of the MA PDU Session and as a result, the AMF removes associated EBI(s).

4.22.6.3. Network Modification to MA PDU Session after moving from EPC

Figure 4.22.6.3-1. Network Modification to MA PDU Session after moving from EPC

1.	When the network supports interworking with N26 interface, a PDN Connection can be moved from EPC to 5GC as described in clause 4.11.1.2.2 and clause 4.11.1.3.3.

2.	If no policy in the UE (e.g. no URSP rule) and no local restrictions mandate a single access for the PDU Session, the UE provides ATSSS Capability to the network by triggering UE requested PDU Session Modification procedure over 3GPP access as described in clause 4.22.8 with following modifications:

-	In step 3a, if the SMF decides to change the PDU Session to MA PDU Session, the SMF includes ATSSS rule(s) in the PDU Session Modification Command message. The SMF may also include Measurement Assistance Information.

-	In step 5, if the UE receives ATSSS rule(s) in the PDU Session Modification Command message, the UE stores it and stores that the PDU Session is MA PDU Session.

3.	After PDU Session Modification procedure is completed, the SMF triggers Nsmf_PDUSession_SMContextStatusNotify service operation to notify that the PDU Session is MA PDU Session. The AMF stores the Access Type of the PDU Session to "Multi-Access".

4.	The UE triggers UE requested PDU Session Establishment over non-3GPP access to add second access to the MA PDU Session.

* * * * End of Changes * * * *