3GPP TSG-SA2 Meeting #134	S2-1908446

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1481 | rev | 2 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to downlink data delivery status event to support PCC and HR roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA2 has received LS S2-1904878 from CT3. This LS asks with respect to the "Information flow for downlink data delivery status" in subclause 4.15.3.2.5 of TS 23.502 how the SMF detects the downlink delivery status. A related discussion paper is provided in S2-1906945. There are several related issues with the present information flow: - Packet filters related with downlink data delivery status imply UPF interactions (N4 impacts) to configure the UPF to buffer packets or forward them to the SMF and to provide notifications about the downlink data delivery status when the first packet was transmitted or is being buffered and subsequently discarded. Related procedural updates are provided in TS 23.501, see S2-1906946. - Traffic filters in the UPF have priorities that need to be coordinated with other installed traffic filters. This is controlled by the PCF via PCC rules. The subscription for downlink data delivery status thus also needs to be integrated in PCC rules. The subscription to the Nsmf_EventExposure notification is thus indirect via the Npcf_SMPolicyControl service. - Subscription is via the PCF, but PCC should be optional to use. - For HR roaming, the SMF that is aware of data being buffered is the V-SMF and the notifications corresponding to data being buffered originate from a V-UPF in the VPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Interactions of the SMF with the UPF for subscribing to downlink data delivery status are added to subclause 4.15.3.2.5. - Subscription is via UDM. However, if PCC is used, the PCF is notified about the subscription. Then, the subscription to the Nsmf_EventExposure notification is indirect via PCC rule information of the Npcf_SMPolicyControl service and comes from the PCF rather than the UDM. - For the home routed roaming case, the V-SMFF and an attached UPF detect downlink data delivery status events and the V-SMF sends related notifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The detection of downlink data delivery status event does not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.5, 5.2.8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-1: Changes from S2-1907493 incorporated: Notification only relate to the first packets |  |  |  |  |  |  |  |  |

1st Change

##### 4.15.3.2.5	Information flow for downlink data delivery status

The procedure is used by the Application Function to subscribe to data delivery status notifications and to explicitly cancel a previous subscription.. The data delivery status notifications relates to high latency communication, see also subclauses 4.24.2 and 4.2.3.3.

Editor´s note: The procedures below assume that buffering at the SMF is used and that PCC is not used. Buffering at the UPF, PCC control, and how to then harmonize traffic filters obtained within the Nsmf_EventExposure_Subscribe with possible traffic filters controlled by PCC are ffs.

Figure 4.15.3.2.5-1: Information flow for downlink data delivery status

0.	At PDU session establishment, the SMF (in the no-roaming case the H-SMF. in the roaming case the V-SMF) configures the relevant UPF to forward packets that are subject to extended buffering to the SMF. The SMF decides to apply this behavior based on the "expected UE behaviour".

1.	AF sends Nnef_EventExposure_Subscribe Request to NEF requesting data delivery status for a UE or group of UEs. Traffic filter information is included in the message to identify the source of the downlink IP or Ethernet traffic. The Downlink delivery status events include:

-	First downlink Packet(s) in extended buffering event.

-	This event is triggered when the first new downlink data packet is buffered with extended buffering matching the traffic filter information.

-	in notifications about this Downlink delivery status, the SMF provides the Extended Buffering time as determined in clause 4.2.3.3.

-	First downlink Packet discarded.

-	This event occurs when the first packet matching the traffic filter information is discraded because the Extended Buffering time, as determined by the SMF, expires or the amount of downlink data to be buffered is exceeded.

-	First Downlink Packet transmitted.

-	This event occurs when the first packet matching the traffic filter information is transmitted after previous buffering or discarding of corresponding packet(s) because the UE of the PDU Session becomes ACTIVE, and buffered data can be delivered to UE as per clause 4.2.3.3.

2.	NEF sends the Nudm_EventExposure_Subscribe Request to UDM. Identifier of the UE or group of UEs, traffic filter information, monitoring event received AF at step 1, and NEF notification endpoint are included in the message.

3.	UDM sends the Nsmf_EventExposure_Subscribe Request message to each SMF where at least one UE identified in step 2 is registered and includes identifier of the UE or group of UEs, traffic filter information, monitoring event and the notification endpoint of NEF received in step 2. If the UDM becomes aware that such a UE is attached at another SMF at a later time than when receiving step 2, the UDM then executes step 3.

4.	SMF sends the Nsmf_EventExposure_Subscribe Response message to UDM.

5.	UDM send sends the Npcf_EventExposure_Subscribe response message to NEF.

6.	NEF sends the Nsmf_EventExposure_Subscribe response to AF.

7.	The SMF or UPF detects a change in Downlink Delivery Status event as described in clause 4.2.3. The SMF becomes aware that Downlink Packet(s) require extended buffering when trying to send packets to the AMF via a Namf_Communication_N1N2MessageTransfer service operation and obtaining a corresponding respone. If the the SMF decides to discard packets, the "Downlink Packet(s) discarded event" is detected. The SMF detects that previously buffered packets can be transmitted by the fact that the related PDU session becomes ACTIVE.

8.	The SMF sends the Nsmf_EventExposure_Notify with Downlink Delivery Status event message to NEF.

9.	The NEF sends Nnef_EventExposure_Notify with Downlink Delivery Status event message to AF.

2nd Change

##### 5.2.8.3.1	General

Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on PDU Sessions. The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for an Event ID on PDU Session(s); and

-	Notifying events on the PDU Session to the subscribed NFs.

-	Allow consumer NFs to acknowledge or respond to an event notification.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE IP address / Prefix change: The event notification may contain a new UE IP address / Prefix or an indication of which UE IP address / Prefix has been released.

-	PDU Session Release.

-	UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed.

The event notification may contain following information:

-	the type of notification ("EARLY" or "LATE").

-	for both the source and target UP path between the UE and the DN, the corresponding information is provided when it has changed:

-	DNAI.

-	UE IP address / Prefix.

-	N6 traffic routing information.

NOTE 1:	UP path change notification, DNAI and N6 traffic routing information are further described in TS 23.501 [2] clause 5.6.7.

-	Change of Access Type; The event notification contains the new Access Type for the PDU Session.

-	PLMN change; The event notification contains the new PLMN Identifier for the PDU Session.

-	Downlink data delivery status. The event notification contains the status of downlink data buffering in the core network including:

-	First downlink packet per AF traffic filter in extended buffering and Estimated maximum wait time.

-	First downlink packet per AF traffic filter discarded.

-	First downlink packet per AF traffic filter transmitted after previous buffering and/or discarding of corresponding packet(s).

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for SMF exposure.

Table 5.2.8.3.1-1: Example of Event Filters for SMF exposure events

| Event ID for SMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| DNAI Change | None |
| PDU Session Release |  |

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI) an Internal Group Identifier or an indication that any UE is targeted (on a specific DNN).

When acknowledgment is expected the SMF also provides Notification Correlation Information to the consumer NF in the event notification.

The consumer NF may provide the following event-specific information when acknowledging an event notification:

-	For UP path change event:

-	N6 traffic routing information related to the target DNAI.

NOTE 2:	Acknowledgement to a UP path change event notification is further described in TS 23.501 [2] clause 5.6.7.

End of Changes