3GPP TSG-SA WG2 Meeting #134	S2-1906955

# Sapporo, Japan, June 24 - 28, 2019 	(revision of S2-19xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0009 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Features for W-AGF to act on behalf of FN-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To provide clear description of features/functionalities to be implemented in the W-AGF to act on behalf of FN-RG as a UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description about how W-AGF acts on behalf of FN-RG as a UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clear description of which feaures/functionalities to be implemented in W-AGF to act on behalf of FN-RG |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes ****

## 4.X	Support of FN-RG

### 4.X.1	General

FN-RG is a legacy type of residential gateway that does not support N1 signalling and is not 5GC capable. The architecture to support FN-RG is depicted in clause 4.2.8.4 in TS 23.501 [2]. Support for FN-RG connectivity to 5GC is provided by means of W-AGF supporting 5G functionality on behalf of the FN-RG, e.g. UE NAS registration and session management functionality. In particular, the W-AGF supports the following functionality on behalf of the FN-RG:

-	Acting as end-point of N1 towards AMF, including maintaining CM and RM states

-	Keeps permanent configuration information and dynamic data for each FN-RG. This could for example include AN parameter to be used for the FN-RG and a list of PDU sessions to be created on behalf of the FN-RG

- 	Mapping between Y5 towards FN-RG and N1/N2 towards 5GC as well as mapping between a Y5 user plane connection and a PDU Session user plane tunnel on N3.

Authentication of FN-RG is done by the W-AGF, as defined by BBF. The W-AGF provides an indication on N2 that the FN-RG has been authenticated and a SUCI or a 5G-GUTI if the FN-RG has been registered in the 5GC previously. Since no authentication is done in 5GC, there are no keys derived for the FN-RG neither for AS security nor for NAS security. This means that NULL NAS integrity protection algorithm and ciphering algorithm must be used.

**** End of Changes ****