SA WG2 Meeting #134	S2-1907530

# June 24 – 28, 2019, Sapporo, Japan 	(revision of S2-19yyyyy)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1617 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to TSN Synchronization solution for compliance with 802.1AS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR corrects issues related to 802.1AS compliancy, including rate ratio compensation for residence time and compensating propagation delay from previous gPTP entity. This CR clarifies provision of 5G clock from UE to DS-TT for time stamping in time stamping unit (TSU) and use of TSN clock for time aware shaping (TAS). With the current sync method, when 5G GM is used as the TSN clock, then there are void gPTP sync messages carried through U-plane, because ingress time stamp (Tsi) from NW-TT is not needed in DS-TT to determine the correction value for outgoing gPTP sync message. DS-TT can use equation: Correction = TSe - Origin timestamp. An option is to consider an optimization by avoiding to transmit void gPTP sync messages from NW-TT to DS-UE through U-plane. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rate ratio can be estimated by NW-TT and signalled to DS-TT in existing cumulative_rate_ratio IE of sync message. DS-TT estimates residence time by multiplying difference of 5G specific timestamps with the rate ratio. Propagation delay from previous gPTP entity can be estimated by NW-TT with gPTP delay measurement procedure. The neighbour rate ratio estimate can be used to compensate error introduced by difference in frequencies of clock. NW-TT can add that value to the existing correction field of gPTP message, instead of combining with TSi resulting an error in residence time calculation. To ensure precision of delivered time, provision of 5G time from UE to DS-TT can exploit physical layer signalling, e.g., commonly used pulse per second PPS together with second level time of day ToD. To avoid transmitting void gPTP messages between TTs when 5G GM is as TSN clock, the time of generation for periodic origin timestamps in 5GS should be made available in DS TT. Then DS TT could generate gPTP sync messages without void user plane gPTP flow from NW-TT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSN time synchronization solution is not compliant with IEEE802.1AS and TS22.104, e.g. it does not tolerate frequency error in external clocks and does not compensate propagation delay from previous PTP entity correctly. Solution does not enable efficient use of 5G internal clock as TSN clock source, when void sync messages are carried over Uu U-plane. Provision of 5G clock from UE to different purposes in DS-TT may remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.1, 5.27.1.2, 5.27.1.3, 5.27.1.4 added. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.27	Time Sensitive Communications

### 5.27.0	General

This clause describes 5G System features that support TSC and allow the 5G System to be integrated transparently as a bridge in an IEEE TSN network.

### 5.27.1	TSN Time Synchronization

#### 5.27.1.1	General

For supporting TSN time synchronization, the 5GS is integrated with the external network as a TSN bridge as described in clauses 4.4.8 and 5.28.1. It shall be modelled as an IEEE 802.1AS [104] compliant entity according to TS 22.104 [105]. For TSN Synchronization, the entire E2E 5G system can be considered as an IEEE 802.1AS [104] "time-aware system". Only the TSN Translators (TTs) at the edges of the 5G system need to support the IEEE 802.1AS [104] operations. UE, gNB, UPF, NW-TT and DS- TTs are synchronized with the 5G GM (i.e. the 5G internal system clock) which shall serve to keep these network elements synchronized. The TTs located at the edge of 5G system fulfil all functions related to IEEE 802.1AS [104], e.g. (g)PTP support, timestamping, Best Master Clock Algorithm (BMCA), rate ratio compensations and link delay measurement. Figure 5.27.1-1 illustrates the 5G and TSN clock distribution model via 5GS.

Figure 5.27.1-1: 5G system is modelled as IEEE 802.1AS compliant time aware system for supporting TSN time synchronization with external TSN GM

Figure 5.27.1-1 depicts the two synchronizations systems considered: the 5GS synchronization and the TSN domain synchronization, as well as the Master (M) and Slave (S) ports considered when the TSN GM is located at TSN working domain.

-	5GS synchronization: Used for NG RAN synchronization. 5G RAN synchronization is specified in TS 38.331 [28].

-	TSN domain synchronization: Provides synchronization service to TSN network. This process follows IEEE 802.1AS [104].

The two synchronization processes can be considered independent from each other and the gNB only needs to be synchronized to the 5G GM clock.

To enable TSN synchronization, the 5GS calculates and adds the measured and rate ratio corrected residence time between the TTs and link propagation delay from the previous gPTP corrected with measured neighbor rate ratio into the Correction Field (CF) of the synchronization packets of the TSN domain.

#### 5.27.1.2	Distribution of timing information

##### 5.27.1.2.1	Distribution of 5G internal system clock

The 5G internal system clock shall be made available to all user plane nodes in the 5G system. The UPF and NW-TT may get the 5G internal system clock via the underlying PTP compatible transport network with mechanisms outside the scope of 3GPP. The 5G internal system clock shall be made available to UE with signaling of time information related to absolute timing of radio frames as described in TS 38.331 [28]. The 5G internal system clock shall be made available to DS-TT by the UE, e.g., as depicted below in Figure 5.27.1-2. For example, physical pulse per second (PPS) signal and second level Time of Day (ToD) can be exploited to transfer precise time information from UE to DS-TT.

Figure 5.27.1-2 illustrates difference between 5G clock used for time stamping unit (TSU) and TSN clock for possible time aware shaping (TAS).

Figure 5.27.1-2: Example for provision of 5G system time to DS-TT

##### 5.27.1.2.2	Distribution of TSN clock and time-stamping

For the downlink, when gPTP frames from TSN working domain(s) arrive at the NW-TT, the NW-TT makes an ingress timestamping (TSi) for each gPTP event (sync) message. The TSi can be carried using suffix field of the gPTP sync or related sync follow-up message together with the original TSN timing information. NW-TT sums the link delay from the previous entity into the correction field. In case of one step sync operation, it is included in sync message and in case of two step sync operation, it is included in the sync Follow up message. The cumulative rate ratio field in is also updated by NW-TT according to estimated cumulative rate ratio between TSN GM and 5G GM.

1-step sync is an optional 802.1AS feature. Modifying 1-step sync messages with a residence time, rate ratios and propagation delay from previous gPTP entity may take variable time e.g. due to CPU load of TT, so 2-step sync (sync with sync follow up) provides likely better accuracy and which shall be supported.

UPF then forwards the gPTP message to the UE via user plane (i.e. using the PDU session applicable for sending gPTP messages). Only one PDU session per UE per UPF is used for sending gPTP messages regardless of how many external TSN working domains have their clock information delivered through a given UPF serving that UE.

Editor's note:	It is FFS whether gPTP messages are forwarded on a distinct QoS Flow or other mechanism. If distinct QoS Flow is used it is FFS whether all gPTP messages are forwarded on it, or only a subset of the gPTP messages (e.g. those that carry information in the Suffix field).

A UE receives the gPTP messages and forwards them to the DS-TT. The DS-TT then performs egress timestamping (TSe) for the gPTP messages. The difference between TSi and TSe multiplied with the cumulative rate ratio is considered as the calculated residence time spent within the 5G system for this gPTP message. The DS-TT modifies the correction field of gPTP messages it sends towards an end station to include the calculated residence time, where the link delay from the previous gPTP entity was already included by NW-TT in the received correction field.

#### 5.27.1.3	Support for multiple TSN working domains

Each TSN working clock domain sends its own gPTP messages. The related Ethernet frames carry the gPTP multicast Ethernet destination MAC address and the gPTP message carries a specific PTP "domainNumber" that indicates the time domain they are referring to. The NW-TT makes ingress timestamping (TSi) for the gPTP event messages of all domains.

NOTE 1:	An end-station can select TSN timing information of interest based on the "domainNumber" in the gPTP message.

The process described in "Distribution of TSN clock and time-stamping" is thus repeated for each TSN working domain between a UE and the UPF it is connected to. When a UE is connected to multiple TSN working domains via the same UPF, then all gPTP messages from these domains are forwarded to the UE via the same PDU session.

NOTE 2:	If all TSN working domains can be made synchronous and the synchronization can be provided by the 5G clock, the TT output ports at the UPF and UE side (NW-TT and DS-TT) towards the connected TSN networks propagate the 5G clock using the 802.1AS profile (i.e. the 5G system as an IEEE 802.1AS [104] compliant time-aware system).

##### 5.27.1.4	Use of 5G GM as TSN clock

Use of 5G GM for TSN applies functionalities related to GM capable gPTP entity, where NW-TT and DS-TT act both as master port transmitting gPTP synch messages. When NW TT forwards 5G time towards DS-TT, DS-TT does not need TSi timestamp, rate ratio or propagation delay information because value in correction field (CF) equals to the difference between TSe and the origin timestamp (CF = TSe – origin timestamp).

To avoid transmitting void gPTP messages between TTs, DS TT could generate gPTP sync messages without void user plane gPTP flow from NW-TT. In this case, the time of generation for periodic origin timestamps in 5GS should be made available in DS TT.

Annex H (normative):
Signalling of ingress time for time synchronization

The ingress time and cumulative rate ratio field (as explained in clause 5.27.1) are provided from the NW-TT/UPF to the DS-TT/UE as part of a gPTP Sync message using the Suffix field defined in clause 13.4 of IEEE 1588-2008 [107]. The structure of the Suffix field follows the recommendation of clause 14.3 of IEEE 1588-2008 [107], with an organizationId specific to 3GPP, an organizationSubType referring to an ingress timestamp, and data field that can carry the originTimestamp datatype of the Sync message defined in clause 13.6.1 of IEEE 1588-2008 [107].

Editor's note:	TS XX.XXX will provide the specification of the fields in the gPTP Sync message.

END OF CHANGES