3GPP TSG-SA WG2 Meeting #134	S2-1907570

# Sapporo, Japan, June 24 – 28, 2019											  (revision of S2-190XXXX)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1609 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MA PDU Session Establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. It is not clear "MA PDU Request" indication and "ATSSS Capability" is included in the UL NAS Transport or PDU Session Establishment. 2. How the AMF stores Access Type of the MA PDU Session is not described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify that "MA PDU Request" indication is sent in the UL NAS Transport and "ATSSS Capability" is sent in the PDU Session Establishment Request. 2. In case of UE Requested MA PDU Session, the AMF stores Access Type as "Multi-Access" when it received UL NAS Transport message. 3. In case of UE Requested PDU Session with Network Modification, the AMF stores Access Type as "Multi-Access" when the SMF notifies that the PDU Session is a MA PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.2.1, 4.22.2.2, 4.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.22.2	UE Requested MA PDU Session Establishment

4.22.2.1	Non-roaming and Roaming with Local Breakout

The signalling flow for a MA PDU Session establishment when the UE is not roaming, or when the UE is roaming and the PDU Session Anchor (PSA) is located in the VPLMN, is based on the signalling flow in Figure 4.3.2.2.1-1 with the following differences and clarifications:

-	The PDU Session Establishment Request message may be sent over the 3GPP access or over the non-3GPP access. In the steps below, it is assumed that it is sent over the 3GPP access.

-	In step 1, the UE provides a "MA PDU Request" indication in UL NAS Transport message and an ATSSS Capability (e.g. an "MPTCP Capability" and/or an "ATSSS-LL Capability"), as defined in TS 23.501 [2], clause 5.32.2 (Multi Access PDU Sessions) in PDU Session Establishment Request message.

The "MA PDU Request" indication and the Request Type = "initial request" indicate to the network that this PDU Session Establishment Request is to establish a new MA PDU Session and to apply the ATSSS-LL functionality, or the MPTCP functionality, or both functionalities, for steering the traffic of this MA PDU session.

If the UE requests an S-NSSAI and the UE is registered over both accesses, it shall request an S-NSSAI that is allowed on both accesses.

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects an SMF, which supports MA PDU sessions. The AMF stores Access Type of the PDU Session as "Multi-Access".

-	In step 3, the AMF informs the SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication) and, in addition, it indicates to SMF whether the UE is registered over both accesses. If the AMF determines that the UE is registered via both accesses but the requested S-NSSAI is not allowed on both accesses, then the AMF shall reject the MA PDU session establishment.

-	In step 7, the SMF sends an "MA PDU Request" indication to the PCF in the SM Policy Control Create message. The PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

The PCF provides PCC rules for the MA PDU session, i.e. PCC rules that include ATSSS policy control information, as specified in TS 23.503 [20]. From the received PCC rules, the SMF derives (a) ATSSS rules, which will be sent to UE for controlling the traffic steering, switching and splitting in the uplink direction, and (b) N4 rules, which will be sent to UPF for controlling the traffic steering, switching and splitting in the downlink direction. If the UE indicates the support of "ATSSS-LL Capability", the SMF may derive the Measurement Assistance Information.

-	In the remaining steps of Figure 4.3.2.2.1-1, the SMF establishes the user-plane resources over the 3GPP access, i.e. over the access where the PDU Session Establishment Request was sent on:

-	In step 10, the N4 rules derived by SMF for the MA PDU session are sent to UPF. If the ATSSS Capability for the MA PDU Session indicates "ATSSS-LL Capability", the SMF may include information for measurement into the N4 rule to instruct the UPF to initiate performance measurement for this MA PDU Session. In step 10a, the UPF allocates addressing information for the Performance Measurement Function (PMF) in the UPF. In step 10b, the UPF sends the addressing information for the PMF in the UPF to the SMF.

-	In step 13, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by SMF. If the ATSSS Capability for the MA PDU Session indicates "ATSSS-LL Capability", the SMF may include the addressing information of PMF in the UPF into the Measurement Assistance Information.

-	After step 18 in Figure 4.3.2.2.1-1, if the SMF was informed in step 2 that the UE is registered over both accesses, then the SMF initiates the establishment of user-plane resources over non-3GPP access too. The SMF sends an N1N2 Message Transfer to AMF including N2 SM Information and indicates to AMF that the N2 SM Information should be sent over non-3GPP access. The N1N2 Message Transfer does not include an N1 SM Container for the UE because this was sent to UE in step 13.

The last step above is not executed when the UE is registered over one access only, in which case the MA PDU Session is established with user-plane resources over one access only. How user-plane resources can be added over an access of the MA PDU Session is specified in clause 4.22.7 below.

4.22.2.2	Home-routed Roaming

When the UE is registered to the same VPLMN over 3GPP access and non-3GPP access, the MA PDU Session is established as specified in Figure 4.3.2.2.2-1 ("UE-requested PDU Session Establishment for home-routed roaming scenarios") with the differences and clarifications:

-	In step 1, the UE provides a "MA PDU Request" indication in UL NAS Transport message and an ATSSS Capability (e.g. an "MPTCP Capability" and/or an "ATSSS-LL Capability"), as defined in TS 23.501 [2], clause 5.32.2 (Multi Access PDU Sessions) in PDU Session Establishment Request message.

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects a V-SMF and an H-SMF, which supports MA PDU sessions. The AMF stores Access Type of the PDU Session as "Multi-Access".

-	In step 3, the AMF also includes an "MA PDU Request" indication.

-	In step 6, the V-SMF also includes an "MA PDU Request" indication.

-	In step 9, the H-SMF sends an "MA PDU Request" indication to H-PCF in the SM Policy Control Create message. The H-PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data. The H-PCF provides the PCC rules for the MA PDU session and the H-SMF derives the ATSSS rules for the UE and the N4 rules for the H-UPF.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by H-SMF, and may include Measurement Assistance Information.

When the UE is registered to different PLMNs over 3GPP access and non-3GPP access, the MA PDU Session is established as specified in Figure 4.3.2.2.2-1 ("UE-requested PDU Session Establishment for home-routed roaming scenarios") with the following differences and clarifications:

-	In step 1, the UE provides a "MA PDU Request" indication and an ATSSS Capability (e.g. an "MPTCP Capability" and/or an "ATSSS-LL Capability"), as defined in TS 23.501 [2], clause 5.32.2 (Multi Access PDU Sessions).

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects a V-SMF, which supports MA PDU sessions. The AMF stores Access Type of the PDU Session as "Multi-Access".

-	In step 3, the AMF informs the V-SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication).

-	In step 6, the V-SMF informs the H-SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication).

-	In step 9, the H-SMF sends an "MA PDU Request" indication to PCF in the SM Policy Control Create message. The PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by H-SMF, and may include Measurement Assistance Information.

-	After the successful completion of the procedure in Figure 4.3.2.2.2-1, i.e. after the MA PDU Session is successfully established on the first access, the UE shall initiate again the procedure in Figure 4.3.2.2.2-1 over the other access. In particular:

-	In step 1, the UE shall send another PDU Session Establishment Request over the other access containing also a "MA PDU Request" indication and the same PDU Session ID that was provided over the first access.

-	In step 16, the UE receives another PDU Session Establishment Accept message, which may contain updated ATSSS rules for the MA PDU session.

* * * * Start of 2nd Change * * * *

4.22.3	UE Requested PDU Session Establishment with Network Modification to MA PDU Session

When an ATSSS-capable UE requests to establish a single-access PDU Session, but no policy in the UE and no local restrictions mandate a single access, the 5GC network may decide to modify it to a Multi-Access PDU (MA PDU) Session. This decision may be taken when e.g. the SMF wants to offload some traffic of the requested PDU Session to non-3GPP access or when the SMF wants to apply MPTCP to provide bandwidth aggregation for the requested PDU Session.

In case of non-roaming or roaming with local breakout, the procedure for establishing a MA PDU Session when the UE requests a single-access PDU Session is the same with the procedure specified in clause 4.22.2.1, with the following clarifications and modifications:

-	In step 1, the UE does not include the "MA PDU Request" indication but it may include the "MA PDU Network-Upgrade Allowed" indication in UL NAS Transport message and its ATSSS Capability in PDU Session Establishment Request message, if no policy in the UE (e.g. no URSP rule) and no local restrictions mandate a single access for the requested PDU Session. The "MA PDU Network-Upgrade Allowed" indication indicates that the requested single-access PDU Session may be converted to a MA PDU Session, if the 5GC network wants to.

-	In step 2, if the AMF receives the "MA PDU Network-Upgrade Allowed" indication, the AMF may select a SMF that supports MA PDU sessions. The AMF does not send the "MA PDU Request" indication to SMF, but it sends the "MA PDU Network-Upgrade Allowed" indication, if received from the UE. If the AMF sends the "MA PDU Network-Upgrade Allowed" indication to SMF, it shall also indicate to SMF whether the UE is registered over both accesses.

-	After step 6, if SMF receives the "MA PDU Network-Upgrade Allowed" indication, the SMF may decide to convert the single-access PDU Session requested by the UE into a MA PDU Session. The SMF may take this decision based on local operator policy and/or other conditions, which are not specified in the present document.

-	In step 7, the SMF indicates to PCF that the SM policy control information is requested for a MA PDU Session.

-	The PDU Session Establishment Accept message includes ATSSS rules which indicate to UE that the requested PDU Session was converted by the network to a MA PDU Session.

-	The SMF triggers the establishment of user-plane resources in both accesses, in case it was informed in step 2 that the UE is registered over both accesses.

-	After step 17, the SMF notifies the AMF that the PDU Session is a MA PDU Session. The AMF stores the Access Type of the PDU Session as "Multi-Access".

In case of home-routed roaming, when the UE is registered to the same VPLMN over 3GPP access and non-3GPP access, the procedure for establishing a MA PDU Session when the UE requests a single-access PDU Session but no policy in the UE and no local restrictions mandate a single access, is the same with the procedure specified above.

In case of home-routed roaming, when the UE is registered to different PLMNs over 3GPP access and non-3GPP access, the procedure for establishing a MA PDU Session when the UE requests a single-access PDU Session but no policy in the UE and no local restrictions mandate a single access, is the same with the procedure specified in clause 4.22.2.2, with the following clarifications and modifications:

-	In step 1, the UE does not include the "MA PDU Request" indication but it may include an "MA PDU Network-Upgrade Allowed" indication and its ATSSS Capability (e.g. the "ATSSS-LL Capability" and/or the "MPTCP Capability"). The UE may include the "MA PDU Network-Upgrade Allowed" indication and its ATSSS Capability as defined above.

-	In step 2, if the AMF receives the "MA PDU Network-Upgrade Allowed" indication, the AMF may select a V-SMF that supports MA PDU sessions. The AMF does not send the "MA PDU Request" indication to V-SMF, but it may send the "MA PDU Network-Upgrade Allowed" indication, if received from the UE.

-	In step 6, the V-SMF does not provide the "MA PDU Request" indication to H-SMF, but it provides the "MA PDU Network-Upgrade Allowed" indication, if received from AMF.

-	After step 6, if the H-SMF receives the "MA PDU Network-Upgrade Allowed" indication, the H-SMF may decide to convert the single-access PDU Session requested by the UE into a MA PDU Session. The H-SMF may take this decision based on local operator policy and/or other conditions, which are not specified in the present document.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which includes ATSSS rules and indicates to UE that the requested single-access PDU session was established as a MA PDU Session.

-	After step 17, the SMF notifies the AMF that the PDU Session is a MA PDU Session. The AMF stores the Access Type of the PDU Session as "Multi-Access".

-	After the MA PDU Session is established over one access, the UE shall send another PDU Session Establishment Request over the other access containing a "MA PDU Request" indication and the same PDU Session ID that was provided over the first access.

* * * * End of Changes * * * *