SA WG2 Meeting #134	S2-1907586

# 24-28 June 2019, Sapporo, Japan

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3536 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE attached for access to RLOS and emergency call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  |  | Date: |  | 2019-6-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CT1 discussed below SA2 text in C1-192357: Subclause 4.3.12a.8 of TS 23.401 states: #### “4.3.12a.8. Emergency service for RLOS attached UE For an RLOS attached UE initiating emergency service, the UE shall first request detach from the network and then perform emergency Attach as defined in clause 4.3.12.” CT1 concluded (in C1-192357) below behavior to avoid potential impacts to emergency call and to follow normal CT1 design principle applied in similar scenarios. “If a UE attached for access to RLOS needs to initiate an emergency call, the UE shall first perform a local detach prior to initiating an attach procedure for emergency bearer services.” Thus we have contradicting requirements. Hence it’s proposed to align as per CT1 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the UE attached for access to RLOS needs to make an emergency call, the UE shall first perform a local detach and then perform emergency registration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Two conflicting requirements for same situation in 3GPP specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

.

START OF CHANGE

#### 4.3.12a.8.	Emergency service for RLOS attached UE

For an RLOS attached UE initiating emergency service, the UE shall first perform a local detach prior to initiating an emergency attach as described in TS 24.301 [46].

END OF CHANGES