3GPP TSG-SA2 Meeting #134	S2-1908100

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1453 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of the PCF Group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-06-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) S2-1905949 (TS 23.501) and S2-1905855 (TS 23.502) introduced the concept of PCF Group. AMF may indicate the PCF Group ID to SMF in Nsmf_PDUSession_CreateSMContext service which can be used for PCF selection in SMF. This implies that SMF can select any PCF instance in the PCF Group to serve the PDU Session. This means the PCF instances within the PCF Group have access to the same context data, which is also a property of the NF Set. On the other hand, the reselection prodedures e.g. in Table 6.3.1.0-1 are applicable to NF Set, but not to NF Group. It is then unclear in case of PCF failure, should the NF consumer/SCP be able to reselect another PCF instance within the PCF Group. Thus the concept of PCF Group is partially overlapping concept with the NF Set concept; in the initial selection, any PCF instance within the PCF Group can be selected, but in case of failure the reselection is only possible within the NF Set. The concept of NF Set has not been defined for PCFs. 2) The introduction of the PCF Group ID is not reflected in the PCF selection procedure in clause 6.3.7.1. 3) The selected PCF needs to include the binding indication, including the PCF ID and/or PCF Group ID, in a response to the NF consumer (SMF or AMF). 4) In AMF relocation, the target AMF receives the PCF ID and/or PCF Group ID of the old PCF from the source AMF. It is up to AMF to decide whether to use the old PCF or assign a new PCF. SCP cannot do this decision on behalf of AMF, because the service operation is different (update vs. create). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) PCF Set ID and PCF Group ID are added as an alternative to PCF ID in PCF selection procedure. 2) In delegated PCF discovery, include the PCF ID or PCF Group ID of the selected PCF in a response to the NF consumer (SMF or AMF). 3) In AMF relocation, when delegated discovery is used, the AMF decides (instead of SCP) whether to use the old PCF, or assign a new PCF. 4) Add a general procedure for redirection within an NF Set. 5) Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reselection of PCF instance in case of failure is not defined. The concept of PCF Group is incomplete. In delegated PCF discovery, AMF or SMF is not aware of the PCF Set ID or PCF Group ID, and SCP is not able to reselect the PCF within the PCF Group in case of failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.21.3.2, 6.3.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The proposal for V-PCF discovery in 6.3.7.1 in this CR is the baseline for |  |  |  |  |  |  |  |  |

**************** First Change ****************

*** BEGIN CHANGES ***

#### 5.21.3.2	NF Set and NF Service Set

Equivalent Control Plane NFs may be grouped into NF Sets, e.g. several SMF instances are grouped into an SMF Set. NFs within a NF Set are interchangeable because they share the same context data, and may be deployed in different locations, e.g. different data centers.

In the case of SMF, multiple instances of SMFs within an SMF Set need to be connected to the same UPF. This also implies that any SMF in the SMF Set should be able to manage the N4 association with the UPF. Furthermore, any SMF in the SMF Set should be able to control any N4 session with the UPF for a given UE and PDU Session (however, at any given time, only one SMF in the SMF Set will control the UPF for a given UE's PDU Session).

A Control Plane NF is composed of one or multiple NF Services. Within a NF a NF service may have multiple instances. These multiple NF Service instances can be grouped into NF Service Set if they are interchangeable with each other because they share the same context data.

NOTE:	The actual mapping of instances to a given Set is up to deployment.

If a new NF is selected for a context data that is provided by existing NF within the same NF Set, and the selected NF is aware of a different NF serving the context data (by implementation specific means), the new may NF redirect the service operation from the peer CP NFs with a cause to indicate that new NF has been selected.

*** NEXT CHANGE ***

#### 6.3.7.1	PCF discovery and selection for a UE or a PDU Session

PCF discovery and selection functionality is implemented in AMF, SMF and SCP, and follows the principles in clause 6.3.1. The AMF uses the PCF services for a UE and the SMF uses the PCF services for a PDU Session.

When NF service consumer performs discovery and selection, the following applies:

-	The AMF may utilize the NRF to discover the candidate PCF instance(s) for a UE. In addition, PCF information may also be locally configured in the AMF. The AMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the AMF, depending on operator's policies. The PCF includes the PCF ID and PCF Set ID in the response to the AMF as part of the binding indication as described in clause 6.3.1.0.

-

In the non roaming case, the AMF selects a PCF instance for AM policy association and selects the same PCF instance for UE policy association. In the roaming case, the AMF selects a V-PCF instance for AM policy association and selects the same V-PCF instance for UE policy association.

The following factors may be considered at PCF discovery and selection for Access and Mobility policies and UE policies:

-	SUPI; the AMF selects a PCF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for PCF discovery.

-	S-NSSAI(s).

-	PCF Set ID

-	PCF Group ID of the UE's SUPI.

NOTE:	The AMF can infer the PCF Group ID the UE's SUPI belongs to, based on the results of PCF discovery procedures with NRF. The AMF provides the PCF Group ID the SUPI belongs to to other PCF NF consumers as described in TS 23.502 [3].

The following factors may be considered at PCF discovery and selection for a PDU session:

a)	Local operator policies.

b)	Selected Data Network Name (DNN).

c)	S-NSSAI of the PDU Session.

d)	SUPI; the SMF selects a PCF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for PCF discovery.

e)	PCF ID or PCF Group ID selected by the AMF for the UE, provided by the AMF to the SMF.

f)	MA PDU Session capability of the PCF, for an MA PDU session.

g)	Void.

h) PCF Set ID

The SMF may utilize the NRF to discover the candidate PCF instance(s) for a PDU Session. In addition, PCF information may also be locally configured in the SMF. The SMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the SMF, depending on operator's policies.

The PCF includes the PCF ID and PCF Set ID in the response to the SMF as part of the binding indication as described in clause 6.3.1.0.

In the case of delegated discovery and selection in SCP, the SMF shall include the available factors in the first request.

Editor's note:	It still needs to be clarified which actual set of parameters the sentence above is referring to.

The selected PCF instance for serving the UE and the selected PCF instance for serving a PDU session of this UE may be the same or may be different.

In the following scenarios, information about the PCF instance that has been selected i.e. the PCF ID or PCF Group ID may be forwarded to another NF. If the NF service consumer performs discovery and selection, this NF may use this PCF. In the case of delegated discovery and selection, this NF may include PCF ID or PCF Group ID in the request and the SCP may use this information to select the PCF instance (discovery may still be needed depending on what level of information is sent by the AMF, e.g. the address of the PCF instance may not be present):

When NF service consumer performs discovery and selection, the following applies:

-	During AMF relocation, the target AMF may receive a PCF ID  or PCF Group ID from the source AMF to enable the usage of the same PCF by the target AMF, and the target AMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	The AMF may, based on operator policies, forward the selected PCF to SMF instance(s) during the PDU Session Establishment procedure(s) to enable the usage of the same PCF for the AMF and the SMF instance(s). The SMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	In the roaming case, the AMF may, based on operator policies, e.g. roaming agreement, select the H-PCF in addition to the V-PCF for a UE by performing the PCF discovery and selection as described above. The AMF sends the H-PCF ID of the selected H-PCF instance to the V-PCF during the policy association establishment procedure.

When the SMF receives a a redirection indication with PCF ID or PCF Set ID from the PCF for the PDU session, the SMF shall terminate the current SM Policy Control association and reselects a PCF based on the received PCF ID or PCF Set ID, respectively. The SMF shall then establish an SM Policy Control association with the reselected PCF.

In the case of delegated discovery and selection in the SCP, the following applies:

-	The SCP selects a (V)-PCF instance for AM policy association. The selected (V-)PCF instance includes binding indication, including the (V-)PCF ID and PCF Set ID in the response to the AMF, as described in clause 6.3.1.0. The selected PCF instance includes the PCF Group ID in the response to the AMF. The AMF includes the received PCF ID and PCF Set ID to the request to establish the UE policy association. The SCP selects the corresponding PCF instance for UE policy association.

-	During AMF relocation, the AMF may receive a PCF ID and PCF Group ID from the source AMF to enable the usage of the same PCF instance by the AMF. The AMF may decide based on operator policy either to use the old PCF instance or select another PCF instance. If the AMF decides to use the old PCF, the AMF includes the PCF ID, and PCF Group ID as received from the source AMF in the AM policy update request to the SCP.

-	The AMF may, based on operator policies, forward the selected PCF ID and PCF Group ID to the SMF during the PDU Session Establishment procedure to enable the usage of the same PCF for the AMF and the SMF. The SMF may include that information in the request in discovery and selection parameters to the SCP. The SCP may decide based on operator policy either to use the indicated PCF instance or PCF Group or select another PCF instance..

-	In the roaming case, SCP may, based on operator policies, e.g. roaming agreement, select the H-PCF in addition to the V-PCF for a UE by performing the PCF discovery and selection as described above.

Editor's note:	If the sentence above applies, it is FFS how the AMF can send the selected H-PCF ID to the V-PCF during the policy association establishment procedure or it is FFS whether the sentence above means that the SCP needs to add an extra information H-PCF ID in the application part of the request; it is also FFS whether "SCP" means a SCP in VPLMN or SCP in HPLMN.

*** END CHANGES ***