3GPP TSG-SA WG2 Meeting #134	S2-1908150

Sapporo, Japan, 24 - 28th June 2019	(revision of S2-1906973)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1382 | rev | 3 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures and call flows for SMF Delegated Discovery and Selection in HR and specific SMF Service Areas scenarios and PCF Delegated Discovery and Selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle Corporation, NTT Docomo, ZTE, Deutsche Telekom AG, Huawei, HiSilicon, Telecom Italia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-06-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SMF discovery in a delegated mode in the HR and specific SMF Service Areas and scenarios and V-PCF/H-PCF discovery in a delegated mode is not covered by the generic delegated discovery procedures described in 23.502 clauses 4.17.9 and 4.17.10. Consequently, a new specific clause Annex X has to be added to 23.502 in order to capture that. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause Annex X is added to capture the concepts and generic functionality required for addressing the scenario of SMF Discovery and Selection in a delegated mode for Home Routed and specific SMF Service Areas scenarios as well as V-PCF/H-PCF discovery in a delegated mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedural description of SMF Discovery and Selection in a delegated mode for Home Routed and specific SMF Service Areas scenarios as well as V-PCF/H-PCF discovery in a delegated mode is missing from 23.502. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.2, 4.23.5, Annex X(new), Annex X.1(new), Annex X.2 (new), Annex X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 4.23.2	I-SMF selection

For non roaming or LBO roaming case, the AMF selects an SMF serving the PDU Session as described in clause 4.3.2.2.3. If the service area of the selected SMF does not control UPF that can serve the UE location, the AMF selects an I-SMF as described in clause 4.3.2.2.3.2 with UE location information as addtional input to NRF in step 3.

For home routed roaming case, the AMF selects V-SMF as described in clause 4.3.2.2.3.2 with UE location information as additional input to NRF in step 3.

When the delegated discovery is used, the SCP selects the SMF as described in Annex X.

* * * * Start of 2nd Change * * * *

### 4.23.5	PDU Session establishment procedure

For non roaming or LBO roaming, it includes the following cases:

-	If the service area of the selected SMF includes the location where the UE camps, the UE requested PDU Session Establishment procedure is same as described in clause 4.3.2.2.1.

-	If the service area of the selected SMF does not include the location where the UE camps, the AMF selects an I-SMF that serves the area where UE camps. The UE requested PDU Session Establishment procedure for Home-routed Roaming defined in clause 4.3.2.2.2 is used to establish the PDU Session. Different comparing to procedure defined in clause 4.3.2.2.2, the V-SMF and V-UPF are replaced by I-SMF and I-UPF, and H-SMF and H-UPF are replaced by SMF and UPF(PSA) respectively. Also only the S-NSSAI with the value defined by the serving PLMN is sent to the SMF.

-	When the delegated discovery is used, the SCP selects the SMF as described in Annex X.

For the Home-Routed roaming case, the UE requested PDU Session Establishment procedure for Home-routed Roaming in clause 4.3.2.2.2 can be reused without change.

* * * * Start of 3rd Change * * * *

Annex X (normative):
Delegated SMF discovery in the Home Routed and specific SMF Service Areas scenarios

### X.1	Delegated SMF discovery in the Home Routed scenario

Figure X.1: Delegated Discovery of SMF in the Home Routed Scenario

1. The AMF sends Nnrf_NFDiscovery Request to the V-NRF. The AMF may indicate the maximum number of H-SMF instances to be returned by the NRF.

2.	The NRF in VPLMN and NRF in HPLMN interact using the Nnrf_NFDiscovery service. See step 2 in clause 4.17.5.

3.	The AMF gets Nnrf_NFDiscovery service response with one or more profile(s) of SMF(s) in HPLMN.

4.	The AMF selects an SMF instance in HPLMN.

5.	The AMF forwards the Nsmf_PDUSession_CreateSMContext Request to the selected SCP in VPLMN. The body of the request includes the H-SMF ID. Discovery & Selection parameters include S-NSSAI, DNN, UE location and other necessary information.

6. [Optional] The SCP in VPLMN sends Nnrf_NFDiscovery Request to the V-NRF.

7. [Optional] The SCP in VPLMN gets Nnrf_NFDiscovery service response with profile(s) of SMF(s) in VPLMN.

8. The SCP in VPLMN selects an SMF instance in VPLMN, which supports the S-NSSAI, DNN and UE location received earlier from the AMF.

9.	The SCP in VPLMN forwards the Nsmf_PDUSession_CreateSMContext Request to the selected SMF instance in VPLMN. The body of the request includes the H-SMF ID.

10. The SMF in VPLMN sends an Nsmf_PDUSession_Create Request to the SCP in VPLMN. Discovery & Selection parameter is set to H-SMF instance ID.

11.	The SCP in VPLMN sends an Nsmf_PDUSession_Create Request to the selected SMF instance in HPLMN indicated in step 10.

When the V-SMF responds to AMF with Nsmf_PDUSession_CreateSMContext Response as in clause 4.3.2.2.2 in Step 3b, if the AMF has not stored the SMF Service Area for the V-SMF, the AMF shall obtain the SMF Service Area for the concerned V-SMF from the NRF using the Nnrf_NFManagement_NFStatusSubscribe service operation.

* * * * Start of 4th Change * * * *

# X.2	Delegated I-SMF discovery

Figure X.2: Delegated Discovery of I-SMF

1. 	The following impacts are applicable to clause 4.23.5 (PDU Session Establishment procedure) to support delegated SMF discovery:

The AMF needs to perform the SMF discovery. The AMF sends an Nsmf_PDUSession_CreateSMContext Request to a SCP. The request includes S-NSSAI, DNN, TAI that corresponds to the UE location, and indication “the SMF is required to support delegated discovery” as discovery and selection parameters.

2.	[Optional] The SCP sends an Nnrf_NFDiscovery Request to the NRF. The request includes S-NSSAI, DNN and TAI.

3.	[Optional] The SCP gets Nnrf_NFDiscovery service response. The response may include one or more profile(s) of SMF(s).

Depending on the available information in the Nnrf_NFDiscovery service response, the SCP may either execute steps in case A or in case B.

Case (A) The NRF response includes one or more SMF instances that support both the TAI that corresponds to the UE Location and S-NSSAI/DNN.

4. The SCP selects an SMF instance.

5. The SCP forwards the Nsmf_PDUSession_CreateSMContext Request to the selected SMF instance.

Case (B) The NRF response does not include any SMF instances that support both the TAI that corresponds to the UE Location and S-NSSAI/DNN.

6. The SCP returns an Nsmf_PDUSession_CreateSMContext Response to the AMF with an error ‘SMF not found’

7. The AMF sends Nnrf_NFDiscovery Request to the NRF. The AMF may indicate the maximum number of SMF instances to be returned by the NRF.

8.	The AMF gets Nnrf_NFDiscovery service response with one or more profile(s) of SMF(s).

9.	The AMF selects an SMF instance.

10. The AMF sends an Nsmf_PDUSession_CreateSMContext Request to a SCP. The request includes S-NSSAI, TAI that corresponds to the UE location, and SMF-ID as discovery and selection parameters.

11.	[Optional] The SCP sends an Nnrf_NFDiscovery Request to the NRF. The request includes TAI that corresponds to the UE location.

12.	[Optional] The SCP gets Nnrf_NFDiscovery service response. The response may include one or more profile(s) of I-SMF(s).

13. The SCP selects an I-SMF instance that supports the TAI.

14. The SCP forwards the Nsmf_PDUSession_CreateSMContext Request to the selected I-SMF instance. The body of the request includes the SMF ID.

15. The I-SMF sends an Nsmf_PDUSession_Create Request to the SCP. Discovery & Selection parameter is set to SMF instance ID.

16.	The SCP forwards the Nsmf_PDUSession_Create Request to the selected SMF instance indicated in step 15.

The procedure continues as described in clause 4.3.2.2.2 from Step 7.

Difference comparing to procedure defined in clause 4.3.2.2.2, the V-SMF and V-UPF are replaced by I-SMF and I-UPF, and H-SMF and H-UPF are replaced by SMF and UPF(PSA) respectively. Also only the S-NSSAI with the value defined by the serving PLMN is sent to the SMF.

When the I-SMF responds to AMF with Nsmf_PDUSession_CreateSMContext Response as in clause 4.3.2.2.2 in Step 3b, if the AMF has not stored the SMF Service Area for the I-SMF, the AMF shall obtain the SMF Service Area for the concerned I-SMF from the NRF using the Nnrf_NFManagement_NFStatusSubscribe service operation.

When the SMF registers to the UDM in clause 4.3.2.2.2 in Step 12c, and the UDM notifies the AMF on the change of the UE context in SMF data, if the AMF has not stored the SMF Service Area for the SMF, the AMF shall obtain the SMF Service Area for the concerned SMF from the NRF using the Nnrf_NFManagement_NFStatusSubscribe service operation.

* * * * Start of 5th Change * * * *

# X.3	Delegated PCF discovery in the Roaming scenario

Figure X.3: Delegated Discovery of PCF in the Home Routed Scenario

1. The AMF sends Nnrf_NFDiscovery Request to the V-NRF. The AMF may indicate the maximum number of H-PCF instances to be returned by the NRF.

2.	The NRF in VPLMN and NRF in HPLMN interact using the Nnrf_NFDiscovery service. See step 2 in clause 4.17.5.

3.	The AMF gets Nnrf_NFDiscovery service response with one or more profile(s) of PCF(s) in HPLMN.

4.	The AMF selects an PCF instance in HPLMN.

5.	The AMF forwards the Npcf_UEPolicyControl Request to the selected SCP in VPLMN. The body of the request includes the H-PCF ID. Discovery & Selection parameter is set to (V-)PCF instance ID for UE policy association.

6. The SCP in VPLMN selects the corresponding (V-)PCF instance for UE policy association received from the AMF.

7.	The SCP in VPLMN forwards the Npcf_UEPolicyControl Request to the selected PCF instance in VPLMN. The body of the request includes the H-PCF ID.

8. The PCF in VPLMN sends an Npcf_UEPolicyControl Request to the SCP in VPLMN. Discovery & Selection parameter is set to H-PCF ID.

9.	The SCP in VPLMN sends an Npcf_UEPolicyControl Request to the selected PCF instance in HPLMN indicated in step 8.

* * * * End of Changes * * * *