3GPP TSG SA WG2 Meeting #134	S2-1908303

# Sapporo, Japan, June 24 - 28, 2019														(Rev of S2-1907029)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1487 | rev | 1 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UEs not supporting 5GC NAS connected to PGW-C+SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#133, 23.502 CR1328 (S2-1906779) addressing the 4G only UEs connected to PGW-C+SMF was agreed. A UE’s 5GC NAS capability may be disabled and re-enabled, which mean that both UE and the PGW-C+SMF may assign PDU Session ID. Currently PDU Session ID has value 1-15 as defined in TS 24.007, considering that both UE and PGW-C may allocate PDU Session, to ensure its uniqueness, it’s proposed that the PGW-C+SMF use an extended value range. To avoid UE impact, it’s proosed that the extended value range is only applicable for CN. ePDG should be covered in a seperate cause since clause 4.11.a0.5 handles EPS procedures as defined in TS 23.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add text to explain that PGW-C+SMF should use extended range of PDU Session ID and this range is only used in CN. Move the ePDG impacts to the correct clause 4.11.4.3.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect/misleading specification; UEs that do not have 5GC NAS capability may not be able to establish PDN connection. UEs with N1 mode capabiltiy disabled and then re-enabled may not be able to establish PDN connection due to duplicated PDU Session ID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.a0.5, 4.11.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of changes ***************

#### 4.11.0a.5	PDN Connection Establishment

During PDN connection establishment in the EPC, the UE and the PGW-C+SMF exchange information via PCO as described in TS 23.501 [2] clause 5.15.7. In case the PGW-C+SMF supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the PGW-C+SMF provides an S-NSSAI to the UE, the PGW-C+SMF should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in TS 23.501 [2] clause 6.3.8). If the PGW-C+SMF is in a VPLMN, the PGW-C+SMF uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values.

If PGW-C+SMF is selected and interaction with UDM, PCF and UPF is required for a UE that has 5GS subscription but does not support 5GC NAS and is accessing via EPC/E-UTRAN, the PGW-C+SMF creates unique PDU Session ID for each PDN connection of the UE. The PGW-C+SMF shall not provide any 5GS related parameters to the UE. The PGW-C+SMF shall ensure that the PDU Session ID assigned to the PDN connection is not duplicated by using an extended range that is only used in the NFs.

*************** Next changes ***************

##### 4.11.4.3.3	Initial Attach with GTP on S2b

In clause 4.11.4.3.3, UEs are assumed to support both 5GC NAS and EPC NAS unless explicitly stated otherwise.

The procedure in TS 23.402 [26] clause 7.2.4 applies with the following modifications:

-	In Step A.1 IKEv2 tunnel establishment procedure, the 5GC NAS capable UE shall indicate its support of 5GC NAS in IKEv2. The UE allocates a PDU Session ID and also includes this in IKEv2 to the ePDG.

-	In Step A.1, UE's mobility restriction parameters related to 5GS or indication of support for interworking with 5GS for this APN or both as defined for MME in clause 4.11.0a.3 apply to the ePDG and are obtained by the ePDG as part of the reply from the HSS via the 3GPP AAA Server. These parameters and the 5G NAS support indicator from the UE, may be used by the ePDG to determine if a combined PGW-C+SMF or a standalone PGW should be selected.

-	In Step B.1, if the UE supports 5G NAS and the PDN connection is not restricted to interworking with 5GS by user subscription, the ePDG shall send the 5GS Interworking Indication and the PDU Session ID to the PGW-C+SMF.

-	In Step D.1 (Create Session Response), the PGW-C+SMF assigns a S-NSSAI to be associated with the PDN connection as specified in TS 23.501 [2] clause 5.15.7.1. The PGW-C+SMF sends the S-NSSAI to the ePDG.

-	In Step B.1 and D.1, if the UE does not support 5GC NAS but has 5GS subscription, and a PGW-C+SMF is selected and interaction with UDM, PCF and UPF is required, the PGW-C+SMF assigns PDU Session ID as specified in clause 4.11.0a.5.  The PGW-C+SMF shall not provide any 5GS related parameters to the UE.

-	In the IKEv2 Authentication Response message, the ePDG sends S-NSSAI to the UE. The UE associates the received S-NSSAI with the PDN Connection.

*************** End of changes ***************