3GPP TSG-SA2 Meeting #135	S2-1908746

# Split, Croatia, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1690 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Control Plane CIoT 5GS Optimisations restriction on NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Vodafone, AT&T, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CIoT 5GS Optimisations include both pure CN features that can run an any 5G AN as no AN support is needed and features that require 5G AN support. In Rel-16, the access network support for CIoT 5GS Optimisations that need 5G AN support is limited to E-UTRA (including LTE-M and NB-IoT). This Rel-16 limitation is not intended as an architectural requirement that should prevent “pure CN” 5G CIoT features to run over any AN. It is specified that Control Plane CIoT 5GS Optimisations are only supported over NB-IoT and WB-UTRA, but the handling of mobility cases to NR with regards to this restriction is not specified. Furthermore, such handling would necessarily have RAN impacts. On the other hand, supporting the Control Plane CIoT 5GS Optimisation over NR without the EDT feature and without the use of RAI, for a UE that has previously registered over E-UTRAN, has no impact on RAN. See the discussion in S2-1907152 for more details. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When connected over any other 5G AN than E-UTRA, a UE which negotiated the use of the CP CIoT 5GS optimisation, may use the Control Plane CIoT 5GS Optimisation, but without using EDT or RAI (as those capabilities would require AN support). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 5.31.1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** Start of changes ***

## 5.31	Support for Cellular IoT

### 5.31.1	General

This clause provides an overview about 5GS optimizations and functionality for support of Cellular Internet-of-Things (Cellular IoT, or CIoT) according to service requirements described in TS 22.261 [2]. Cellular IoT is in earlier 3GPP releases aka Machine Type Communication (MTC) (see TS 23.401 [26], clause 4.3.17). The specific functionality is described in the affected procedures and features of this specification, TS 23.502 [3], TS 23.503 [45] and other specifications.

CIoT 5GS Optimisations that are not dependant on 5G AN for their functionality, are supported by the 5GC over any 5G AN. In this Release Control Plane CIoT 5GS Optimisations (clause 5.31.4) with EDT or with the provision of RAI, and User Plane CIoT 5GS Optimisations (clause 5.31.18) are only supported over E-UTRA (including NB-IoT and LTE-M).

CIoT functionality is provided by the visited and home networks when the networks are configured to support CIoT. It applies to both the non-roaming case and the roaming case and some functionality may be dependent upon the existence of appropriate roaming agreements between the operators.

Some of the CIoT functions are controlled by subscriber data. Other CIoT functions are based on indicators sent by the UE to the network. CIoT functionality is performed by UEs that are configured to support different options as described in clause 5.31.2

Though motivated by scenarios and use cases defined in TS 22.261 [2], the functions added to support CIoT have general applicability and are in no way constrained to any specific scenario, use case or UE types, except where explicitly stated.

In the context of CIoT the term AF denotes an SCS/AS as defined TS 23.682 [36].

# *** End of changes ***