3GPP TSG-SA WG2 Meeting #135	S2-1909765

# Split, Croatia, 14 October – 18 October 2019 	(revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0052 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Session-TMBR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on current description, SMF will receive: - Session-TMBR for both GBR and Non-GBR traffic flows, - Session-AMBR for all Non-GBR traffic flows, - GBR traffic flows. No matter it is a signle PDU Session or a MA PDU Session, the new parameter Session-TMBR will be provided to 5G-RG, W-AGF and UPF entities. However, current Session-TMBR provision cannot solve single PDU Session with multiple PDU Session Anchors situation: - UL CL and PSA both receives Session-TMBR = 10MB, PSA has GBR_1=2MB, UL CL diverts GBR_2 = 3MB. - When the Session-TMBR is enforcemed, UL CL can afford maximum non-GBR traffic flows 10-(2+3) = 5MB; while PSA can afford maximum non-GBR traffic flows 10-2 = 8MB; - If PSA sends 8 MB packets to UL CL, UL CL will discard the packets, even if UL CL can afford 5MB. Therefore, a variable which controls all the Non-GBR QoS flows via wireline access is needed. SMF knows the whole activated GBRs compared to UL CL and PSA. If SMF firstly does the calculation: Session-TMBR (10MB) – GBR_1 (2MB) – GBR_2 (3MB) = 5MB, and then distribute 5MB (Non-GBR traffic flows via wireline) as Session-AMBR to UL CL and PSA, no packet loss. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We introduce a parameter which captures the characteristics of the Aggregated Maximum Bit Rate that can be expected to be provided across all Non-GBR QoS Flows for a specific PDU Session via wireline access. This parameter is calculated by SMF and it is functionally equivalent to Session-AMBR. Parameter = Session-TMBR – SUM(GBRs via wireline) For a single PDU Session setup for a 5G-RG or a FN-RG, - SMF calculates this parameter and compares the value with Session-AMBR, the t Session-AMBR is set to the smaller one. - then SMF signals Session-AMBR to the RG and UPF entities. - For UL: 5G-RG enforces Session-AMBR instead of Session-TMBR; - For DL: UPF acting as anchor UPF enforces Session-AMBR instead of Session-TMBR. For a MA PDU Session setup for a 5G-RG, - SMF first calculates this parameter and compares the value with Session-AMBR, the Session-AMBR is set to the value of the parameter. - then SMF signals Session-AMBR to 5G-RG and UPF entities; - For UL: 5G-RG enforces Session-AMBR instead of Session-TMBR; - For DL: UPF acting as anchor UPF enforces Session-AMBR instead of Session-TMBR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Firstly, in UL CL situation, packet loss will be reduced. Secondly, we offer a method to enable 5G-RG and UPF only enforcing Session-AMBR for wireline access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.5.1	Wireline access specific 5G QoS parameters

In addition to the 5G QoS parameters specified in clause 5.7.2 of TS 23.501 [2], the parameters defined in clause 4.5.1.1 and in clause 4.5.1.2 are applicable for the wireline access network related PDU sessions.

#### 4.5.1.1	Total Maximum Bit Rates

For wireline access networks (W-5GAN) that do not provide dynamic resource reservation, the GBR QoS flows cannot be catered for independently of non-GBR flows. In such W-5GANs, the aggregate bitrate need to reflect both GBR and non-GBR flows instead.  These reflect the characteristics of the wireline access provided for the 5G-RG or FN-RG.

Each PDU Session of a 5G-RG or FN-RG may be associated with the following aggregate rate limit QoS parameter, when using a wireline access network (W-5GAN):

-	per Session Total Maximum Bit Rate (Session-TMBR).

The subscribed Session-TMBR is a subscription parameter which is retrieved by the SMF from UDM. The Session-TMBR limits the aggregate bit rate that can be expected to be provided across all GBR and Non-GBR QoS Flows for a specific PDU Session. The Session-TMBR is measured over a TMBR averaging window which is an operator specific value.

In case of a single PDU session established via wireline access for a RG, the SMF first calculates the Aggregated Maximum Bit Rate that can be expected to be provided across all Non-GBR QoS Flows for a specific PDU session via wireline access. This is a parameter that depends on the sum of all activated GBR traffic flows via wireline access. The SMF calculates as the difference between  the Session-TMBR and the sum of the traffic of all current activated GBR traffic flows transmitted via wireline access. Then the SMF compares the result with the Session-AMBR, and sets the Session-AMBR to be the smaller one.

In case of a MA PDU session established via both wireless and wireline accesses for a 5G-RG, the SMF executes the same evaluation as for the single PDU session established via wireline access use case. After having perform the evaluation, the SMF compares result with the Session-AMBR. If the value of this variable is bigger than the Session-AMBR, the SMF sends Session-AMBR to the UPF entity/ies as usual.

NOTE: 	The situation when Session-AMBR is larger than the value calculated for the parameter above is not considered in this release.

The SMF provides Session-TMBR to W-AGF via N2 SM signalling.

The UE subscription data parameters for Session-TMBR are defined in clause 8.1.1.

**** End of Changes ****