3GPP TSG-SA WG2 Meeting #135	S2-1909817

14 - 18 October, 2019, Split Croatia	(revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1905 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility support limitiaton for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, AT&T, Vodafone, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IABARC |  |  |  |  |  | Date: |  |  | 2019-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#134, it was clarified that the IAB mobility support requirements were yet to be finalized by RAN WGs. Sepcifically, whether there is a need to support inter-IAB-donor gNB mobility.Therefore, an editor's note was added to address this in clause 5.35.4. However, in SA2 discussion, it is identified that when the IAB-node changes its IAB-donor gNB, i.e. change the IAB-donor-CU, there is a need to change the cell ID of the IAB-node, since it is managed by the CU. From SA2 perspective, change of the cell ID will have significant impacts to system level operation, including the service support to end UEs, and the charging of the traffic. All these would require significant amount of time invested in the study and specification update. Therefore, it is proposed to clarify that in this release, there is no system level support of the service continuity when an IAB-node has an inter-IAB-donor gNB mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that in this release there is no system level support of service continuity when an IAB-node changes IAB-donor-CU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect service level expectation for IAB mobility support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.35.4	Mobility support with IAB

For UEs, all existing NR intra-RAT mobility and dual-connectivity procedures are supported when the UE is served by an IAB-node.

In this release of the specification, there is no system level support of service continuity for a UE served by an IAB-node when its serving IAB-node changes its IAB-donor-CU.

*** END CHANGES ***