3GPP TSG SA WG2 Meeting #135	S2-1909010

# Split, Croatia, October 14 - 18, 2019											(revision of S2-1907060)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0008 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Merge of NF Load Analytics and Network Performance Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NF Load Analytics and Network Performance Analytics share most of the input and output data. There is confusion as for when one or the other should be used. Additionally, the Slice Load Level related information is sharing similar inputand output data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed solution merges the NF Load Analytics and Network Performance Analytics into a single analytics, respecting the existing input and output. This allows consumers of this analytics to receive detailed analytics report without bothering of which analytics ID should be requested. A note has been added to the Slice Load Level related network analytics to indicate that is main purpose is to be backward compatible with Release 15. The Network Perofrmance Analytics has been extended, so that it can be used with the same input and output as the Slice load level analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Consumers of this analytics will find difficult to understand which analytics ID should be requested. Implementations will suffer from implementing two similar analytic reports. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 6.5, 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1 was not handled. Revision 2 is merely updated to the latest verson of TS 23.288 |  |  |  |  |  |  |  |  |

**** First Change ****

## 6.3	Slice load level related network data analytics

### 6.3.1	General

The NWDAF provides slice load level information to an NF on a network slice instance level. The NWDAF is not required to be aware of the current subscribers using the slice. The NWDAF notifies slice specific network status analytics information to the NFs that are subscribed to it. An NF may collect directly slice specific network status analytics information from NWDAF. This information is not subscriber specific.

NOTE:	This analytics report is maintained here for Release 15 backward compatibility. But it has been superceeded by the Network Performances Analytics (see clause 6.6).

The NWDAF services as defined in the clause 7.2 and clause 7.3 are used to expose slice load level analytics from the NWDAF to the consumer NF (e.g. PCF or NSSF).

The following Analytics ID is used for the slice load level related network data analytics:

-	Load level information

The following Analytics Filters can be included by the consumer in the related Nnwdaf_AnalyticsSubscription_Subscribe and Nnwdaf_AnalyticsInfo_Request service operation:

-	Network Slice Instance.

-	Load Level Threshold value.

### 6.3.2	Void

### 6.3.2A	Input data

There is no input data specification for support of slice load level analytics in this Release of the specification.

### 6.3.3	Void

### 6.3.3A	Output analytics

The NWDAF reports when the load level crosses the threshold provided in the analytics subscription; if no threshold is provided in the subscription, the reporting (Notify operation) is assumed to be periodic.

**** Next Change ****

## 6.5	Void

**** Next Change ****

## 6.6	Network Performance Analytics

### 6.6.1	General

Network Performance Analytics provides either statistics or predictions on the load in an Area of Interest; in addition it may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The service consumer may be an NF (e.g. PCF, NEF, AF), or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID set to "Network Performance".

-	Target of Analytics Reporting containing either a UE, or Internal Group Identifier that refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Analytics target period is requested.

-	a list of NF instance IDs;

- 	a Network Slice Instance ID for requesting overall network slice instance load level information;

-	Analytics Filter Information containing:

-	Optional maximum number of results,

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target Of Event Reporting is set to "any UE", optional otherwise),

-	Load Level Threshold value: the NWDAF reports when the load level crosses the threshold provided in the analytics subscription; if no threshold is provided in the subscription, the reporting (Notify operation) is assumed to be periodic.

-	An Analytics target period indicates the time period over which the statistics or prediction are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

If the NF Instance IDs are provided, the NWDAF shall provide the analytics for each designated NF instance. In such case the Target of Analytics Reporting should be ignored. Otherwise, the NWDAF shall use the specific UE ID to determine which instances (AMF, N3IWF, SMF and PCF) are serving this specific UE.

NOTE:	The list of possible instances is limited to AMF, N3IWF, SMF, PCF, and UPF.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

### 6.6.2	Input Data

The NWDAF collects Load and Performance information in an Area of Interest from the sources listed in Table 6.6.2-1 and number of UEs within Area of Interest from the sources listed in Table 6.6.2-2.

NOTE 1:	The tables below show the information provided to the NWDAF, in other words the information provided by NWDAF is not shown here but in the procedures in clause 6.6.4.

Table 6.6.2-1:  Load and Performance information collected by NWDAF

| Load information | Source | Description |
| --- | --- | --- |
| Load and performance information | OAM | Statistics on RAN load and performance per Cell Id in the Area of Interest. |
| NF Load information | NRF | Load per NF |
| NF status | NRF | The status of a specific NF instance as defined per TS 29.510 [18]. |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for a specific NF instance (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7 |

Editor's note:	It is FFS whether the number of registered instances of an NF is needed input data for NWDAF to perform predictions on NF instance load and resource usage.

NOTE 2:	The OAM information can be used as a complement to NRF information on resources, or as a way to correlate NRF information or as an alternative if NRF information on load is not available.

Table 6.6.2-2:  Number of UEs in Area of Interest information collected by NWDAF

| Number of UEs information | Source | Description |
| --- | --- | --- |
| Number of UEs | AMF | Number of UEs in an Area of Interest |

If target NF type is UPF, the NWDAF may collect the information as listed in Table 6.5.2-2, in addition with Table 6.6.2-3.

Table 6.6.2-3: Collecting Data for analysing UPF load analytics

| Information | Source | Description |
| --- | --- | --- |
| Traffic usage report | UPF | Report of user plane traffic in the UPF for the accumulated usage of network resources (see TS 29.244 [17]) |

### 6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |
| --- | --- |
| List of performance information (1..max) | Observed statistics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area of interest |
| > List of resource status (1..n) | List of observed load information for each NF instance |
| >> NF type | Type of NF (e.g, AMF, N3IWF, SMF, PCF, UPF) |
| >> NF status | Availability status of the NF on the Observation period, expressed as a percentage of time |
| >> NF resource usage | Average usage of assigned resources (CPU, memory, disk) |
| >> NF load | Aaverage load of the NF instance over the observation period |
| >> NF peak load (optional) | Amaximum load of the NF instance over the observation period |

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-2: Network performance predictions

| Information | Description |
| --- | --- |
| List of performance information (1..max) | Predicted analytics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area |
| > gNB resource usage | Usage of assigned resources (CPU, memory, disk) (average, peak) |
| > Number of UEs | Number of UEs predicted in the area |
| > Communication performance | Ratio of successful setup of PDU Sessions |
| > Mobility performance | Ratio of successful handover |
| > Confidence | Confidence of this prediction |
| > List of resource status (1..n) | List of observed load information for each NF instance |
| >> NF type | Type of NF (e.g, AMF, N3IWF, SMF, PCF, UPF) |
| >> NF status | Availability status of the NF on the Observation period, expressed as a percentage of time |
| >> NF resource usage | Average usage of assigned resources (CPU, memory, disk) |
| >> NF load | Aaverage load of the NF instance over the observation period |
| >> NF peak load (optional) | A maximum load of the NF instance over the observation period |

NOTE:	The variations on per-instance NF load and resource usage could be influenced by the number of running NF instances in addition to the load itself.

The NWDAF provides Network Performance Analytics to a consumer that includes the load in the area of interest and the number of UEs located in the area of interest or both at the time requested by the consumer in the Analytics target period:

-	Analytics ID set to "Network Performance".

-	Notification Target Address including the address of the consumer.

-	Notification Correlation Id, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytics specific parameters including the load in the Area of Interest and the number of UEs in the Area of Interest, at the time indicated in the Analytics target period.

### 6.6.4	Procedures

Figure 6.6.4-1: Procedure for subscription to network performance analytics

1.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID="Network Performance", Target of Analytics Reporting="Internal Group Identifier" or list of NF instance IDs, Analytics Filter="Area of Interest" and Analytics target Period(s)) to the NWDAF.

2a-2d.	The NWDAF discovers from NRF the NFs related to the UE that can supply data (e.g., AMF(s) belonging to the AMF Region(s) that include(s) the Area of Interest, SMF, N3IWF, etc.) and subscribes to NF load information from NRF about these NF(s).

3a-3b.	The NWDAF subscribes to OAM services to get the load information on the Area of Interest, following the procedure captured in Clause 6.2.3.2.

Editor's note:	The exact SA WG5 service, granularity (NF, eNB) and measurement or KPI that are used for load.

4a-4b.	The NWDAF collects the number of UEs located in the Area of Interest from AMF using Namf_EventExposure_Subscribe service, including the Target of Event Reporting provided as an input parameter (i.e. any UE or Internal Group Identifier).

5.	The NWDAF derives the requested analytics.

6.	The NWDAF sends Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion).

7-8. A change of network performance information, i.e. change in the load in the area of interest at the observed period, is detected by OAM, or a change in the NF load information is reported by NRF, and is notified to NWDAF.

9.	The NWDAF derives new analytics based on the most recent data collected.

10.	The NWDAF provides a notification using Nnwdaf_AnalyticsSubscription_Notify (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion.

.

*************** End of changes ***************