3GPP TSG-WG SA2 Meeting #135 	S2-1909273

# Split, Croatia, October 14 – 18, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0068 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the data collection of the OSE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the last CT3#105 meeting, a LS (i.e. C3-1903429 / S2-1908665) on SMF Event Exposure enhancement for service experience was sent out to ask the following questions: - If the NWDAF subscribes to the SMF to collect information related to the QoS flow level Network Data event by invoking Nsmf_EventExposure service, what information is needed by the NWDAF and how the SMF determine the event is detected or not? - When requesting the QoS flow level Network Data from the SMF, whether the above highlighted information are all mandatory? In fact, the two questions not only apply to the data collection from SMF but also some others NFs. Furthermore, the level of the service data defined in Table 6.4.2-1 is unclear, whether it is UE level or Application level? |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the current specification during the Data Collection for the Service Expierence Analytics in order to answer the questions in the LS from CT3; - Clarify the level of the service data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear for the eNA CT3 definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.4.2	Input Data

The service data collected from the AF, the network data from other 5GC NFs and the network data from OAM for observed service experience are defined in Table 6.4.2-1, Table 6.4.2-2 and Table 6.4.2-3, respectively.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Presence |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Application ID | AF | Mandatory |  | To identify the service and support analytics per type of service (the desired level of service) |  |
| IP filter information | AF | Mandatory | Identify a service flow of the UE for the application |
| Locations of Application | AF/NEF | Optional |  | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |  |
| Service Experience | AF | Mandatory |  | Refers to the QoE per service flow as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |  |
| Timestamp | AF | Mandatory |  | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |  |

NWDAF subscribes the service data from AF in the Table 6.4.2-1 either directly by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID) as defined in TS 23.502 [3], or via NEF by invoking Nnef_EventExposure_Subscribe service.

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Presence | Description |  |
| --- | --- | --- | --- | --- |
| Timestamp | 5GC NF | Mandatory | A time stamp associated with the collected information. |  |
| Location Info | AMF | Optional | The UE location information when the service is delivered. |  |
| DNN | SMF | Optional | DNN for the PDU Session which contains the QoS flow |  |
| S-NSSAI | SMF | Conditional (NOTE) | S-NSSAI for the PDU Session which contains the QoS flow |  |
| Application ID | PCF/SMF | Mandatory | Used by NWDAF to identify the application service provider and application for the QoS flow |  |
| IP filter information | SMF | Mandatory | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |  |
| QFI | SMF | Optional | QoS Flow Identifier |  |
| QoS flow Bit Rate | UPF | Optional | The observed bit rate for UL direction; and The observed bit rate for DL direction |  |
| QoS flow Packet Delay | UPF | Optional | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |  |
| Packet transmission | UPF | Optional | The observed number of packet transmission |  |
| Packet retransmission | UPF | Optional | The observed number of packet retransmission |  |
| NOTE: The S-NSSAI is mandatory if the NWDAF needs to derive the Service Experience analytics for UEs on multiple Network Slices. |  |  |  |

NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source |  | Presence |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| Timestamp | OAM | Mandatory | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM |  | Optional |  | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |  |
| Reference Signal Received Quality | OAM |  | Optional |  | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |  |
| Signal-to-noise and interference ratio | OAM |  | Optional |  | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |  |

NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

The Event Filter for the service data collection from AF, the Event Filter for the network data collection from 5GC NFs and the input of the subscription for the network data from OAM are defined in Table 6.4.2-4, Table 6.4.2-5 and Table 6.4.2-6, respectively.

Table 6.4.2-4: Event Filter Information related to the service data from AF

| Information | Presence | Description |
| --- | --- | --- |
| Application ID (1..n) | Conditional (NOTE) | An identification of the application or a set of identifications of the applications. |
| Data target period | Optional | Identifies the observation period or time window of the Application(s). |
| NOTE: If no Application ID is provided, the Event Filter information applies to any Application (i.e. all Applications). |  |  |

Table 6.4.2-5: Event Filter Information related to the network data from 5GC NFs

| Information | Presence | Description |
| --- | --- | --- |
| Application ID (1..n) | Conditional (NOTE 1) | An identification of the application or a set of identifications of the applications. |
| S-NSSAI | Conditional (NOTE 2) | Identifies the Network Slice for which will be analysed. |
| Data target period | Optional | Identifies the observation period or time window of the Application(s). |
| NOTE 1: If no Application ID is provided, the Event Filter information applies to any Application (i.e. all Applications). NOTE 2: The S-NSSAI is mandatory if the NWDAF needs to derive the Service Experience analytics for UEs on a Network Slice. |  |  |

Table 6.4.2-6: Input of the subscription related to the network data from OAM

| Information | Presence | Description |
| --- | --- | --- |
| Data target period | Optional | Identifies the observation period or time window of the Application(s). |

Upon successful receipt of the NWDAF request or subscription:

-	the AF collects service data according to the Event Filter information, and notifies the NWDAF the collected data as long as the Event Reporting Information (e.g. periodicity, immediate or one-time reporting) is fulfilled.

-	the others 5GC NFs collect network data according to the Event Filter information, and notify the NWDAF the collected data as long as the Event Reporting Information (e.g. periodicity, immediate or one-time reporting) is fulfilled.

-	the OAM collects network data according to the input of the subscription.

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

# * * * * End of changes * * * *