3GPP TSG-SA WG2 Meeting #135	S2-1909331

# Split, Croatia, 14 – 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0045 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the support of RG-LWAC and UDM procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The RG-LWAC is wireline specific information sent over N2 to W-AGW, but it has so far not been clearly described in which messages it is sent. In clause 7.5 it is not explicit that 23.502 procedures apply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RG-LWAC to Registration call flows. Clarify clause 7.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing stage 2 requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.1, 7.2.1.3, 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 7.2.1.1	5G-RG Registration via W-5GAN

The 5G-RG registration management procedures are followed for both W-5GBAN and W-5GCAN.

Clause 7.2.1.1 specifies how a 5G-RG can register to 5GC via aW-5GAN. It is based on the Registration procedure specified in TS 23.502 [3] clause 4.2.2.2.2 and it uses the EAP method "EAP-5G" as specified in clause 4.12.2.1 in TS 23.502 [3]. If the 5G-RG needs to be authenticated, mutual authentication is executed between the 5G-RG and AUSF. The details of the authentication procedure are specified in TS 33.501 [11]. In Registration and subsequent Registration procedures via W-5GAN access, the NAS messages are always exchanged between the 5G-RG and the AMF. When possible, the 5G-RG can be authenticated by reusing the existing UE security context in AMF for the 5G-RG.

Figure 7.2.1.1-1: 5G-RG Registration via W-5GAN

NOTE 1:	EAP-5G is assumed to be used during authentication but need to be verified with BBF.

NOTE 2:	The procedure assuming that EAP-5G is used between the 5G-RG and the W-AGF. This assumption may be revised based on SA WG3 and BBF decision

1.	The 5G-RG connects to a W-5GAN with procedures outside the scope of 3GPP and creates an initial W-CP EAP signalling connection. This connection shall support EAP messages transfer between 5G-RG and W-AGF.

2.	The W-AGF sends an EAP-Request/5G-Start packet over the W-CP connection. The EAP-Request/5G-Start packet informs the 5G-RGto initiate an EAP-5G session, i.e. to start sending NAS messages encapsulated within EAP-5G packets.

3.	The 5G-RG sends an EAP-Response/5G-NAS packet that contains the Access Network parameters (SUCI or 5G-S-TMSI or GUAMI, the selected PLMN, Requested NSSAI and Establishment Cause) and a NAS Registration Request message (SUCI or 5G-GUTI, security parameters/UE security capability, NSSAI parameters, UE MM Core Network Capability, PDU session status, Follow-on request. The Establishment cause provides the reason for requesting a signalling connection with 5GC.

NOTE 3:	The steps from 1 to 3 depend on BBF decision for what protocols to use for NAS transport. The step needs to be revised based on their decision.4.	The W-AGF shall select an AMF based on the received AN parameters and local policy, as specified in TS 23.501 [2], clause 6.3.5. The W-AGF shall then forward the Registration Request received from the UE to the selected AMF within an N2 initial UE message (NAS message, Line-id based ULI, Establishment cause, UE context request).

5.	The selected AMF may decide to request the SUCI by sending a N2 Downlink NAS transport message (NAS Identity Request) message to W-AGF. This NAS message and the response are sent between W-AGF and 5G-RG encapsulated within EAP/5G-NAS packets. In this case the RG shall answer with a NAS Identity response that is carried over EAP-5G and N2 signalling.

6.	The AMF may decide to authenticate the 5G-RG by invoking an AUSF. In this case, the AMF shall select an AUSF as specified in TS 23.501 [2] clause 6.3.4 based on SUPI or SUCI.

The AUSF executes the authentication of the UE as specified in TS 33.501 [11]. The AUSF selects a UDM as described in TS 23.501 [2], clause 6.3.8 and gets the authentication data from UDM. The authentication packets are encapsulated within NAS authentication messages and the NAS authentication messages are encapsulated within EAP/5G-NAS packets. Between W-AGF and AMF, the messages are encapsulated within N2 downlink/uplink NAS transport messages. After the successful authentication:

In step 6c, the AUSF shall send the anchor key (SEAF key) to AMF which is used by AMF to derive NAS security keys. The 5G-RG also derives the anchor key (SEAF key) and from that key it derives the NAS security keys and the security key for W-AGF (W-AGF key).

Editor's note:	Whether the security key for W-AGF (W-AGF key) is also derived is to be determined with BBF and SA WG3.

In step 6c, the AUSF shall also include the SUPI, if in step 6a the AMF provided to AUSF a SUCI.

The AMF decides if the Registration Request needs to be rerouted as described in clause TS 23.502 [3] clause 4.2.2.2.3, where the initial AMF refers to the AMF.

Editor's note:	Check that procedure for authentication of 5G-RG is added to TS 33.501 [11].

NOTE 4:	EAP-AKA' or 5G-AKA are allowed for the authentication of 5G-RG via W-5GAN access, as specified in TS 33.501 [11]. Figure 7.2.1.1-1 only shows authentication flow using EAP-AKA'. . Other EAP authentication methods (see TS 33.501 [11]) for 5G-CRG with non-3GPP identities and credentials can be used for Stand-alone Non-Public Network.

7a.	The AMF shall send a NAS Security Mode Command (Selected NAS security algorithm, ngKSI, Replayed UE security capabilities, IMEISV request, Additional 5G security information, EAP message) to 5G-RG in order to activate NAS security. If an EAP-AKA' authentication was successfully executed in step 6, the AMF shall encapsulate the EAP-Success received from AUSF within the NAS Security Mode Command message. The message is encapsulated within a N2 downlink NAS transport message.

7b.	The W-AGF shall forward the NAS Security Mode Command message to 5G-RG within an EAP/5G-NAS packet.

7c.	The 5G-RG completes the authentication procedure (if initiated in step 6), creates a NAS security context  and sends the NAS Security Mode Complete message (IMEISV) within an EAP/5G-NAS packet.

Editor's note:	Whether a security key for W-AGF (W-AGF key) is also derived is to be determined by BBF and SA WG3.

7d.	The W-AGF relays the NAS Security Mode Complete message to the AMF in a N2 Uplink NAS transport message.

8a.	Upon receiving NAS Security Mode Complete, the AMF shall send an N2 Initial Context Setup Request message (Old AMF, UE Aggregate MBR, RG LWAC, GUAMI, Allowed NSSAI, UE security capability, Security Key, Trace Activation, Masked IMEISV).

8b.	This triggers the W-AGF to send an EAP-Success to 5G-RG, which completes the EAP-5G session. After this step, NAS messages between 5G-RG and W-AGF are transported without EAP-5G using W-CP signalling connection.

9.	[Conditional]: An W-CP signalling connection is established between the 5G-RG and W-AGF.

Editor's note:	Whether this step is performed or not requires input from BBF and CableLabs.

10.	W-AGF notifies the AMF that the 5G-RG context (including AN security) was created by sending a N2 Initial Context Setup Response.

11.	[Conditional] The AMF may request the PEI from the 5G-RG as described in TS 23.502 [3], clause 4.2.2.2.2 step 11.

12.	The AMF performs step 12-16 in TS 23.502 [3] clause 4.2.2.2.2. At AMF registration to UDM for the 5G-RG, the Access Type non-3GPP access is used. The RAT type used toward PCF and UDM shall indicate wireline access. The AMF determines Access Type and RAT Type based on the Global RAN Node ID associated with the N2 interface.

13.	The AMF sends N2 Downlink NAS transport with the NAS Registration Accept message (as defined in step 21 TS 23.502 [3] clause 4.2.2.2.2) to the W-AGF. which forwards the NAS Registration accept message to the 5G-RG.

14.	[Conditional]The 5G-RG responds with NAS Registration Complete message as described in TS 23.502 [3] clause 4.2.2.2.2 step 22 and W-AGF forwards the NAS Registration Accept to AMF in a N2 Uplink NAS transport message.

15. The AMF performs step 23-24 in TS 23.502 [3] clause 4.2.2.2.2.

**** Next Change ****

#### 7.2.1.3	FN-RG Registration via W-5GAN

The FN-RG registration management procedures are followed for both W-5GBAN and W-5GCAN. The FN-RG does not support N1 but instead the W-AGF handles the NAS signalling on behalf of the FN-RG.

Editor's note:	The procedure below may need to be updated based on outcome of BBF and SA WG3 discussions

When the connectivity is established between the FN-RG and the W-AGF in the W-5GAN, the W-AGF may authenticate the FN-RG; this is controlled by local policies and defined in BBF specifications. Then when the RM state of the FN-RG is "RM-DEREGISTERED" the W-AGF shall perform registration to 5GC as described in this clause, otherwise it performs Service Request as defined in clause 7.2.2.

Once the FN-RG is in RM-REGISTERED and CM-CONNECTED the W-AGF may setup PDU session(s) on behalf of the FN-RG (as described in clause 7.3.4).

Figure 7.2.1.3-1: FN-RG Registration via W-5GAN

1.	The FN-RG connects to a W-AGF (W-5GAN) via a layer-2 (L2) connection, based on Wireline AN specific procedure.

The FN-RG is authenticated by the W-5GAN based on Wireline AN specific mechanisms.

2.	W-AGF selects an AMF based on the AN parameters and local policy. W-AGF may use the Line ID / HFC identifier provided from the Wireline AN to determine the 5GC and AN parameters to be used for the FN-RG registration. How the W-AGF can determine the necessary 5GC and AN parameters is defined in BBF WT-456 [9] or CableLabs WR-TR-5WWC-ARCH [27].

3.	W-AGF performs initial registration on behalf of the FN-RG to the 5GC. The W-AGF sends a Registration Request to the selected AMF within an N2 initial UE message (NAS Registration Request, Line-ID/HFC-node-ID based ULI, Establishment cause, UE context request, Allowed NSSAI, Authenticated Indication).

The NAS Registration Request contains the SUCI or 5G-GUTI of the FN-RG, security parameters/UE security capability, UE MM Core Network Capability, PDU Session Status, Follow-on request, the selected PLMN, Requested NSSAI and Establishment Cause. The 5G-GUTI, if available, has been received from the AMF during a previous registration and stored in W-AGF.

The NSSAI parameters are provided based on W-AGF configuration. Based on W-AGF configuration of the 5GC NAS parameters, one or multiple Requested S-NSSAI may be used; e.g. when the W-AGF has been configured to use a specific slice for RG management purposes.

The following differences exist, compared to 5G-RG case:

-	The W-AGF use SUCI as defined in clause 4.7.3 and clause 4.7.4.

-	The Authenticated Indication indicates to AMF and 5GC that the FN-RG has been authenticated by the access network.

The SUCI is built by the W-AGF based on:

-	In the case of a BBF access: the Line ID together with an operator identifier of the operator administrating the Line ID.

-	In the case of a Cable access: the a HFC_Identifier together with an operator identifier of the operator administrating the HFC_Identifier.

NOTE 1:	Further description for how W-AGF obtain parameters required in AS and NAS message e.g. to build the SUCI is defined in BBF WT-456 [9] and CableLabs WR-TR-5WWC-ARCH [27].

4	If the AMF receives a SUCI, the AMF shall select an AUSF as specified in TS 23.501 [2] clause 6.3.4 based on SUCI. If 5G-GUTI is provided, there is no need to map SUCI to SUPI and steps 5-9 can be skipped.

Editor's note:	Steps 5-9 can be later consolidated into a box in reference to the SA WG3 specification.

5.	AMF sends an authentication request to the AUSF in the form of, Nausf_UEAuthentication_Authenticate. It contains the SUCI of the FN-RG. It also contains an indication that the W-5GAN has authenticated the FN-RG.

6.	AUSF selects a UDM as described in TS 23.501 [2], clause 6.3.8 and sends a Nudm_UEAuthentication_Get Request to the UDM. It contains the SUCI of the FN-RG and indication that the W-5GAN has authenticated the FN-RG.

7.	UDM invokes the SIDF to map the SUCI to a SUPI.

8.	UDM sends a Nudm_UEAuthentication_Get Response to the AUSF. It contains the SUPI corresponding to the SUCI. It also contains an indication that authentication is not required for the FN-RG.

9.	AUSF sends a Nausf_UEAuthentication_Authenticate Response to the AMF. This response from AUSF indicates that authentication is successful, but no Kseaf is included. The response contains the SUPI corresponding to the SUCI.

The procedure described in TS 23.502 [3] clause 4.2.2.2.3 may apply (the AMF decides if the Registration Request needs to be rerouted, where the initial AMF refers to the AMF).

10a.	AMF initiates a NAS security mode command procedure upon successful authentication. This is like that of the scenario of unauthenticated emergency calls for setting the NAS security algorithms.

The NAS security mode command is sent from the AMF to the W-AGF. The selected NAS security algorithms of integrity protection algorithm and of ciphering algorithm are set to NULL. This message is encapsulated in a N2 Downlink NAS message.

Editor's note:	The above selection of NULL security algorithms should be confirmed by SA WG3.

10b.	W-AGF responds to the AMF with a NAS Security Mode Complete message in a N2 Uplink NAS transport message. A NAS security context is created between W-AGF and AMF.

11a.	Upon receiving NAS Security Mode Complete, the AMF shall send an N2 Initial Context Setup Request message (Old AMF, UE Aggregate MBR, RG LWAC, GUAMI, Allowed NSSAI, UE security capability, Trace Activation, Masked IMEISV) to the W-AGF.

11b.	W-AGF notifies the to the AMF that the FN-RG context was created by sending a N2 Initial Context Setup Response.

12.	The AMF performs step 13-16 in TS 23.502 [3] clause 4.2.2.2.2.

At FN-RG registration to UDM, the Access Type non-3GPP access is used. The UDM, based on Access and Mobility Subscription information authorizes the FN-RG to access the 5GC. The AMF compares the list of serving areas it receives from the UDM against the ULI from the W-AGF to check if the location information is allowed for the FN-RG. The AMF may also interact with the PCF for obtaining the Access and Mobility policy for the FN-RG.

13.	The AMF sends the N2 Downlink NAS transport with NAS Registration Accept message (5GS registration result, 5G-GUTI, Equivalent PLMNs, Non-3GPP TAI, Allowed NSSAI, Rejected NSSAI, Configured NSSAI, 5GS network feature support, network slicing indication, Non-3GPP de-registration timer value, Emergency number lists, SOR transport container, NSSAI inclusion mode) to the W-AGF.

The following parameters are ignored by the W-AGF if received from the AMF: Emergency number lists, SOR transport container, NSSAI inclusion mode.

NOTE 2:	Further description on how W-AGF handles the parameters received from 5GC is provided in BBF WT-456 [9] and CableLabs WR-TR-5WWC-ARCH [27].

14.	The W-AGF sends a N2 Uplink NAS Registration Complete message back to the AMF when the procedure is completed. The W-AGF shall store the 5G-GUTI to be able to send it in potential later NAS procedures.

15.	The AMF performs step 23-24 in TS 23.502 [3] clause 4.2.2.2.2.

The W-AGF may continue by establishing PDU session(s) on behalf of the FN-RG.

**** Next Change ****

## 7.5	User Profile management procedures

When 5G-RG or FN-RG is used, the User Profile management procedures in TS 23.502 [3] clause 4.5 applies, with the differences described below:

-	The UE in TS 23.502 [3] clause 4.5 is replaced by 5G-RG or FN-RG.

-	When 5G-RG or FN-RG is connected via W-5GAN, steering of roaming information is not applicable, since roaming is not supported.

-	The SMF updates 5G-RG context and FN-RG context stored at W-AGF to modify the subscribed Session-TMBR.

-	The AMF updates 5G-RG context and FN-RG context stored at W-AGF to modify the RG Level Wireline Access Characteristics.

*** End of Changes ****