SA WG2 Meeting #135 	S2-1909757

14 - 18 Oct 2019, Split, Croatia	 (revision of S2-190xxxx)

Source:	CATT

Title:	New KI: Support of Network Slicing for UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: this paper proposes to add a new key issue on how to support network slicing for 5G ProSe.

Discussion

Network slicing is a mandatory feature supported in 5G system. During Registration procedure, a UE provides Requested NSSAI to 5GC. The 5GC determines the Allowed NSSAI and sent to UE based on the Requested NSSAI and the Subscribed S-NSSAI(s). The Allowed NSSAI includes S-NSSAIs the UE could use in the Serving PLMN for the current Registration Area. When the UE requests PDU Session, it shall indicate in the PDU Session Establishment procedure the S-NSSAI from Allowed NSSAI according to the NSSP in the URSP rule.

For the UE-to-Network Relay case, the Relay UE shall establish PDU Session to the 5GC and provide services for the Remote UE. The network slicing should be considered in this case.

This P-CR is proposed to add a new key issue about how to support network slicing for UE-to-Network Relay.

Proposal

It is proposed to add a new key issue as following: (all new text).

FIRST CHANGE

5.x	Key Issue x: Support of Network Slicing for UE-to-Network Relay

5.x.1	General Description

Network slicing is a mandatory feature supported in 5G system. During Registration procedure, a UE provides Requested NSSAI to 5GC, the 5GC determines the Allowed NSSAI for UE based on the Requested NSSAI and the Subscribed S-NSSAI(s). The Allowed NSSAI includes S-NSSAIs the UE could use in the Serving PLMN for the current Registration Area. When the UE requests PDU Session, it shall indicate in the PDU Session Establishment procedure the S-NSSAI from Allowed NSSAI according to the NSSP in the URSP rule.

For the UE-to-Network Relay case, the Relay UE shall establish PDU Session to the 5GC and provide services for the Remote UE. The network slicing should be considered in this case.

This key issue will study:

-	Whether and How to configure/provision the Remote UE and Relay UE with NSSAI?

-	Whether and How NSSAI is used for Relay UE discovery and (re)selection?

End of CHANGE