3GPP TSG-SA WG2 Meeting #135	S2-1910141

# Oct 14 - 18, 2019, Split, Croatia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0049 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Broadcast of Assistance Data by an LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to request from RAN, in last SA2 meeting, broadcast of assistance data by an LMF was added as supported in TS23.271. In the text, it is described “LCS stateless manner” in order to highlight that AMF does not retain LMF identifier which send Network assistance data. But in 23.273 there is no description on LCS state at all, it is better to make sure implementation of “LCS stateless manner” in the operation without using the word. And in operation of broadcasting network assistance data, LMF provides NG-RAN node information to indicate the destination which AMF forwards the information to, and it is the same method which is used at TS23.271, therefore AMF do not need to have any additional functionality to derterming the NG-RAN nodes to transfer the network assistance data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the procedure for broadcasting assistance data by an LMF to be “AMF LCS stateless” And remove editor’s note |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and misleading specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.14.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

6.14.1	Broadcast of Assistance Data by an LMF

The following procedure is used by the LMF to support broadcasting of network assistance data to target UEs. This procedure is not associated with a UE location session. It is used in an AMF to send network assistance data to an NG-RAN node for broadcasting by the NG-RAN node to target UEs. NG-RAN positioning procedures related to LMF and NG-RAN node communication are specified in TS 38.455 [15].

Figure 6.14.1-1: Broadcasting Network Assistance Data

1.	The LMF invokes the Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request the transfer of a Network Assistance Data message to an NG-RAN node (gNB or ng-eNB) in the NG-RAN. The service operation includes the Network Assistance Data message and the target NG-RAN node identity. The Network Assistance Data message includes assistance data which may be optionally ciphered.

Editor's note:	It is FFS if AMF can be used to determine the NG-RAN nodes to transfer the Network Assistance Data.

2.	The AMF forwards the Network Assistance Data message to the target NG-RAN node indicated in step 1 in an N2 Transport message. The AMF includes a Routing identifier, in the N2 Transport message, identifying the LMF.

3.	The NG-RAN node broadcasts the assistance data contained in the Network Assistance Data message.

**** End of Changes ****