3GPP TSG-SA2 Meeting #135	S2-1910571

# Split, Croatia, Oct 14th – Oct 18th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0056 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of 5GC-MT-LR Procedure for TS 23.273 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019-9-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Now, external AF or client can invoke the location request towards core network for positioning specific UE. If location is required for more than one UE, and in that case the NEF or H-GMLC receiving location request, shall verify whether the number of Target UEs in the LCS request is equal to or less than the Maximum Target UE Number of the LCS client. If Maximum Target UE Number is exceeded, the NEF or H-GMLC shall reject the LCS request. However, just rejecting the total target UE number of the location request is not reasonable. For example, the Maximum Target UE Number of the LCS client is 100, and the the number of Target UEs in the LCS request is 120, it may be much more reasonable to partially accept the 100 target UEs and reject the left 20 target UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case of a rejection of the received bulk location request, GMLC or NEF decides whether to completely or partly reject the received location request and, in case of a partial rejection, accepts a part of the received target UEs, processes the accepted target UEs and rejects the other part of the received target UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete MT-LR procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 6.8	Bulk Operation of LCS Service Request Targeting to Multiple UEs

The procedure described in this clause applies to 5GC_MT_LR and Deferred 5GC-MT-LR request targeting to a group of UE identified by an internal group ID, if available.

Figure 6.8.1: Bulk operation of LCS service request targeting to multiple UEs

1.	This step is the same as step 1 of clause 6.1.2 and step 1 of clause 6.3, with the difference that the LCS request is targeting a group of UE identified by a group ID. NEF/GMLC may map the external group ID to the list of UE ID (i.e. SUPI) using Nudm_SDM_Get (Group Identifier Translation, External Group Identifier) service operation.

2.	If deferred location is requested, the GMLC responds to the LCS Client / AF with an acknowledgment.

GMLC may decide whether completely or partially (i.e. accept part of the UE(s) within the group identified by the group ID in step 1) reject the location request. If GLMC deciedes to partially reject the location request, GMLC will respond to the LCS client /AF with a proper error cause.

Steps 3 to 5 are carried out once per UE.

3.	The GMLC invokes a Nudm_SDM_Get (LCS privacy, SUPI) service operation towards the UDM to get the privacy settings of the target UE. The GMLC checks the privacy settings. For the UE whose privacy setting does not allow it to be located, steps 4 and 5 are skipped. The GMLC may also subscribe to and receive notification of UE privacy profile updates according to steps 0 and 4 of clause 6.12.1.

4.	The GMLC invokes a Nudm_UECM_Get service operation towards the UDM of the target UE with SUPI of this UE. The UDM returns the current serving AMF ID to the GMLC.

5a.	If no AMF ID is returned at step 4:

-	if it is a deferred location request and the GMLC supports the storage of the LCS service request for a group of UE, the GMLC subscribes the UE registration status to the UDM using Nudm_EventExpoture_Subscribe service operation with the data key "SUPI";

-	otherwise, the GMLC returns an appropriate error message to the LCS client or AF.

5b. If the UDM returns the current serving AMF ID to the GMLC at step 4:

-	the GMLC initiates 5GC_MT_LR procedure (from step 4 onwards) as described in clause 6.1.3;

-	or the GMLC initiates Deferred 5GC-MT-LR Procedure (from step 4 onwards) as described in clause 6.3 with the difference that Step 8 is skipped.

6.	For the deferred location request, if any UE in the group didn't get its serving AMF ID at step 4, Tthe GMLC may store the LCS service request locally if the GMLC supports the storage of the LCS service request for a group of UE; otherwise, this step is skipped.

Further steps apply to the UEs of the group who was not registered to the network when the LCS service request is received at GMLC.

7.	UE performs the registration as described in clause 4.2.2 of TS 23.502, during which an AMF is selected to serve the UE, and the AMF ID is stored into UDM.

8. UDM notifies the GMLC who had subscribed the UE registration at step 4a using Nudm_ EventExpoture_Notify service operation, which includes "SUPI" and UE registration status.

9.	GMLC initiates 5GC_MT_LR procedure (from step 2 onwards) or Deferred 5GC-MT-LR Procedure (from step 2 onwards) as described in clause 6.1.3 or clause 6.3.

***** End of CHANGE *****