SA WG2 Meeting #S2-136 Ad-hoc	S2-2000689

13 - 17 January, 2020, Incheon, Korea	(was S2-19XXXX)

Source:	CATT

Title:	Solution for KI #5: Activating the traffic routing towards Local Data Network per AF request

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: this contribution proposes to add a new key solution on how to activate the traffic routing towards Local Data Network by the I-SMF insertion or reselection per AF request.

# Proposal

It is proposed to include the following solution in TR 23.748.

***** BEGIN 1st CHANGE *****

## 6.X	Solution #X: < I-SMF Insertion and Reselection per AF request >

### 6.X.1	Description

The following solution corresponds to the key issue #5 on activating the traffic routing towards Local Data Network per AF request as specified in clause 5.5.

This solution is able to support activating the traffic routing towards Local DN by the I-SMF insertion or reselection per AF request. In this solution, the AF creates and sends a request to 5GC, and the message is used to request the insertion or reselection of I-SMF according to content of this message (e.g. the target DNAI, N6 traffic routing info). After the PCF receives the notification regarding the modifications of AF, the PCF updates the PCC rule and notified the change to SMF. The AMF is responsible to select a suitable I-SMF according the indicator of the SMF (e.g. the target DNAI). Then the new I-SMF selects the DNAI corresponding UPF which acts as the PDU Session Anchor UPF to support the traffic routing towards Local DN. To support this PDU session with multiple PSAs, the UL CL/BP controlled by I-SMF is inserted into the data path of the PDU Session.

Figure 6.X.2-1 Non-roaming architecture with I-SMF insertion to the PDU Session in reference point representation, with UL CL/BP

### 6.X.2	Procedure

Figure 6.X.2-2: Solution – I-SMF Insertion and Reselection

1.	AF invokes a Nnef_TrafficInfluence_Create service operation to create a request, and the target DNAI(indicates to access the target Local DN, which is identified by a DNAI), N6 traffic routing info and other content (defined in clause 5.2.6.7 of TS 23.502[3]) are included in the AF request.

2.	The AF sends its request to the NEF, or it sends the request to PCF directly by invoking Nbsf_management_Discovery service or by configuration.

3.	The NEF stores the AF request information in the UDR.

4.	The PCF(s) that have subscribed to requests receive(s) a Nudr_DM_Notify notification regarding the modifications of AF. The service procedure is described in the step 4 in clause 4.3.6.2 of TS 23.502[3].

5.	The PCF updates the SMF with corresponding new PCC rule(s) for the PDU sessions by invoking Npcf_SMPolicyControl_UpdateNotify service operation. The service procedure is described in the step 5 in clause 4.3.6.2 of TS 23.502[3].

6.	The SMF invokes a Nsmf_EventExposure service operation if it can not serve the target DNAI, and the content of the message includes the target DNAI, N6 traffic routing info. This message triggers the AMF to select a suitable I-SMF.

If there is an I-SMF or V-SMF serving the PDU session, the target DNAI needs to be sent via the I-SMF or V-SMF.

7.	The AMF selects a new I-SMF which can serve the target DNAI.

8. 	The AMF sends a Nsmf_PDUSession_CreateSMContext Request to the new I-SMF, the request message includes the target DNAI.

9. The new I-SMF retrieves SM Context from the old I-SMF (in case I-SMF change) or SMF (in case I-SMF insertion) by invoking Nsmf_PDUSession_Context Request.

10. The new I-SMF selects a new I-UPF based on the received SM context, e.g. target DNAI, S-NSSAI, and UE location information.

11. The new I-SMF initiates a N4 Session Establishment to the new I-UPF.

12. The new I-SMF invokes Nsmf_PDUSession_Update Request (in case I-SMF insertion) or Nsmf_PDUSession_Create Request (in case I-SMF change) towards the SMF. The SMF initiates N4 Session Modification toward the PSA1. And The SMF responds Nsmf_PDUSession_Create Response or Nsmf_PDUSession_Update Response to the new I-SMF.

13. If the AF has subscribed to user plane management event notifications, the SMF notifies the AF of the PSA change by invoking Nsmf_EventExposure_Notify service operation.

14. The new I-SMF sends a Nsmf_PDUSession_CreateSMContext Response to the AMF.

15. In the case of I-SMF reselection, the AMF sends Nsmf_PDUSession_ReleaseSMContext Request to old I-SMF for the release of resources in old I-SMF.

16. The same procedure is performed as described in clause 4.23.9-1 starting from step 3 in TS 23.502[3].

### 6.X.3	Impacts on Existing Nodes and Functionality

FFS.

***** END of CHANGEs *****