3GPP TSG-SA2 Meeting #136-AH	S2-2000305

# Incheon, Korea (Republic Of), 13th Jan 2020 - 17th Jan 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1880 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PCF discovery via BSF to consider eSBA binding principles – AF/NEF/SCP re-selection functionality (23.503) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR addresses the re-selection functionality for consumers of the BSF so that they can have the same ability to perform PCF selection using a binding indication retrieved from the BSF as a consumer using the N7 interface to perform PCF selection. This is an alignment CR to CR 1986 (S2-2000304). In addition this CR adds the SCP as a possible entity, which may need to perform the above retrieval on behalf of the AF or NEF (delegated discovery described in 23.501 Annex E model D). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PCF may register the PCF binding indication with the PCF instance at the BSF, and the BSF then provides this information as part of the discovery procedure to AF or NEF or SCP on their behalf. If Binding indication is provided by the BSF, the AF or NEF or SCP on their behalf shall apply the NF consumer or SCP selection and reselection behaviour as described in TS 23.501 [2] Table 6.3.1.0-1. In this respect, a binding indication provides greater capability than a simple NF Set ID. This change replaces “PCF Set ID” with “PCF binding indication” to allow the AF, NEF, and other consumers of the BSF service to select a PCF to serve an existing PDU session consistent with the SMF selection of a PCF to serve an existing PDU session. NOTE: In addition to registering a binding indication at the BSF, the PCF registers PCF address(es) and PCF id at the BSF. This is to keep backward compatibility with R15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If an AF or NEF uses the BSF after the PCF instance within a set has changed, it will obtain an outdated PCF instance that may no longer be available. A binding indication provides more information for selection and re-selection than PCF Set ID alone. In addition not using the binding indication stored at the BSF would introduce inconsistent behaviour with the N7 interface. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2.1, 6.1.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 6.1.1.2	Binding an AF request targeting an UE address to the relevant PCF

##### 6.1.1.2.1	General

When multiple and separately addressable PCFs have been deployed, a network functionality is required in order to ensure that an AF needing to send policies about UE traffic identified by an UE address can reach over N5 the PCF holding the corresponding PDU Session information. This network functionality has the following characteristics:

-	It has information about the user identity, the DNN, the UE (IP or Ethernet) address(es), the DN information (e.g. S-NSSAI) and the selected PCF address for a certain PDU Session.

-	For IP PDU Session type, it shall receive information when an IP address is allocated or released for a PDU Session.

-	For Ethernet PDU Sessions supporting binding of AF request based on MAC address, it shall receive information when a MAC address is detected as being used by the UE over the PDU Session; this detection takes place at the UPF under control of SMF; This is defined in TS 23.501 [2] clause 5.8.2.

-	The functionality determines the PCF address, and the associated PCF instance ID and binding indication as defined in TS 23.501 [2] Table 6.3.1.0-1 if available, selected by the PCF discovery and selection function described in TS 23.501 [2], according to the information provided by the AF or the NEF.

A private IPv4 address may be allocated to different PDU sessions, e.g.:

-	The same UE IPv4 address is allocated to different PDU sessions to the same DNN and different S-NSSAI;

-	The same UE IPv4 address is allocated to different PDU sessions to the same S-NSSAI and different DNN.

In the case of private IPv4 address being used for the UE, the AF or the NEF may send DNN and DN information (e.g. S-NSSAI), in addition, in Npcf_PolicyAuthorization_Create request and Nbsf_Management_Discovery request. The DNN and DN information can be used by the PCF for session binding, and they can be also used to help selecting the correct PCF.

##### 6.1.1.2.2	The Binding Support Function (BSF)

The BSF has the following characteristics:

-	The BSF stores information about the user identity, the DNN, the UE (IP or Ethernet) address(es), the DN information (e.g. S-NSSAI) and the selected PCF address, and the associated PCF instance ID andbinding indication if available (see subclause 6.3.1.0 of TS 23.501 [2]) for a certain PDU Session. This information is stored internally in the BSF.

-	The PCF registers, updates and removes the binding information from the BSF using the Nbsf management service operations defined in TS 23.502 [3].

-	The PCF ensures that it is updated each time an IP address is allocated or de-allocated to the PDU Session or, for Ethernet PDU Sessions supporting binding of AF request based on MAC address, each time it has been detected that a MAC address is used or no more used by the UE in the PDU Session.

-	Based on operator's policies and configuration, the PCF determines whether the same PCF shall be selected for the SM Policy associations to the same UE ID, S-NSSAI and DNN combination in the non-roaming or home-routed scenario.

NOTE 1:	This applies to usage monitoring.

-	The selected PCF (if needed) downloads the user profile from the UDR as described in TS 23.502 [3] 4.16.4 step 2. If usage monitoring is enabled for the user, and based on operator's policies, the PCF checks if the BSF has already existing PCF serving the combination of SUPI, S-NSSAI, DNN.

-	If no such PCF is found the PCF shall register itself to the BSF as described above in this clause.

-	Else if an existing PCF is found for the above combination, the PCF shall return to the SMF the PCF ID of the existing PCF and a redirection indication.

NOTE 2:	The assumption is that for DNN, S-NSSAI combinations where usage monitoring be applied, the same BSF instance or the same BSF SET is selected for all UE PDU sessions to the same DNN, S-NNSAI.

-	For retrieval binding information, any NF, such as NEF or AF or SCP on their behalf, that needs to discover the selected PCF address(es), and associated PCF instance ID and binding indication if available (see subclause 6.3.1.0 of TS 23.501 [2]) for the tuple (UE address, DNN, S-NSSAI, SUPI, GPSI) (or for a subset of this Tuple) uses the Nbsf management service discovery service operation defined in TS 23.502 [3].

-	The NF may discover the BSF via NRF or based on local configuration. In case of via NRF the BSF registers the NF profile in NRF. The Range(s) of UE IPv4 addresses, Range(s) of UE IPv6 prefixes supported by the BSF may be provided to NRF.

-    If the NF or SCP on its behalf received PCF binding indication, it shall apply the NF consumer or SCP selection and reselection behaviour as described in TS 23.501 [2] Table 6.3.1.0-1.

-

-	For an ongoing NF service session, the PCF may provide binding indication to the NF (see clause 6.3.1.0 of TS 23.501 [2]). This binding indication shall then be used instead of any PCF binding indication and/or a PCF instance ID received from the BSF.

The BSF may be deployed standalone or may be collocated with other network functions, such as PCF, UDR, NRF, SMF.

NOTE 3:	Collocation allows combined implementation.

End of changes