3GPP TSG-WG SA2 Meeting #136-AH 	S2-2000433

# Incheon, South Korean, 13th - 17th January 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0062 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location information for UE behind 5G-RG using trusted Non-3GPP based solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In case of Trusted non-3GPP, current location information for UE is TNAP Identifier, a UE local IP address (used to reach the TNGF), and optionally UDP or TCP source port number (if NAT is detected). However, in case UE is behind 5G-BRG via W-5GBAN using trusted Non-3GPP based solution, Global Line ID, which is a more accurate location, can be used as location information as well. Similarly, in case UE is behind 5G-CRG via W-5GCAN using trusted Non-3GPP based solution, Global Cable ID, which is a more accurate location, can be used as location information as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the interactions between two AMFs to receive User Location information during UE registration procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no accurate location information for UE which behind 5G-RG via W-5GAN using trusted Non-3GPP based solution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

4.10	UE behind 5G-RG and FN-RG.

An RG connecting via W-5GAN or NG-RAN access towards 5GC can provide connectivity for a UE behind the RG to access an N3IWF or TNGF. It is assumed that the UE is 5G capable, i.e. supports untrusted non-3GPP access and/or trusted non-3GPP access. This allows the RG, W-5GAN and the RG's connectivity via 5GC to together act as untrusted/trusted N3GPP access to support UEs behind the RG.

When FN-RG/5G-RG is serving a  UE, the control and user plane packets of the  UE is transported using a FN-RG/5G-RG IP PDU session and then from PSA UPF of that PDU session to an IWF. A single FN-RG/5G-RG IP PDU session can be used to serve multiple UEs.

Figure 4.10-1 shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via TNGF where the combination of 5G-RG, W-5GAN and UPF serving the 5G-RG is acting as a trusted Non-3GPP access network.

NOTE 1:	FN-RG and W-5GAN acting as trusted Non-3GPP access is not considered in this specification as it is assumed that FN-RG does not support EAP-based access control (e.g. 802.1X).

Figure 4.10-1: Non-roaming architecture for UE behind 5G-RG using trusted N3GPP access

The 5G-RG can be connected to 5GC via W-5GAN, NG-RAN or via both accesses. The UE can be connected to 5GC via 5G-RG, NG RAN or via both accesses.

The TNGF and Ta reference point are defined in TS 23.501 [2].

NOTE 2:	The reference architecture in figure 4.10-1 only shows the architecture and the network functions directly connected to W-5GAN or TNGF, and other parts of the architecture are the same as defined in TS 23.501 [2], clause 4.2.

NOTE 3:	The reference architecture in figure 4.10-1 supports service based interfaces for AMF, SMF and other NFs not represented in the figure.

NOTE 4:	The two N2 instances in Figure 4.10-1 apply to a single AMF for a 5G-RG which is simultaneously connected to the same 5G Core Network over 3GPP access and W-5GAN.

For a UE behind a 5G-RG, the Registration procedure as defined in clause 4.12a.2.2, TS 23.502 [3] applies via 5G-RG replacing TNAP with the following difference:

- In step 3, 5G-RG, i.e. TNAP in the procedure, may send its 5G-GUTI to TNGF over AAA message;

- In step 6b, if TNGF identify the AMF for UE and AMF for 5G-RG belongs to the same PLMN based on the received 5G-GUTI for 5G-RG, TNGF may send the 5G-GUTI for 5G-RG to AMF for UE via N2 message;

- After step 15a, AMF for UE may request 5G-RG context from AMF for 5G-RG, as specified in Table 5.2.2.2.2-1 TS 23.502[3], which include User location information for 5G-RG, i.e. Global Line Identifier/Global Cable Idenfier for 5G-RG. AMF for UE store the received Global Line Identifier/Global Cable Idenfier for 5G-RG as User location information for UE.

  - AMF may invoke the Namf_Communication_UEContextTransfer specified in clause 5.2.2.2.2 TS23.502[3], which include 5G-GUTI for 5G-RG, to the AMF for 5G-RG identified by the 5G-GUTI for 5G-RG to request 5G-RG Context from AMF for 5G-RG.

  - Correspondingly, the AMF for 5G-RG response via the Namf_Communication_UEContextTransfer Response message, specified in clause 5.2.2.2.2 TS23.502[3], which include 5G-RG Context.

**** End of Change ****