3GPP SA WG2 Meeting #S2-136AH	S2-2000523

13 - 17 January, 2020, Incheon, Korea

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0371 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | session binding using DS-TT MAC and bridgeID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | S2-1911699 provides analysis of the background. MAC addresses of Ethernet ports can be configured for local usage, it gives MAC address of a port is unique locally (e.g. within a bridge). However, when among different virtual bridges, the local used MAC address of a port can be the same as a MAC that is used by other bridge ports, therefore Bridge ID is needed together with MAC address of a port to identify a port. In order for PCF to find the PDU sessions based on AF request, PCF needs to store the information about bridge ID, DS-TT MAC addresses and port numbers. PCF stores the information about MAC address of DS-TT port and Bridge ID (i.e. bridge MAC address) for session binding purpose. PCF shall support transpatently transfer of port management container and TSCAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For time sensitive communicaiton, MAC address of a DS-TT port, and Bridge ID are used by PCF for session binding - SMF provides MAC addresses of DS-TT, and bridge ID to PCF. This is an implicit indication to PCF that the PDU session is for Time Sencitive Communication. - PCF stores information about bridge ID, MAC addresses of DS-TT ports. - This information shall be provided to TSN AF first, prior to PCF can receive a request from the TSN AF. TSN AF address is configured at PCF based on DNN and NSSAI. - PCF shall support transparently exchange of port management information container between TSN AF and DS-TT or NW-TT, as defined in TS 23.501 [2] clause 5.28.3. PCF shall support transparently transfer of TSCAI received from TSN AF to SMF and 5G-AN as defined in TS 23.501 [2] clause 5.27.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TSN specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2, 6.1.3.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

##### 6.1.3.2.2	Session binding

Session binding is the association of the AF session information to one and only one PDU Session. The PCF shall perform the session binding, which may take the following PDU Session parameters into account:

a)	For an IP type PDU Session, the UE IPv4 address and/or IPv6 network prefix, in addition when using W-5GAN the description in TS 23.316 [27] applies;

For an Ethernet type PDU Session, the UE MAC address(es);

For the Ethernet type PDU session used for time sensitive communication, see clause 6.1.3.23;

b)	The UE identity (e.g. SUPI), if present;

c)	The information about the Data Network (DN) the user is accessing, i.e. DNN, if present.

*** NEXT CHANGES ***

#### 6.1.3.23	Support of integration with Time Sensitive Networking

Time Sensitive Networking (TSN) support is defined in TS 23.501 [2], where the 5GS represents virtual TSN bridge(s) based on the defined granularity model. The TSN AF and PCF interact to perform QoS mapping as described in clause 5.28.4 of TS 23.501 [2].

The PCF provides the following parameters to the TSN AF: Port Management Container, port numbers associated with the NW-TT and DS-TT,a UE MAC address (i.e. MAC address of the DS-TT port), and a bridge ID (i.e. bridge MAC address). The TSN AF may use this information to construct IEEE managed objects, to interwork with IEEE TSN networks. This information shall be provided to TSN AF first, prior to PCF can receive a request from the TSN AF. TSN AF address is configured at PCF based on DNN and NSSAI.

Note1: SMF provides port numbers and MAC addresses associated with DS-TT ports, and bridge ID to PCF as defined in TS 23501, clause 5.28.1. This is an imlicity indication to PCF that the PDU session is for Time Sencitive Communication.

Note2: if MAC address of DS-TT port is not globle unique, a bridge ID (i.e. bridge MAC address) is needed together with MAC address of DS-TT port.

The TSN AF decides the TSN QoS information (i.e. priority and delay) based on the received the configuration information of 5GS Bridge from the CNC as defined in clause 5.28.2 of TS 23.501 [2] and the bridge delay information at the TSN AF.

The PCF receives a request from the TSN AF that includes UE MAC address (i.e. MAC address of the DS-TT port) and bridge ID (e.g. bridge MAC address) for PDU session and the TSN QoS parameters, i.e. priority and delay. The PCF performs Session binding using the UE MAC address (i.e. MAC address of the DS-TT port) and bridge ID (e.g. bridge MAC address), and then the PCF derives the TSN QoS parameters into a 5QI. The PCF generates a PCC Rule with service data flow filter containing the UE MAC address and the mapped 5QI. The SMF binds the PCC Rule to a QoS Flow as defined in clause 6.1.3.2.4.

PCF shall support transparently exchange of port management information container between TSN AF and DS-TT or NW-TT, as defined in TS 23.501 [2] clause 5.28.3.  PCF shall support transparently transfer of TSCAI received from TSN AF to SMF and 5G-AN as defined in TS 23.501 [2] clause 5.27.2.

*** END CHANGES ***