3GPP TSG-SA2 Meeting #136-AH	S2-2000591

# Incheon, Korea (Republic Of), 13th Jan 2020 - 17th Jan 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0080 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameters Provisioned to the UE for V2X Communications over Uu reference point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 5.1.3.1, mapping of the V2X service types (e.g. PSIDs or ITS-AIDs of the V2X application) to the following: - PDU Session Type (i.e. IP type or Unstructured type); - Transport layer protocol (i.e. UDP or TCP, only applicable for IP PDU Session type); - SSC Mode; - S-NSSAI(s); - DNN(s). While in TS 23.503, besides the above elementss, it also includes “Time Window” and “Location Criteria” parameters, which are also applicable for V2X services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add parameters provisioned to the UE for V2X communications over Uu reference point in section 5.1.3.1 to align with the related UE Local Configuration specified in TS 23.503. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Paremeters for V2X Communications over Uu reference point are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First change * * * *

5.1.3	Authorization and provisioning for V2X communications over Uu reference point

5.1.3.1	Policy/Parameter provisioning

The following set of information may be provisioned to the UE for V2X communications over Uu reference point:

1)	Mapping of the V2X service types (e.g. PSIDs or ITS-AIDs of the V2X application) to:

-	PDU Session Type (i.e. IP type or Unstructured type);

-	Transport layer protocol (i.e. UDP or TCP, only applicable for IP PDU Session type);

-	SSC Mode;

-	S-NSSAI(s);

-	DNN(s);

-	Time Window;

-	Location Criteria.

NOTE:	Above listed information elements are optional and used by UE as UE Local Configuration specified in TS 23.503 [16].

2)	Validity timer indicating the expiration time of the V2X Policy/Parameter.

The following sets of information may be provisioned to the UE and is applicable for V2X communications over both LTE-Uu and Uu reference points:

1)	Mapping of the V2X service types (e.g. PSIDs or ITS-AIDs of the V2X application) to V2X Application Server address information (consisting of IP address/FQDN and transport layer port#) for unicast.

2)	List of FQDNs or IP addresses of the V2X Application Servers, associated with served geographical area information and list of PLMNs that the configuration applies to.

# * * * End of changes * * * *