SA WG2 Meeting #136AH 	S2-2000707

13 - 17 January 2020, Incheon, South Korea	 (resubmission of S2-1911489)

Source:	CATT

Title:	Solution for ProSe QoS

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This paper proposes a solution for ProSe QoS to address KI#2 QoS aspect of TR 23.752.

1	Introduction

It was agreed at SA2#135 meeting that the 5G ProSe QoS shall take Rel-16 V2X QoS as baseline:

-	QoS handling for V2X communication over NR PC5 reference point defined in clauses 5.4.1 to 5.4.4 of TS 23.287 [5] is used as basis for supporting PC5 direct communication.

-	The standardized PQI values defined for NR PC5 in TS 23.287 [5] can be used to support performance requirements defined in clause 7.6.2 of TS 22.261 [3] for interactive services if applicable.

and furthermore in KI#2:

-	For QoS support differences with what is documented in TS 23.287 [5] clause 5.4 will be documented.

In this contribution, the ProSe QoS solution is proposed in which the differences with TS 23.287 clause 5.4 are highlighted.

2	Proposal

It is proposed to capture the following solution into TR 23.752.

FIRST CHANGE

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  | X |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X | X |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |
| 6 |  |  | X |  |  |  |  |  |
| 7 |  |  | X |  |  |  |  | X |
| X |  | x |  |  |  |  |  |  |

Second CHANGE

6.X	Solution #X: QoS handling for ProSe communication

6.X.1	General Description

In order to support QoS handling for ProSe communication, the mechanism defined in TS 23.287 [y] clause 5.4 is reused with the following differences:

-	For LTE based PC5, the QoS handling is defined in TS 23.303 [x], based on ProSe Per-Packet Priority (PPPP) and ProSe Per-Packet Reliability (PPPR).

-	V2X Packet Filter Set is not applied to QoS handling for ProSe communication.

For commercial interactive services, PQI = 82 can be reused. For public safety services, considering the higher priority and higher reliability, new PQI values needs to be defined. The one-to-one mapping of standardized PQI values to PC5 QoS characteristics in table 5.4.4-1 of TS 23.287 [y] is modified as followings:

Table 5.4.4-1: Standardized PQI to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 3 | 20 ms | 10-4 | N/A | 2000 ms | Platooning between UEs – Higher degree of automation; Platooning between UE and RSU – Higher degree of automation |
| 2 | (NOTE 1) | 4 | 50 ms | 10-2 | N/A | 2000 ms | Sensor sharing – higher degree of automation |
| 3 |  | 3 | 100 ms | 10-4 | N/A | 2000 ms | Information sharing for automated driving – between UEs or UE and RSU - higher degree of automation |
| 31 |  | 0.7 | 75 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| 32 |  | 2 | 100 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 33 |  | 1.5 | 100 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 55 | Non-GBR | 3 | 10 ms | 10-4 | N/A | N/A | Cooperative lane change – higher degree of automation |
| 56 |  | 6 | 20 ms | 10-1 | N/A | N/A | Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU |
| 57 |  | 5 | 25 ms | 10-1 | N/A | N/A | Cooperative lane change – lower degree of automation |
| 58 |  | 4 | 100 ms | 10-2 | N/A | N/A | Sensor information sharing – lower degree of automation |
| 59 |  | 6 | 500 ms | 10-1 | N/A | N/A | Platooning – reporting to an RSU |
| 34 |  | 0.5 | 60 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| 35 |  | 5.5 | 200 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |
| 82 | Delay Critical GBR | 3 | 10 ms | 10-4 | 2000 bytes | 2000 ms | Cooperative collision avoidance; Sensor sharing – Higher degree of automation; Video sharing – higher degree of automation; Gaming or Interactive Data Exchanging |
| 83 | (NOTE 1) | 2 | 3 ms | 10-5 | 2000 byte | 2000 ms | Emergency trajectory alignment; Sensor sharing – Higher degree of automation |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. Editor's note: It is FFS if GBR and Delay Critical GBR can also be used for broadcast and groupcast. |  |  |  |  |  |  |  |

NOTE 1:	For Standardized PQI to QoS characteristics mapping, the table will be extended/updated to support service requirements for other identified V2X services.

NOTE 2:	The PQIs may be used for other services than V2X.

NOTE 3:	A PQI may be used together with an application indicated priority, which overrides the Default Priority Level of the PQI.

6.X.2	Procedures

The QoS handling defined in clauses 5.4.1.2 to 5.4.1.4 of TS 23.287 [y] is used.

6.X.3	Impacts on Existing Nodes and Functionality

UE impact:

-	UE is enhanced to support QoS handling for ProSe communication over NR based PC5 reference point.

END of CHANGES