3GPP TSG-SA WG2 Meeting #136AH 	S2-2000768

Incheon, Korea, 13 – 17 January 2020	 (revision of S2-xxxxxxx)

Source:	CATT

Title:	Solution for KI#2: Mitigating the load for Data Collection

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes a solution to mitigate the load in 5GC for data collection.

Discussion

In the KI#2, multiple NWDAF instances deployed in the same PLMN may request same data collection from same NFs, which introduces lots of signalling for same data report. This paper therefore proposes a new solution which allows same data to be utilized by multiple NWDAF instance.

This solution relies on a shared data storage function to cache data collected from 5GC NFs, so that any data available in the data storage function can be re-used.

Proposal

It is proposed to include the new solution into the TR 23.700-91.

FIRST CHANGE (all new text)

6.X	Solution for Key Issue #: mitigation of the load for Data Collection

6.X.1	Description

Editor's note:	Describe the solutions. (sub) clause(s) may be added to capture details, procedural flow etc.

There may be multiple NWDAF instances deployed in the same PLMN and such NWDAF instances may request same data from same OAM/NFs, thus it is unnecessary for an OAM/NF to report duplicated data to all such NWDAFs.

In order to avoid different NWDAF instances to collect same data for different analysis purposes, a new NWDAF Data Storage Function (NDSF) is introduced to cache the data collected from OAM/NFs, where any authorized NWDAF instance can retrieve the cached data from NDSF. By this way, the signalling load for same data collection can be mitigated. The following figure 6.x-1 shows the data collection architecture supporting NDSF.

Editor's note:	Whether the NDSF is a NWDAF functionality or a standalone network function is FFS.

NOTE:	As NWDAFs in a PLMN may be deployed in different operator’s data centers, there may be multiple NDSFs deployed in a PLMN, e.g. each per data center.

The NDSF supports following functionalities:

-	Data collection from 5GC NFs and OAM;

-	Caching the collected data;

-	Exposure of the cached data to authorized NWDAF instance.

6.X.2	Procedures

Figure 6.X-2: Data collection with support of NDSF

1.	When a NWDAF needs to collect network data from OAM/NFs, it invokes Nndsf_EventExposure_Subscribe (NF ID, Event ID, Target of Event Reporting, other information related to data collection ) to retrieve data from the NDSF instead of sending request to the OAM/NFs directly.

2.	The NDSF may reject the subscription request if the NWDAF is not allowed to retrieve data from the NDSF. In this case, the NWDAF may need to collect data from the OAM/NFs directly.

If the NDSF has no related network data, it invokes Nnf_EventExposure_Subscribe (NF ID, Event ID, Target of Event Reporting, other information related to data collection ) to collect the data from the OAM/NFs identified by the NF ID.

3.	The NDSF receives and caches the data reported by the OAM/NFs.

4.	The NDSF notifies the NWDAF 1 of the requested data.

5.	Another NWDAF (e.g. NWDAF 2) may request same data collection from the NDSF.

6.	As the same data has been cached in the NDSF, the NDSF can directly respond the NWDAF 2 with the cached data.

7.	The NWDAF 1 can send unsubscribe request to the NDSF if current data collection is not needed any longer.

8.	Other NWDAFs may also unsubscribe network data from the NDSF.

9.	If all NWDAFs subscribing same data collection unsubscribe from the NDSF, the NDSF can send Nnf_EventExposure_unSubscribe to the related OAM/NFs.

6.X.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

End of CHANGE