SA WG2 Meeting #136AH	S2-2001106

# Jan 13th-17th, 2020 ; Incheon, Korea		                               	  (revision of S2-2001066)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 1833 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access type and RAT type per Non-3GPP accesses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is a companion paper of 23.502 CR 1950 to define how RAT Type is determined for Non 3GPP 5G AN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Depending on the Non 3GPP 5G AN technology and on local configuration, the 5G AN may at the establishment of a N2 connection for a UE provide the 5GC with RAT Type information giving information on the features of the Non 3GPP Access (e.g. the Trusted access relies on WLAN, the Wireline access corresponds to Cable, etc…). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete definition of RAT Type in BBF / Cable cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 23.502 CR 1950 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The first change touches the same clause than CR 1832 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 10.1	User Location Information

The User Location Information may correspond to:

-	In the case of W-5GCAN: HFC node ID.

NOTE 1:	HFC node ID identifies the point of attachment of the 5G-CRG.

-	In the case of W-5GBAN: Line ID and operator identifier of the operator administrating the Line ID value.

NOTE 2:	A combination of Line ID and operator identifier identifies the attachment point of the 5G-BRG.

An indication of whether the ULI corresponds to a DSL or to a PON line may also be provided.

-	In the case of 5G-RG connected via 3GPP access: Cell Information (as described in TS 23.502 [3] clause 4.10 and TS 23.401 [24] clause 5.9.1).

NEXT CHANGE (2)

### 4.7.x	Dedicated RAT types for Wireline access

The AMF, as described in TS 23.501 [2] clause 5.3.2.3, determines the RAT Type for Wireline access, taking into account the Global W-AGF Node ID and possibly ULI information provided by the W-AGF. This RAT Type may allow to distinguish between Wireline-Cable access, Wireline-DSL access and Wireline-PON access.

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES