SA WG2 Meeting #S2-136	S2-1910929

18 - 22 November, 2019, Reno, Nevada, USA

Source:	Thales S.A

Title:	Evaluation of Solutions and Conclusions for KI#1

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_5GSAT_ARCH/Rel.16

Abstract of the contribution: Evaluation of Solutions and Conclusions for KI#1

# Discussion

- Solution 1 provides a solution with static coverage;

- Solution X provides a solution based on the location of the UE and on the geographical definition of the cells.

- It is proposed to elaborate clause 7.1 on the basis of the evaluation of this function:

  - Solution #1 is simple for the 3GPP system, but it relies on so called static TA concept.

  - Solution #X is more elaborated but provides additional answers with respect to questions  addressing mobility managements & associated services (operational & management, location and regulatory).

It is proposed to reflect these aspects in the conclusions part of the TR 23.737 in section 8.1 as shown below.

# Proposal

It is proposed to modify clause  7.1 & 8.1 as follows:

***** The 1st change *****

## 7.1	Evaluation of solutions for Key Issue #1: Mobility management with large satellite coverage areas

1.

For Key Issue #1 - " Mobility management with large satellite coverage areas " Candidate Solutions have been proposed, Candidate solutions #1 and #X all serve the purpose of mitigating the large coverage areas of satellite access, whether at global scale (for NGSO satellite systems with embarked NG-RAN) or generated by large radio coverage by satellite beams.

Candidate Solution #1 requires that satellites capable of projecting static coverage area on Earth surface. Satellite capabilities are however outside of the scope of 3GPP specifications. Candidate Solution #1 requires the satellite NG RAN to be able to dynamically change the SIB. The size of the satellite coverage is also a factor which imposing constraints on the satellite definition and could imply the introduction of positioning functionalities in the satellite network.

Candidate Solution #X requires that UEs are capable of locating themselves, and that cells are geographically defined. This presents also the advantage of not impacting mobility management procedures.  NG-RAN would have to be capable of geographically managing and handling cells. For NG-RAN embarked in NGSO satellites, NG-RAN would have to be provided their position and to dynamically change the SIB. This Candidate Solution is also applicable Key Issues 6 and 10.  No assumptions are imposed on whether the satellite coverage is fixed or not, and on the size of the satellite coverage.

In both cases CellID remains the location information that can be used for operations & management services, for location service and for regulatory services. The size and shape of the satellite cell remains the determining acceptability parameter.

***** end of change *****

***** The 2nd change *****

## 8.1 	Conclusion on solutions for Key Issue #1

.

Following the evaluation for Key Issue #1 “Mobility management with large satellite coverage areas” in clause 7.1, it is proposed to use Candidate Solutions #1 and #X for the basis of normative work.

NOTE: 	As the Candidate Solutions rely on an approach for which some features are defined in TSG-RAN WGs, the specification of this solution during the normative phase shall take into account inputs received from TSG-RAN WGs.

***** end of change *****