SA WG2 Meeting #136	S2-1910948

# Nov 18th-22nd, 2019 ; Reno, US	                               			  (revision of S2-1908771)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1670 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to N16a for the transfer of N4 requests from SMF / responses from local UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, NTT DOCOMO? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A) In current version of 23.502, when ISMF notifies SMF of DNAI insertion (via Nsmf_PDUSession_Update Request initiated by ISMF), the SMF sends a N4 request in Nsmf_PDUSession_Update response but needs to wait for a N4 response which is not supported by current spec; It is better to have - First a N16a Update request initiated by ISMF carrying the notification of DNAI addition / removal / change - and then a N16a Nsmf_PDUSession_Update Request initiated by SMF carrying the N4 request from SMF: followed by a Nsmf_PDUSession_Update Response carrying the N4 Response Cf 29502 CR 203 agreed in C4-194029 # B) this SA2 CR aligns with 29.502 CR(s) where Basically the SMF does as if the local UPF / UL CL were under its control, but with the N4 signalling carried over N16; the SMF may provide N4Information that may contain multiple occurences of a (N4 Message Type, N4 Message payload and DNAI information) - 29.502 CR0160 = C4-193550 (Insertion of a PSA and UL CL/BP into the data path of a PDU session with an I-SMF) - 29.502 CR 161 - Etc… C) SMF always reports allocated or released UE IP addresses (the corresponding PCRT is always set) D) Following sentences are incorrect “the SMF updates the (R)AN with the new I-UPF CN Tunnel Info” (cannot be the SMF but should be I-SMF) E) the actual change of User Plane path enforced by I-SMF under request of SMF may have to take place after the SMF has (early) notified the AF |  |  |  |  |  |  |  |  |
| Summary of change: |  | As defined in the Reason for change (§4.23.9) + § 4.23.6.3 renamed and aligned |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to properly support N4 exchange over N16a |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.9.0 (new) ; 4.23.9.1 ; 4.23.9.2 ; 4.23.9.3; 4.23.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev 1 adding - the actual change of User Plane path enforced by I-SMF under request of SMF may have to take place after the SMF has (early) notified the AF. See changes in yellow in § 4.23.9.3 - change 2 : §4.23.6.3 aligned |
| --- | --- |

FIRST CHANGE

### 4.23.9	Branching Point or UL CL controlled by I-SMF

#### 4.23.9.0	Overview

The procedures in this clause describe the Addition, Removal and Change of PDU Session Anchor (PSA2), Branching Point or UL CL controlled by I-SMF. They all rely on following principles:

1.	When the I-SMF is inserted (e.g. as described in clause 4.23.7 or 4.23.11), the I-SMF provides the DNAI list it supports to the SMF. This list is assumed to remain constant during the N16a association between the I-SMF and the SMF for a PDU Session.

2.	Based on the DNAI list information received from I-SMF, the SMF may then at any time provide or update the list of DNAI(s) of interest for this PDU Session to I-SMF. This may take place e.g. when the I-SMF provides the DNAI list it supports or when new or updated or removed PCC rule(s) is/are received by the SMF as defined in clause 4.23.6. This list of DNAI(s) of interest for this PDU Session indicates to the I-SMF the list of DNAI(s) candidate for local traffic steering within the PDU Session.

An indication of whether Multi-homing is possible is also provided to the I-SMF, and the I-SMF uses this information to decide whether multi-homing can be used for the PDU Session.

3.	Whenever the I-SMF has inserted or removed or changed a local offload capability the I-SMF invokes a Nsmf_PDUSession_Update Request to indicate to the SMF the list of corresponding DNAI(s). Based on this indication the SMF invokes a Nsmf_PDUSession_Update Request to send the corresponding N4 rules towards the local UPF(s) via the I-SMF.

4.	Then SMF may then at any time invoke a Nsmf_PDUSession_Update Request to send N4 rules towards the local UPF(s) via the I-SMF.

5.	The I-SMF may at any time send a Nsmf_PDUSession_Update Request to forward to the SMF N4 events received from a local UPF; this may e.g. correspond to traffic reporting.

#### 4.23.9.1	Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF

This clause describes a procedure to add a PDU Session Anchor and Branching Point or UL CL controlled by I-SMF.

Figure 4.23.9.1-1: Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF

1.	UE has an established PDU Session with a UPF including the PDU Session Anchor 1, which is controlled by SMF.The I-SMF and an I-UPF controlled by I-SMF have already been inserted for the PDU Session. Events described in item 1 and 2 of clause 4.23.9.0 have taken place

When the I-SMF is inserted, the I-SMF provides the DNAI list it supports to SMF. Based on the DNAI list information received from I-SMF, the SMF provides the DNAI(s) of interest for this PDU Session to I-SMF e.g. immediately or when (a) new or updated or removed PCC rule(s) is/are received as defined in clause 4.23.6. This DNAI(s) of interest for this PDU Session indicates to the I-SMF the list of DNAI(s) to be considered for local traffic steering within the PDU Session.

An indication of whether or Multi-homing is possible is also provided to the I-SMF, and the I-SMF uses this information to decide whether multi-homing can be used for the PDU Session.

2.	At some point, using the list of DNAI(s) of interest for this PDU Session received from the SMF, the I-SMF decides to establish a new PDU Session Anchor e.g. due to UE mobility. The I-SMF selects a UPF and using N4 establishes the new PDU Session Anchor 2 (PSA2) of the PDU Session. During this step:

-	(if needed) the PSA2 CN Tunnel Info of the local N9 termination on the PSA2 may be determined,

-	In the case of IPv6 multi-homing applies to the PDU Session, a new IPv6 prefix corresponding to PSA2 is allocated by the I-SMF or by the UPF supporting the PSA2.

3.	The I-SMF may select a UPF that will be acting as UL CL or Branching Point and replace the current I-UPF.

If a new UPF that will act as UL CL/Branching Point is selected (i.e. the existing I-UPF is replaced), the I-SMF uses N4 establishment to provide the 5G AN Tunnel Info, the PSA1 and (where applicable) PSA2 CN Tunnel Info to the new UPF and updates also the (R)AN with the Tunnel Info for the Branching Point or UL CL.

NOTE 1:	If the Branching Point or UL CL and the PSA2 are co-located in a single UPF then steps 2 and 3 can be merged.

4.	The I-SMF invokes Nsmf_PDUSession_Update Request (Indication of UL CL or Branching Point insertion, IPv6 prefix @PSA2, DNAI(s) supported by PSA2, DL Tunnel Info of the new UL CL/Branching Point if any) to SMF. Whether the UL CL/Branching Point and PSA2 are supported by the same UPF is transparent to the SMF.

The I-SMF informs the SMF that a UL CL or Branching Point is inserted, the I-SMF provides DNAI(s) supported by PSA2 to the SMF. The DL Tunnel Info of UL CL/Branching Point is provided to SMF if a new UPF is selected to replace I-UPF in step 3.

In the case of multi-homing, the IPv6 prefix @PSA2 is also provided to SMF.

The SMF performs the Session Management Policy Modification procedure as defined in clause 4.16.5 to provide the new allocated IPv6 prefix to the PCF. The SMF may also send a notification to the AF, as described in clause 4.3.6.3.

The DNAI(s) supported by PSA2 may be used by the SMF to determine which PCC rules are to be applied at UPF(s) controlled by the I-SMF. The SMF acknowledges the Nsmf_PDUSession_Update from the I-SMF

5.	If a new DL Tunnel Info of UL CL/ Branching Point has been provided in step 4, the SMF updates the PSA1 via N4 with the CN Tunnel Info for the downlink traffic. Now the downlink packets from PSA1 are sent to UE via the new UPF which will act as Branching Point/UL CL. The SMF may also update the forwarding rules in PSA1 if some traffic is to be moved to UPFs controlled by I-SMF.

6.	The SMF provides I-SMF with N4 information for the PSA and for the UL CL with a SMF initiated Nsmf_PDUSession_Update Request (set of (N4 information, involved DNAI, Indication of no DNAI change, Indication of no local PSA change)). The SMF generates N4 information for local traffic handling based on PCC rules and CHF requests that will be enforced by UPFs controlled by I-SMF. The N4 information for local traffic handling corresponds to N4 rules (PDR, FAR, URR, QER, etc.) related with the support of a DNAI. This is described in TS 23.501 [2] clause 5.34.6. N4 information for local traffic handling may indicate information (as the 5G AN Tunnel Info) that the SMF does not know and that the I-SMF needs to determine itself to build actual rules sent to the UPF(s). Each element of N4 information for local traffic handling provided to the I-SMF that refers to a PSA includes the DNAI it corresponds to.

If the "Indication of application relocation possibility" or "UE IP address preservation indication" attributes are included in the PCC rule, the SMF includes the corresponding Indication of no DNAI change and Indication no local PSA change respectively.

If the CN Tunnel Info at the PSA1 has changed, the SMF may also provide its new value.

The I-SMF uses N4 information for local traffic handling received from the SMF as well as 5G AN Tunnel Info received from the 5G AN via the AMF and local configuration to determine N4 rules to send to the UPF(s) it is controlling.

7.	The I-SMF updates the PSA2 via N4 providing N4 rules determined in step 6. It also provides the Branching Point or UL CL CN Tunnel Info for down-link traffic if the PSA2 and the UL CL/Branching Point are supported by different UPF(s).

8. The I-SMF updates the Branching Point or UL CL via N4 providing N4 rules determined in step 6.

NOTE 2:	If the Branching Point or UL CL and the PSA2 are co-located in a single UPF then step 7 and step 8 can be merged.

9.	The I-SMF Issues a Nsmf_PDUSession_Update Response to SMF including the N4 answers from the local UPF(s);

10.	Steps 7-8 of clause 4.3.5.4 are performed. In the case of IPv6 multi-homing PDU Session, the SMF notifies the UE of the IPv6 prefix @PSA2 and updates the UE with routing rule.

11.	If a new UPF is selected to replace I-UPF in step 3, the I-SMF uses N4 Release to remove the I-UPF of the PDU Session. The I-UPF releases resources for the PDU Session.

#### 4.23.9.2	Removal of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF

This clause describes a procedure to remove a PDU Session Anchor, and Branching Point or UL CL controlled by I-SMF.

Figure 4.23.9.2-1: Removal of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF

1.	UE has an established PDU Session with a UPF including the PDU Session Anchor 1 (controlled by SMF), and the UL-CL/BP and the PDU Session Anchor 2 (controlled by I-SMF). Events described in item 1 and 2 of clause 4.23.9.0 have taken place

At some point the I-SMF decides to remove the PDU Session Anchor 2 and UL-CL/BP function, e.g. due to UE mobility.

2.	The I-SMF may select a new UPF acting as new I-UPF, and replace the existing I-UPF which was acting as UL-CL/BP before.

If a new UPF acting as new I-UPF is selected, the I-SMF uses N4 establishment to provide the PSA1 CN Tunnel Info and (R)AN Tunnel Info to the new I-UPF. The I-SMF updates also the (R)AN with CN Tunnel info of the new I-UPF

3.	The I-SMF invokes Nsmf_PDUSession_Update Request (the list of DNAIs no more supported, Indication of Removal of traffic offload, Removal of IPv6 prefix @PSA2, DL Tunnel Info of new I-UPF if any) to SMF.

The I-SMF informs the SMF that local traffic offload is removed.

In the case of IPv6 multi-homing, the I-SMF also notifies the SMF with the removal of the IPv6 prefix @PSA2.

The SMF issues a SM Policy Association Modification (clause 4.16.5) corresponding to the IP address allocation/release PCRT (Policy Control Request Trigger). The SMF may also send a notification to the AF, as described in clause 4.3.6.3.

4.	If a new UPF that replaces existing I-UPF is selected in step 2, the SMF updates the PSA1 via N4. It provides the CN Tunnel Info of the new I-UPF for the downlink traffic. The SMF may update the packet handling rules in PSA1 as now all traffic is to be moved to PSA1.

5. 	In the case of IPv6 multi-homing, the SMF notifies the UE to stop using the IPv6 prefix corresponding to PSA2. Also the SMF sends IPv6 multi-homed routing rule along with the IPv6 prefix corresponding to PSA1 to the UE. Based on the information provided in the Router Advertisement, the UE starts using the IPv6 prefix (corresponding to PSA1) for corresponding traffic.

6.	The SMF responds I-SMF with Nsmf_PDUSession_Update Response.

7.	The SMF provides I-SMF with N4 information for the local UPF(s) with a SMF initiated Nsmf_PDUSession_Update Request; The N4 information indicates the removal of the traffic offload rules.

8.	If a new UPF that replaces existing I-UPF is selected in step 2, the I-SMF releases the old I-UPF. Otherwise the I-SMF updates the existing I-UPF with new rules in order to remove the UL-CL/BP functionality from that I-UPF.

If the PSA2 is not collocated with UL-CL/BP function, the I-SMF releases it via N4.

9.	The I-SMF answers to the SMF with a Nsmf_PDUSession_Update Response that contains N4 answers received from local UPF

#### 4.23.9.3	Change of PDU Session Anchor for IPv6 multi-homing or UL CL controlled by I-SMF

This clause describes a change of UL-CL/BP function, e.g. addition of a new PDU Session Anchor (i.e. PSA2) and release of the existing additional PDU Session Anchor (i.e. PSA0), via modifying IPv6 multi-homing or UL CL rule in the same Branching Point or UL CL under controlled by the same I-SMF.

Figure 4.23.9.3-1: Change of PDU Session Anchor for Branching Point or UL CL controlled by I-SMF

1.	The UE has an established PDU Session with a UPF including the PDU Session Anchor 1(controlled by SMF), and the PDU Session Anchor 0 and an I-UPF acting as UL CL or BP (controlled by I-SMF). Events described in item 1 and 2 of clause 4.23.9.0 have taken place

2.	At some point the I-SMF decides to establish a new PDU Session Anchore.g. due to UE mobility. The I-SMF selects a UPF and using N4 establishes the new PDU Session Anchor 2 of the PDU Session.

In the case of IPv6 multi-homing PDU Session, the I-SMF ensures allocation of  a new IPv6 prefix corresponding to PSA2.

3.	The I-SMF invokes Nsmf_PDUSession_Update Request (Indication of Change of traffic offload, (new allocated IPv6 prefix @PSA2, DNAI(s) supported by PSA2), (Removal of IPv6 prefix @PSA0, DNAI(s) supported by PSA0)) to SMF.

The I-SMF informs the SMF that a change of traffic offload may occur.

The I-SMF provides the list of DNAI(s) no more reachable and the list of DNAI now reachable to SMF. The SMF generates the corresponding N4 information based on this DNAI(s) information.

In the case of multi-homing, the new allocated IPv6 prefix @PSA2 and removal of IPv6 prefix @PSA0 are also provided to SMF.

4.	The SMF may issue a SM Policy Association Modification (clause 4.16.5) corresponding to the IP address allocation/release PCRT. The SMF may also send an “early” notification to the AF, as described in clause 4.23.6.3.

5.	The SMF generates the N4 information based on DNAI(s) information received in step 3.The SMF provides I-SMF with N4 information for the PSA and for the UL CL with a SMF initiated Nsmf_PDUSession_Update Request (set of (N4 information, involved DNAI, Indication of no DNAI change, Indication of no local PSA change)). The information includes N4 information to remove the traffic offload related to the DNAI(s) that are no more reachable and to enable the traffic offload related to the DNAI(s) that are now reachable.

6-9.	Same as step 7-9 of clause 4.23.9.1

10.	The I-SMF releases via N4 the PSA0 if PSA0 is not collocated with UL CL/BP, or updates the UL CL/BP to remove corresponding rules if it is collocated with UL CL/BP.

NEXT CHANGE (2)

#### 4.23.6.3	Reporting UP path change to the AF

Figure 4.23.6.3-1 shows procedures related Reporting UP path change to the AF.

Figure 4.23.6.3-1: Reporting UP path change to the AF

1a.	I-SMF indicates that UP path change may occur for the PDU Session via Nsmf_PDUSession_Update Request as described in clause 4.23.9; the SMF responds to the I-SMF; .

2.	If early notification has been requested by a PCC rule on behalf of AF as described in clause 4.3.6.2, then the SMF notifies the AF accordingly by invoking Nsmf_EventExposure_Notify service operation as described in clause 4.3.6.3.. In this case the SMF may wait for further instructions of the AF.

3. 	SMF initiates Nsmf_PDUSession_Update Request with N4 information to control the local PSA and ULCL / BP as described in clause 4.23.9

4.	I-SMF enforces the change of DNAI or addition, change, or removal of UPF as described in clause 4.23.9.

5	I-SMF answers back to the Nsmf_PDUSession_Update from the SMF

6.	If late notification has been requested by a PCC rule on behalf of AF as described in clause 4.3.6.2, then the SMF notifies the AF accordingly by invoking Nsmf_EventExposure_Notify service operation as described in clause 4.3.6.3

.

NEXT CHANGE (3)

END OF CHANGES