3GPP TSG-WG SA2 Meeting #136 	S2-1911258

# Reno, NV, USA, November 18 – 22, 2019	(revision of S2-191xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0048 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous V2X corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CT1 sent an LS in S2-1910874 asking clarifications on the following aspects: - Is validity timer for V2X Policy/Parameters (see clause 6.2.4) provided with the V2X policy or, e.g., locally configured? - In clause 6.3.3.1 (Layer-2 link establishment over PC5 reference point), can the indication of whether IP communication be removed as it is always implicitly provided by the inclusion or absent of the IP address configuration information in the message? |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addressed the issues raised by CT1 in clauses 6.2.4 and 6.3.3.1. In clause 6.1.1 the NOTE indicating that the Non-IP Type header will be defined by Stage 3 has been replaced by normative text with a reference to TS 24.587. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies between Stage 2 and Stage 3 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.4, 6.1.1, 6.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.185: "Service requirements for V2X services; Stage 1".

[3]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

[4]	ISO 17419:2018: "Intelligent transport systems - Cooperative systems - Globally unique identification".

[5]	IEEE Std 1609.12-2016: "IEEE Standard for Wireless Access in Vehicular Environments (WAVE) - Identifier Allocations".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[8]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[9]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[10]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[11]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[12]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[13]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[14]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[15]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); protocol specification".

[16]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[17]	3GPP TS 23.303: "Proximity-based Services (ProSe); Stage 2".

[18]	IEEE Std 1609.3-2010: "IEEE Standard for Wireless Access in Vehicular Environments (WAVE) - Networking Services".

[19]	ISO 29281-1:2013: "Intelligent Transport Systems - Communications access for land mobiles (CALM) - Non-IP networking - Part 1: Fast networking & transport layer protocol (FNTP)".

[20]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[y]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Protocol aspects; Stage 3".

# * * * * Second change * * * *

### 6.2.4	Procedure for UE triggered V2X Policy provisioning

The UE triggered Policy Provisioning procedure is initiated by the UE to request V2X Policy/Parameter from the PCF when UE determines the V2X Policy/Parameter is invalid in the following cases:

-	if the validity timer indicated in the V2X Policy/Parameter expires;

-	if there are no valid parameters, e.g., for current area, or due to abnormal situation.

Figure 6.2.4-1: UE triggered V2X Policy provisioning procedure

1.	The UE sends the UE Policy provisioning request including the UE Policy Container (V2X Policy/Parameter) to the AMF.

2.	The AMF sends the Npcf_UEPolicyControl_Update request to the PCF including the UE Policy Container received from UE.

3.	The UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [7] is triggered.

# * * * * Third change * * * *

### 6.1.1	User plane for NR PC5 reference point supporting V2X services

Legend:

-	PC5-U: The SDAP/PDCP/RLC/MAC/PHY functionality is specified in TS 38.300 [11].

-	For PDCP SDU type "Non-IP", a "Non-IP Type" header included in the SDU by upper layer to indicate the type of non-IP messages carried will be specified in stage 3 specification.

# Figure 6.1.1-1: User Plane for PC5 interface

IP and Non-IP PDCP SDU types are supported for the V2X communication over PC5.

For IP PDCP SDU type, only IPv6 is supported. The IP address allocation and configuration are as defined in clause 5.6.1.1.

The Non-IP PDCP SDU contains a Non-IP Type header, which indicates the V2X message family used by the application layer, e.g. IEEE 1609 family's WSMP [18], ISO defined FNTP [19].

NOTE:	The Non-IP Type header and allowed values are defined in TS 24.587 [y].

# * * * * Fourth change * * * *

#### 6.3.3.1	Layer-2 link establishment over PC5 reference point

To perform unicast mode of V2X communication over PC5 reference point, the UE is configured with the related information as described in clause 5.1.2.1.

Figure 6.3.3.1-1 shows the layer-2 link establishment procedure for unicast mode of V2X communication over PC5 reference point.

Figure 6.3.3.1-1: Layer-2 link establishment procedure

1.	The UE(s) determine the destination Layer-2 ID for signalling reception for PC5 unicast link establishment as specified in clause 5.6.1.4. The destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.2.1.

2.	The V2X application layer in UE-1 provides application information for PC5 unicast communication. The application information includes the service type(s) (e.g. PSID or ITS-AID) of the V2X application and the initiating UE's Application Layer ID. The target UE's Application Layer ID may be included in the application information.

The V2X application layer in UE-1 may provide V2X Application Requirements for this unicast communication. UE-1 determines the PC5 QoS parameters and PFI as specified in clause 5.4.1.4.

If UE-1 decides to reuse the existing PC5 unicast link as specified in clause 5.2.1.4, the UE triggers Layer-2 link modification procedure as specified in clause 6.3.3.4.

3.	UE-1 sends a Direct Communication Request message to initiate the unicast layer-2 link establishment procedure. The Direct Communication Request message includes:

-	Source User Info: the initiating UE's Application Layer ID (i.e. UE-1's Application Layer ID).

-	If the V2X application layer provided the target UE's Application Layer ID in step 2, the following information is included:

-	Target User Info: the target UE's Application Layer ID (i.e. UE-2's Application Layer ID).

-	V2X Service Info: the information about V2X Service(s) requesting Layer-2 link establishment (e.g. PSID(s) or ITS-AID(s)).

-	If IP communication is used:

-	IP Address Configuration: For IP communication, IP address configuration is required for this link and indicates one of the following values:

-	"IPv6 Router" if IPv6 address allocation mechanism is supported by the initiating UE, i.e., acting as an IPv6 Router; or

-	"IPv6 address allocation not supported" if IPv6 address allocation mechanism is not supported by the initiating UE.

-	Link Local IPv6 Address: a link-local IPv6 address formed locally based on RFC 4862 [21] if UE-1 does not support the IPv6 IP address allocation mechanism, i.e. the IP Address Configuration indicates "IPv6 address allocation not supported".

-	QoS Info: the information about PC5 QoS Flow(s). For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

The source Layer-2 ID and destination Layer-2 ID used to send the Direct Communication Request message are determined as specified in clauses 5.6.1.1 and 5.6.1.4.

UE-1 sends the Direct Communication Request message via PC5 broadcast using the source Layer-2 ID and the destination Layer-2 ID.

4.	A Direct Communication Accept message is sent to UE-1 as below:

4a.	(UE oriented Layer-2 link establishment) If the Target User Info is included in the Direct Communication Request message, the target UE, i.e. UE-2 responds with a Direct Communication Accept message.

4b.	(V2X Service oriented Layer-2 link establishment) If the Target User Info is not included in the Direct Communication Request message, the UEs that are interested in using the announced V2X Service(s), so decide to establish Layer-2 link with UE-1 respond to the request by sending a Direct Communication Accept message (UE-2 and UE-4 in Figure 6.3.3.1-1).

The Direct Communication Accept message includes:

-	Source User Info: Application Layer ID of the UE sending the Direct Communication Accept message.

-	QoS Info: the information about PC5 QoS Flow(s). For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters requested by UE-1 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

-	IP Address Configuration: For IP communication, IP address configuration is required for this link and indicates one of the following values:

-	"IPv6 Router" if IPv6 address allocation mechanism is supported by the target UE, i.e., acting as an IPv6 Router; or

-	"IPv6 address allocation not supported" if IPv6 address allocation mechanism is not supported by the target UE.

-	Link Local IPv6 Address: a link-local IPv6 address formed locally based on RFC 4862 [21] if the target UE does not support the IPv6 IP address allocation mechanism, i.e. the IP Address Configuration indicates "IPv6 address allocation not supported", and UE-1 included a link-local IPv6 address in the Direct Communication Request message. The target UE shall include a non-conflicting link-local IPv6 address.

If both UEs (i.e. the initiating UE and the target UE) selected to use link-local IPv6 address, they shall disable the duplicate address detection defined in RFC 4862 [21].

NOTE 1:	When either the initiating UE or the target UE indicates the support of IPv6 router, corresponding address configuration procedure would be carried out after the establishment of the layer 2 link, and the link-local IPv6 addresses are ignored.

The source Layer-2 ID used to send the Direct Communication Accept message is determined as specified in clauses 5.6.1.1 and 5.6.1.4. The destination Layer-2 ID is set to the source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the Direct Communication Accept message from peer UE, UE-1 obtains the peer UE's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

The V2X layer of the UE that established PC5 unicast link passes the PC5 Link Identifier assigned for the unicast link and PC5 unicast link related information down to the AS layer. The PC5 unicast link related information includes Layer-2 ID information (i.e. source Layer-2 ID and destination Layer-2 ID). This enables the AS layer to maintain the PC5 Link Identifier together with the PC5 unicast link related information.

Editor's Note: Steps for mutual authentication and security association establishment will be determined based on feedback from SA WG3.

5.	V2X service data is transmitted over the established unicast link as below:

The PC5 Link Identifier and PFI are provided to the AS layer, together with the V2X service data.

UE-1 sends the V2X service data using the source Layer-2 ID (i.e. UE-1's Layer-2 ID for this unicast link) and the destination Layer-2 ID (i.e. the peer UE's Layer-2 ID for this unicast link).

NOTE 2:	PC5 unicast link is bi-directional, therefore the peer UE of UE-1 can send the V2X service data to UE-1 over the unicast link with UE-1.

Editor's Note: The parameters included in the Direct Communication Request/Accept messages can be updated depending on RAN WGs' decision on how the Direct Communication Request/Accept messages are sent by the AS layer (e.g. by using PC5-RRC signalling).

Editor's Note: Additional parameters included in the Direct Communication Request/Accept messages (e.g. security related) are FFS.

Editor's Note: Whether the unicast communication requires security protection at link layer will be determined based on feedback from SA WG3.

# * * * * End of changes * * * *