SA WG2 Meeting #S2-136	S2-1911290

18 - 22 November, 2019, Reno, Nevada, USA	(revision of S2-1910151)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1764 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Notification receiver information in a subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A subscription to notification only includes the so called notification endpoint (e.g. URI). In case of failure of this notification endpoint, the subscription to notification does not include the information to allow re-select a new receiver for the expected notification. More information should be included in the subscription request to achieve reliability for the notifications. Subscription to notifications with indirect communication requires to insert the SCP as a notification receiver in the flow. Binding can apply to notification requests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Subscription on behalf of another NF use case is clarified. Subscription to notification procedures with indirect communication are updated. Binding for subscription requests is updated to consider re-selection. Binding is applied to notification requests. Subscription request operations are updated to include attributes for reselection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of failure of the notification endpoint, notifications requests will fail, withouth the option to re-select an alternative destination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.1, New 4.17.Y, 4.17.12.4, new 4.17.12.X, 5.2.2.2.8, 5.2.2.2.14, 5.2.2.3.2, 5.2.3.2.1, 5.2.3.3.4, 5.2.3.5.2, 5.2.5.3.6, 5.2.5.7.2, 5.2.6.2.2, 5.2.6.3.3, 5.2.6.12.2, 5.2.6.16.2, 5.2.6A.2.2, 5.2.8.3.3, 5.2.12.2.6, 5.2.16.3.4, 5.2.17.2.2, 5.2.18.3.3, 5.2.19.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 1765 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

##### 4.15.3.2.1	AMF service operations information flow

The procedure is used by the NF to subscribe to notifications and to explicitly cancel a previous subscription. Cancelling is done by sending Namf_EventExposure_UnSubscribe request identifying Subscription Correlation ID. The notification steps 3 and 4 are not applicable in cancellation case. If AMF receives Monitoring event subscription for roaming UEs and I-NEF is deployed as defined in clause 6.2.5a of TS 23.501 [2], clause 4.15.3.2.3a applies after step 2.

Figure 4.15.3.2.1-1: Namf_EventExposure_Subscribe, Unsubscribe and Notify operations

1.	A NEF sends a request to subscribe to a (set of) Event ID(s) in AMF in Namf_EventExposure_Subscribe request. The NEF could be the same NF subscribing to receive the event notification reports (i.e. Event Receiving NF) or it could be a different NF.

If the NEF subscribes itself as the Event Receiving NF for one or several Event ID(s), it provides its Notification Target Address (+ Notification Correlation Id) for each related event.

If the NEF subscribes on behalf of another NF (i.e. Event Receiving NF) for one or several Event ID(s), it provides the Notification Target Address (+ Notification Correlation Id) of the Event Receiving NF. Additionallly, the NEF provides its own Notification Target Address (+ Notification Correlation Id) to assure the NEF can receive the notification of subscription change related event (e.g. Subscription Correlation ID Change).

Event Reporting information defines the type of reporting requested. If the reporting event subscription is authorized by the AMF, the AMF records the association of the event trigger and the requester identity.

2.	AMF acknowledges the execution of Namf_EventExposure_Subscribe.

3.	[Conditional - depending on the Event] The AMF detects the monitored event occurs and sends the event report by means of Namf_EventExposure_Notify message, to the notification endpoint of the Event Receiving NF.

4.	[Conditional- depending on the Event] The AMF detects the subscription change related event occurs, e.g. Subscription Correlation ID change due to AMF reallocation, it sends the event report by means of Namf_EventExposure_Notify message to the NEF.

*** NEXT CHANGE ***

### 4.17.Y	Subscription to notifications with indirect communication

The subscription to notification procedure with indirect communication is described in figure 4.17.Y-1 below.

Figure 4.17.Y-1: Subscription to notification using Indirect Communication

- The NF service consumer sends a subscription request via SCP, indirect communication. The request includes the expected notification endpoint (i.e. Notification Target Address) and a Notification Correlation ID, and additionaly, either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id may be included to identify alternative default notification endpoints. The expected notification receiver may be either NF-A or NF-A may subscribe on behalf of another NF (see clause 5.21.3.X 3GPP TS 23.501 [2]).

- The SCP defines itself as the notification endpoint.

- The SCP forwards the subscription request with the new notification receiver information, including the original notification receiver information.

- The NF service producer sends the subscription response

- The SCP forwards the subscription response to the NF service consumer.

The notification procedure with indirect communication is described in figure 4.17.Y-2 below.

Figure 4.17.Y-2:  Notification using Indirect Communication

- The NF producer sends the notification request via SCP, indirect communication. The request includes the information of the original notification receiver as provided during the subscription to notification procedure.

- The SCP forwards the notification request to the indicated original Notification Target Address. Alternativelly, if the Notification Target Address is not accessible, the information received in step 1 is used to discover from the NRF any other available default notification endpoint.  This alternative Notification Target Address is used to reach the notification receiver. In the Notification request it is included the Notification Correlation Id.

- NF-X sends the notification response.

- The SCP forwards the response to the NF service consumer.

*** NEXT CHANGE ***

#### 4.17.12.4	Binding for subscription requests

Binding may be created as part of the implicit subscription request. In this case, the response to the service request that creates the implicit subscription may include a binding indication as in 4.17.12.2. For subsequent operations on the created implicit subscription, bindings may be used. The description in 4.17.12.1 applies for Figure 4.17.12.4.-1.

If the binding needs to be updated, the NF service consumer may initiate a new request to the NF service producer with an updated Notification Correlation ID and binding indication.

Figure 4.17.12.4-1: Binding for implicit subscription requests

#### 4.17.12.X	Binding for notification requests

Binding may be included as part of a subscription request, in this case, this may be applied for the notification re-selection.

Figure 4.17.12.4-1: Binding for notification requests

*** NEXT CHANGE ***

##### 5.2.2.2.8	Namf_Communication_N2InfoSubscribe service operation

Service operation name: Namf_Communication_N2InfoSubscribe.

Description: An NF invokes this service operation to subscribe for the delivery of information contained in a specific N2 message type.

Input, Required: CN NF ID, N2 information type to be subscribed, Notification Target Address (+ Notification Correlation ID).

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription). For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

.

Output, Required: None.

Output, Optional: None.

*** NEXT CHANGE ***

##### 5.2.2.2.14	Namf_Communication_AMFStatusChangeSubscribe service operation

Service operation name: Namf_Communication_AMFStatusChangeSubscribe

Description: This service operation is used by an NF to subscribe for AMF Status Change notification.

Input, Required: GUAMI(s), Notification Target Address (+ Notification Correlation ID))s.

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription). For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

.

Output, Required: None.

Output, Optional: None.

See clause 5.21.2.2, TS 23.501 [2] for the example usage of this service operation. The GUAMI(s) is used to identify the AMF.

*** NEXT CHANGE ***

##### 5.2.2.3.2	Namf_EventExposure_Subscribe service operation

Service operation name: Namf_EventExposure_Subscribe.

Description: The consumer NF uses this service operation to subscribe to or modify event reporting for one UE, a group of UE(s) or any UE.

Input, Required: NF ID, Target of Event Reporting: UE(s) ID (SUPI or Internal Group Identifier or indication that any UE is targeted), ((set of) Event ID(s) defined in clause 5.2.2.3.1, Notification Target Address (+ Notification Correlation ID))s, Event Reporting Information defined in Table 4.15.1-1.

Input, Optional:  (Event Filter (s) associated with each Event ID; Event Filter (s) are defined in clause 5.2.2.3.1, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time. For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1).

The NF consumer subscribes to the event notification by invoking Namf_EventExposure to the AMF. The AMF allocates an Subscription Correlation ID for the subscription and responds to the consumer NF with the Subscription Correlation ID. UE ID identifies the UE, SUPI and/or GPSI. Event ID (see clause 4.15.1) identifies the events that the NF consumer is interested in. The Subscription Correlation ID is unique within the AMF Set.

The ((set of) Event ID(s), Notification Target Address (+ Notification Correlation ID)) helps the Event Receiving NF to co-relate a notification against a corresponding event subscription for the indicated Event ID.

In the case that the NF consumer subscribes to the AMF on behalf of other NF, the NF consumer include the Notification Target Address(+Notification Correlation ID) of other NF for the Event ID which is to be notified to other NF directly, and the Notification Target Address(+Notification Correlation ID) of itself for the Subscription Correlation ID change event. Each Notification Target Address(+ Notification Correlation ID) is associated with the related (set of) Event ID(s).When the Subscription Correlation ID change due to the AMF reallocation, the notification is sent to NF consumer which triggers this subscription.

Event filter may include "AN type(s)" as part of the list of parameter values to match, and it indicates to subscribe the event per Access Type.

Event receiving NF ID identifies the NF that shall receive the event reporting.

When the consumer NF needs to modify an existing subscription previously created by itself in the AMF, it invokes Namf_EventExposure_Subscribe service operation which contains the Subscription Correlation ID and the new Event Filters with Event ID to the AMF.

*** NEXT CHANGE ***

##### 5.2.3.2.1	Nudm_UECM_Registration service operation

Service operation name: Nudm_UECM_Registration

Description: Register UE's serving NF (if NF Type is AMF, SMSF) or Session's serving NF (if NF Type is SMF) on the UDM. This operation implies the following:

-	The authorization, if applicable, to register the NF service consumer in UDM for the UE (e.g. based on UE roaming/RAT restrictions applicable when NF type is AMF). If this is successful, the NF service consumer is set as a serving NF for the corresponding UE/Session context.

-	When the consumer is AMF or SMF, it is implicitly subscribed to be notified when it is deregistered in UDM. This notification is done by means of Nudm_UECM_DeregistrationNotification operation.

-	When the consumer is AMF or SMF, it may optionally use this operation to subscribe to be notified of the need for P-CSCF Restoration. This notification is done by means of Nudm_UECM_PCscfRestoration operation. For more information regarding P-CSCF restoration procedures see TS 23.380 [38].

Inputs, Required: NF ID, SUPI, PEI, NF Type, Access Type (if NF Type is AMF, SMSF), PDU Session ID (if NF Type is SMF). If NF Type is SMF: DNN or Indication of Emergency Services, PGW-C+SMF FQDN for S5/S8 if the PDU Session supports EPS interworking. If NF type is AMF and Access Type is 3GPP access: Registration type. If NF type is SMSF: SMSF MAP address and/or Diameter address. Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: P-CSCF Restoration notification information, GUAMI, backup AMF(s) (if NF Type is AMF), "Homogeneous Support of IMS Voice over PS Sessions" indication (if NF Type is AMF), UE SRVCC capability (if NF Type is AMF), indication that access is from ePDG (shall be sent if NF Type is SMF and PDU Session is setup via S2b). Backup AMF(s) sent only once by the AMF to the UDM in its first interaction with the UDM. For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs, Required: Result indication.

Outputs, Optional: None.

See step 14a of clause 4.2.2.2.2 for an example usage of this service operation.

*** NEXT CHANGE ***

##### 5.2.3.3.4	Nudm_SDM_Subscribe service operation

Service operation name: Nudm_SDM_Subscribe

Description: The NF consumer subscribes for updates to UE's Subscriber Data indicated by the 'subscription data type' input. The UDM shall check the requested consumer is authorized to subscribe to requested updates.

Inputs, Required: Subscription data type(s), Key for each Subscription data type(s). Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Data Sub Key(s). For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs, Required: None.

Outputs, Optional: None.

*** NEXT CHANGE ***

##### 5.2.3.5.2	Nudm_EventExposure_Subscribe service operation

Service operation name: Nudm_EventExposure_Subscribe

Description: The NF consumer subscribes to receive an event, or if the subscription is already defined in UDM, then the subscription is updated.

NF Consumers: NEF.

Inputs (required): Target of Event Reporting: UE(s) ID (SUPI or GPSI, Internal Group Identifier or External Group Identifier, or indication that any UE is targeted), Event filter containing the Event Id(s) (see clause 4.15.3.1) and Event Reporting Information defined in Table 4.15.1-1. Notification Target Address (+ Notification Correlation ID).

Inputs (optional): Expiry time. For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs (required): Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs (optional): First corresponding event report is included, if corresponding information is available (see clause 4.15.1), Number of UE if the External Group Identifier and Maximum Number of Reports are included in the inputs.

Number of UEs indicates the number of UEs within the group identified by the External Group Identifier. The NEF uses this value to determine whether the monitoring event has been reported for all group member UEs.

*** NEXT CHANGE ***

##### 5.2.5.3.6	Npcf_PolicyAuthorization_Subscribe service operation

Service operation name: Npcf_PolicyAuthorization_Subscribe

Description: provided by the PCF for NF consumers to explicitly subscribe the notification of events.

Inputs, Required:  (Set of) Event ID(s) as specified in Npcf_PolicyAuthorization_Notify service operation,  target of PCF event reporting (defined below), NF ID, Event Reporting Information defined in Table 4.15.1-1 (only the Event Reporting mode and the inmediate reporting flag when applicable), Notification Target Address (+ Notification Correlation ID).

The target of PCF event reporting  the subscription for an individual AF session: An UE IP address (IPv4 address or IPv6 prefix) optionally together with a (DNN, S-NSSAI) or with a UE ID (SUPI or GPSI).

Inputs, Optional: Event Filter, Subscription Correlation ID (in case of modification of the event subscription). For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

*** NEXT CHANGE ***

##### 5.2.5.7.2	Npcf_EventExposure_Subscribe service operation

Service operation name: Npcf_EventExposure_Subscribe.

Description: The consumer NF uses this service operation to subscribe to or modify event reporting for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI).

NF Consumers: NEF.

Inputs (required): NF ID, Target of Event Reporting (Internal Group Identifier or indication that any UE accessing a combination of (DNN, S-NSSAI)is targeted, (set of) Event ID(s) defined in clause 5.2.5.7.1, Notification Target Address (+ Notification Correlation ID) and Event Reporting Information defined in Table 4.15.1-1.

Inputs (optional): Event Filter (s) associated with each Event ID. For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs (required): Operation execution result indication. When the subscription is accepted: Subscription Correlation ID.

Outputs (optional): First corresponding event report is included, if corresponding information is available (see clause 4.15.1).

The NF consumer subscribes to the event notification by invoking Npcf_EventExposure to the PCF. The PCF allocates a Subscription Correlation ID for the subscription and responds to the consumer NF with the Subscription Correlation ID. Event receiving NF ID identifies the NF that shall receive the event reporting.

*** NEXT CHANGE ***

##### 5.2.6.2.2	Nnef_EventExposure_Subscribe operation

Service operation name: Nnef_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated.

Inputs (required): (Set of) Event ID(s) as specified in clause 4.15.3.1 or Npcf_PolicyAuthorization_Notify and Naf_EventExposure_Subscribe service operation, Target of Event Reporting (GPSI or External Group Identifier), Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID).

Inputs (optional): Event Filter, Subscription Correlation ID (in case of modification of the event subscription), Expiry time. For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs (required): When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs (optional): First corresponding event report is included, if available (see clause 4.15.1).

*** NEXT CHANGE ***

##### 5.2.6.3.3	Nnef_PFDManagement_Subscribe service operation

Service operation name: Nnef_PFDManagement_Subscribe

Description: provided by the NEF (PFDF) for NF consumers to explicitly subscribe the notification of changes of PFDs for Application Identifier.

Inputs (required): Application Identifier(s). Notification Target Address (+ Notification Correlation ID).

Inputs (optional): Subscription Correlation ID (in case of modification of the event subscription). For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id..

Outputs (required): None.

*** NEXT CHANGE ***

##### 5.2.6.12.2	Nnef_APISupportCapability_Subscribe service operation

Service operation name: Nnef_APISupportCapability_Subscribe

Description: The AF subscribes to receive notification about the availability or expected level of support of a service API for a UE or a group of UEs.

Inputs (required): UE ID or External Group ID, Report Type (One-time report or Continuous report). Notification Target Address (+ Notification Correlation ID).

Inputs (optional): Duration of Reporting. Subscription Correlation ID (in case of modification of the event subscription). For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs (required): Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, API indication.

*** NEXT CHANGE ***

##### 5.2.6.16.2	Nnef_AnalyticsExposure_Subscribe operation

Service operation name: Nnef_AnalyticsExposure_Subscribe

Description: The NF consumer subscribes or modifies an existing subscription on analytics information.

Inputs (required): (Set of) Analytic ID(s), Analytic Filter Information, Target of Analytic Reporting (UEs, External Group Identifier, any UEs), Analytic Reporting Information, Notification Target Address (+ Notification Correlation ID). These input parameters are detailed in TS 23.288 [50].

Inputs (optional): Subscription Correlation ID (in case of modification of the analytic subscription), Expiry time. For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs (required): When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs (optional): First corresponding analytic report is included, if available.

*** NEXT CHANGE ***

##### 5.2.6A.2.2	Ninef_EventExposure_Subscribe operation

Service operation name: Ninef_EventExposure_Subscribe

Description: the consumer subscribes to an event with monitoring configuration in I-NEF.

Inputs (required): NF ID, (Set of) Event ID(s) as specified in clause 4.15.3.1, Target of Event Reporting (GPSI or External Group Identifier), Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID), Subscription Correlation ID.

Inputs (optional): Event Filter, Expiry time, Chargeable Party Identifier. For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs (required): Expiry time (required if the subscription can be expired based on the operator's policy), cause.

Outputs (optional): None.

*** NEXT CHANGE ***

##### 5.2.8.3.3	Nsmf_EventExposure_Subscribe service operation

Service operation name: Nsmf_EventExposure_Subscribe.

Description: This service operation is used by an NF to subscribe or modify a subscription for event notifications on a specified PDU Session or for all PDU Sessions of one UE, group of UE(s) or any UE.

Input, Required: NF ID, Target of Event Reporting as defined in clause 5.2.8.3.1, (set of) Event ID(s) defined in clause 5.2.8.3.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Input, Optional: Event Filter(s) associated with each Event ID; Event Filter(s) are defined in clause 5.2.8.3.1, Subscription Correlation ID (in case of modification of the event subscription), Expiry time. For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1).

Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by SMF with this subscription.

*** NEXT CHANGE ***

##### 5.2.12.2.6	Nudr_DM_Subscribe service operation

Service operation name: Nudr_DM_Subscribe.

Description: NF service consumer performs the subscription to notification to data modified in the UDR. The events can be changes on existing data, addition of data.

Inputs, Required: Data Set Identifier as defined in clause 5.2.12.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Inputs, Optional: Target of Event Reporting as defined in clause 5.2.12.2.1, Data Subset Identifier(s) as defined in clause 5.2.12.2.1, Subscription Correlation ID (in the case of modification of the event subscription). For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

*** NEXT CHANGE ***

##### 5.2.16.3.4	Nnssf_NSSAIAvailability_Subscribe service operation

Service operation name: Nnssf_NSSAIAvailability_Subscribe

Description: This service operation enables a NF Service Consumer (e.g. AMF) to subscribe to a notification of any changes in status of the NSSAI availability information (e.g. S-NSSAIs available per TA and the restricted S-NSSAI(s) per PLMN in that TA in the serving PLMN of the UE) upon this is updated by another AMF.

Inputs, Required: Notification Target Address (+ Notification Correlation ID), list of TAIs supported by the NF service consumer, event to be subscribed.

Inputs, Optional: Expiry time. For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs, Required: SubscriptionID,

Outputs, Conditional Required: Expiry time (if present in the request, may be included in the response based on operator's policy and taking into account the expiry time present in the request (i.e. should be less than or equal to that value); if not present in the request, may be included in the response based on operator's policy. Whatever the case, if not included in the response, this means that the subscription is valid without an expiry time).

Outputs, Optional: A list of TAIs and, for each TAI, the S-NSSAIs supported by the AMF and 5G-AN, and authorized by the NSSF for the TAI, and a list of S-NSSAIs restricted per PLMN for the TAI.

*** NEXT CHANGE ***

##### 5.2.17.2.2	Nchf_SpendingLimitControl Subscribe service operation

Service operation name: Nchf_SpendingLimitControl_Subscribe.

Description: Subscribe to notification of changes in the status of the policy counters available at the CHF and retrieval of the status of the policy counters for which subscription is accepted by CHF.

Inputs, Required: SUPI (for the Initial Spending Limit request), SubscriptionCorrelationId (for the Intermediate Spending Limit report), Event Id "policy counter status change", Event Filter Information "List of policy counter identifier (s)".

Inputs, Optional: Notification Correlation Target (required for the Initial Spending Limit request), Event Filter Information "List of policy counter identifier (s)", Event Reporting Information (continuous reporting). For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs, Required: Status of the requested subscribed policy counters to the subscriber in the Event Information.

Outputs, Optional: Pending policy counter statuses and their activation times, for all policy counter(s) available for this subscriber. If list of policy counter identifier(s) was provided, the CHF returns only the pending policy counter statuses and their activation times, per required policy counter identifier in the Event Information, SubscriptionCorrelationId.

*** NEXT CHANGE ***

##### 5.2.18.3.3	Nucmf_UECapabilityManagement_Subscribe service operation

Service operation name: Nucmf_UECapabilityManagement_Subscribe

Description: The NF consumer subscribes for updates to UCMF dictionary entries. The UCMF shall check the requested consumer is authorized to subscribe to requested updates.

Inputs, Required: None.

Inputs, Optional: For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

Outputs, Required: None.

Outputs, Optional: None.

*** NEXT CHANGE ***

##### 5.2.19.2.2	Naf_EventExposure_Subscribe service operation

Service operation name: Naf_EventExposure_Subscribe

Description: The consumer NF subscribes the event to collect AF data for UE(s), group of UEs, or any UE, or updates the subscription which is already defined in AF.

Input, Required: Target of Event Reporting: external UE ID(s), External Group Identifier, or indication that any UE is targeted, (set of) Event ID(s), Notification Target Address (+ Notification Correlation ID) and Event Reporting Information as defined in Table 4.15.1-1, Expiry time.

Input, Optional: NF ID, Event Filter(s) associated with each Event ID, Application ID, Subscription Correlation ID (in case of modification of the existing subscription). For notification reselection: notification receiver information, i.e. either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id.

NOTE:	In the case of untrusted AF, NEF ID is used as NF ID.

Output, Required: Operation execution result indication. When the subscription is accepted: Subscription Correlation ID.

Output, Optional: First corresponding event report is included, if corresponding information is available (see clause 4.15.1).

*** END CHANGES ***