SA WG2 Meeting #S2-136	 S2-1911302

18 - 22 October 2019, Reno NV, USA	(revision of S2-19xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1899 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Propose updated to Application Data in UDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | xBDT |  |  |  |  |  | Date: |  |  | 08-11-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Application Data includes Background Data Transfer as one of the Data Subsets, that includes both the identifier of the ASP that requests to apply a BDT policy and the BDT reference id to identify the BDT policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the content of the Background Data Transfer Data Subset |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 0321 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 5.2.12.2	Nudr_DataManagement (DM) service

##### 5.2.12.2.1	General

The operations defined for Nudr_DM service use following set of parameters defined in this clause:

-	Data Set Identifier:. uniquely identifies the requested set of data within the UDR (see clause 4.2.5).

-	Data Subset Identifier: it uniquely identifies the data subset within each Data Set Identifier. As specified in the procedures in clause 4. e.g. subscription data can consist of subsets particularised for specific procedures like mobility, session, etc.

-	Data Keys defined in Table 5.2.12.2.1-1

For Nudr_DM_Subscribe and Nudr_DM_Notify operations:

-	The Target of Event Reporting is made up of a Data Key and possibly a Data Sub Key both defined in Table 5.2.12.2.1-1. When a Data Sub Key is defined in the table but not present in the Nudr_DM_Subscribe this means that all values of the Data Sub Key are targeted.

-	The Data Set Identifier plus (if present) the (set of) Data Subset Identifier(s) corresponds to a (set of) Event ID(s) as defined in clause 4.15.1

An NF Service Consumer may include an indicator when it invokes Nudr_DM Query/Create/Update service operation to subscribe the changes of the data, to avoid a separate Nudr_DM_Subscribe service operation.

Depending on the use case, it is possible to use a Data Key and/or one or multiple Data sub keys to further identify the corresponding data, as defined in Table 5.2.12.2.1-1 below.

Table 5.2.12.2.1-1: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
|  | Access and Mobility Subscription data | SUPI | - |
|  | SMF Selection Subscription data | SUPI | - |
|  | UE context in SMF data | SUPI | PDU Session ID or DNN |
| Subscription Data (see clause 5.2.3.3.1) | SMS Management Subscription data | SUPI | - |
|  | SMS Subscription data | SUPI |  |
|  | Session Management Subscription data | SUPI | S-NSSAI |
|  |  |  | DNN |
|  | Slice Selection Subscription data | SUPI | - |
|  | Group Data | Internal Group Identifier or External Group Identifier | - |
| Application data | Packet Flow Descriptions (PFDs) | Application ID | - |
|  | AF traffic influence request information | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 in TS 23.501 [2]). | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
|  | Background Data Transfer | Internal Group Identifier or SUPI (NOTE 3) |  |
| Policy Data | UE context policy control data (See clause 6.2.1.3 in TS 23.503 [20]) | SUPI |  |
|  | PDU Session policy control data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 in TS 23.503 [20]) |  | DNN |
|  | Policy Set Entry data (See clause 6.2.1.3 in TS 23.503 [20]) | SUPI (for the UDR in HPLMN) |  |
|  |  | PLMN ID (for the UDR in VPLMN) |  |
|  | Remaining allowed Usage data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 in TS 23.503 [20]) |  | DNN |
|  | Sponsored data connectivity profiles (See clause 6.2.1.6 in TS 23.503 [20]) | Sponsor Identity |  |
|  | Background Data Transfer data (See clause 6.2.1.6 in TS 23.503 [20]) | Background Data Transfer Reference ID. (NOTE 2) |  |
|  |  | None. (NOTE 1) |  |
| Exposure Data | Access and Mobility Information | SUPI or GPSI | PDU Session ID or |
| (see clause 5.2.12.1) | Session Management information | SUPI or GPSI | UE IP address or DNN |
| NOTE 1: Retrieval of the stored Background Data Transfer References for all ASP identifiers in the UDR requires Data Subset but no Data Key or Data Subkey(s). NOTE 2: Update of a Background Data Transfer Reference in the UDR requires a Data key to refer to a Background Data Transfer Reference as input data. NOTE 3: The Background Data Transfer includes the Background Data Reference ID and the ASP id that requests to apply the the Background Data Reference ID to the UE(s). |  |  |  |

The content of the UDR storage for (Data Set Id= Application Data , Data Subset Id = AF TrafficInfluence request information) is specified in TS 23.501 [2], clause 5.6.7, Table 5.6.7-1. This information is written by the NEF and read by the PCF(s). PCF(s) may also subscribe to changes onto this information.

End changes