SA WG2 Meeting #136	S2-1911414

18 - 22 November, 2019, Reno, Nevada, USA	(revision of S2-19xxxxx)

Source:	Intel

Title:	Prose Direct Discovery KI update

Document for:	Agreement

Agenda Item:	8.6

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This paper proposes to update the text for KI direct discovery to cover Relay UE discovery aspects.

# 1.	Discussion

The procedures for discovering normal UE, UE-to-UE Relay UE and UE-to-Network Relay UE should be the same, the only difference should be IEs in the discovery messages, in order to avoid competing solutions for discovering different kind of UEs, it’s proposed to consolidate all discovery aspects under one single key issue.

* * * * Start of the 1st Change * * * *

## 5.1	Key Issue #1: ProSe Direct discovery

### 5.1.1	General description

ProSe direct discovery is used for a UE to discover other UE(s) (i.e. normal UE, UE-to-UE Relay UE, UE-to-Network Relay UE) in proximity over PC5 interface. The UE can discover other UE(s) with interested or associated application(s) and/or associated group(s) using the ProSe direct discovery procedure.

Following issues need to be addressed in this key issue:

- Discover (or discovered by) other UE(s) in the associated group(s) over PC5 interface, the group information can come from application layer.

- Discover (or discovered by) other UE(s) also supporting interested or associated application(s) over PC5 interface.

- Discover other UE(s) supporting UE-to-UE Relay or UE-to-Network Relay functionalities.

- Discovered by other UE(s) requesting PC5 communication over a UE-to-UE Relay or UE-to Network Relay.

- If the UE is out of coverage, discover (or discovered by) other UE(s) in coverage or out of coverage.

- Network is able to control the ProSe direct discovery procedure when the UE is in coverage.

NOTE:	The ProSe direct discovery procedure can be used to perform the ProSe direct communication, e.g. one-to-many direct communication, one-to-one direct communication.

* * * * End of Changes * * * *