SA WG2 Meeting #136	S2-1911901

# 18 - 22 November, 2019, Reno, USA

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1839 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provisioning of Manufacturer-assigned UE Radio Capability ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 02/10/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Signaling flow to provision Manufacturer-assigned UE Radio Capability ID(s) is currently missing from the specification. Nucmf_UECapabilityManagement_Notify service operation requires an acknowledgement to indicate the AMF has successfully updated its local database with the provisioned Manufacturer-Assigned UE Radio Capability ID(s) and corresponding UE capability information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Signaling flow to provision Manufacturer-assigned UE Radio Capability ID(s) added. Add a mandatory acknowledgement output to Nucmf_UECapabilityManagement_Notify service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may start reqistering with a new UE Manufacturer-assigned UE Radio Capability ID that is not successfully provisioned in 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.x (new), 5.2.18.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START of CHANGE

#### 4.15.6.x	Provisioning of Manufacturer-assigned UE Radio Capability ID

Figure 4.15.6.x-1 describes the procedure to provision Manufacturer-assigned UE Radio Capability IDs to 5GC by an AF prior to provisioning new Manufacturer-assigned UE radio capability ID(s) to the UE.

UE

NG-RAN

AMF

UCMF

AF

1. Nucmf_Provisioning_Create_request

2. New Dictionary created

3. Nucmf_UECapabilityManagement_Notify

4. Nucmf_UECapabilityManagement_Notify_ACK

5. Nucmf_Provisioning_Create_response

7. UE performs Registration with new Manufacturer-assigned ID as in Figure 4.2.2.2.2-1

8. Nucmf_Provisioning_Delete_request

9. Nucmf_UECapabilityManagement_Notify

11 .Nucmf_Provisioning_Delete_response

6. New Manufacturer-assigned UE Radio Capability ID(s) provisioned to the UE

10. Nucmf_UECapabilityManagement_Notify_ACK

Figure 4.15.6.x-1: Provisioning of Manufacturer-assigned UE Radio Capability ID

The procedure requires the AMF to subscribe to UCMF to receive any updates related to creation or deletion of Manufacturer-assigned UE radio capability ID(s) in UCMF dictionary.

This procedure is initiated by AF when it receives new UE capability information with new Manufacturer-assigned UE Radio Capability ID(s) from an external party.

1.	The AF sends the new UE capability information along with the corresponding Manufacturer-assigned UE Radio Capability ID(s) to the UCMF using the service Nucmf_Provisioning_Create_request.

2.	The UCMF creates a new dictionary entry for the received information. The dictionary entry contains a mapping between the new UE radio capability information and the new Manufacturer-assigned UE Radio Capability ID(s).

3.	The UCMF notifies the new Manufacturer-assigned UE Radio Capability ID using Nucmf_UECapabilityManagement_Notify to the AMF. The AMF is subscribed to UCMF to be notified for any addition or deletion of an entry in the UCMF database. The AMF updates its local database with the new Manufacturer-Assigned UE Radio Capability ID(s) and corresponding UE capability information.

4.	 The AMF sends and acknowledgement in Nucmf_UECapabilityManagement_Notify_ACK to the UCMF.

NOTE:	In case an acknowldgement is not received, the UCMF re-initates the procedure from Step 3.

5.	On receiving an acknowledgement from AMF, the UCMF informs the AF that the new Manufacturer-assigned UE Radio Capability ID has been updated in Nucmf_Provisioning_Create_response.

6.	The new UE Manufacturer-assigned UE Radio Capability ID(s) are provisioned to the UE via implementation-specific means.

7.	The UE performs the registration procedure with new Manufacturer-assigned UE Radio Capability ID(s) as described in clause 4.2.2.2.

8. 	The AF initiates Nucmf_Provisioning_Delete_request procedure to delete the old Manufacturer-assigned UE Radio Capability ID in UCMF.

9.	The UCMF initiates Nucmf_UECapabilityManagement_Notify procedure to notify the subscribed AMF about the deletion of the old Manufacturer-assigned UE Radio Capability ID.

10. The AMF sends and acknowledgement in Nucmf_UECapabilityManagement_Notify_ACK to the UCMF.

11.	The UCMF responds to the AF in Nucmf_Provisioning_Delete_response indicating that old Manufacturer-assigned UE Radio Capability ID is successfully deleted from the UCMF dictionary

* * * Next Changes * * *

##### 5.2.18.3.5	Nucmf_UECapabilityManagement_Notify service operation

Service Operation name: Nucmf_UECapabilityManagement_Notify

Description: Producer NF provides notifications about creation or deletion of UCMF dictionary entries to subscribed consumer NF.

Inputs, Required: Notification Type ("creation", "deletion").

Inputs, Optional: If Notification Type is set to "creation": One or more UCMF dictionary entries, each UCMF dictionary entry consisting of a UE Radio Capability ID and the corresponding UE radio access capability. If Notification Type is set to "deletion": One or more UE Radio Capability IDs.

Outputs, Required: ACK.

Outputs, Optional: None.

End of CHANGE