SA WG2 Meeting #136	S2-1912294

# 18 – 22 Nov 2019, Reno NV, USA				(revision of S2-1912290)

# Title:	Reply LS on SMSF change

# Release:	Rel-15

# Work Item:	5GS_Ph1

Source:	SA2

To:	CT4

Cc:

Contact Person:

Name:	Hannu Hietalahti

Tel. Number:

E-mail Address:	hannu.hietalahti@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	23.501 CR 1934

SA2 thanks CT4 for their LS on SMSF change and gives the following answers to the CT4 questions.

- “Q1: Can the SMSF serving the UE may be changed during the inter-AMF mobility procedure?”

SA2 answer: When SMSF has been selected for the serving PLMN, there is no need to change it.

Assuming the subscription and the UE registration for SMS service require it, the AMF shall re-use the already selected SMSF for the serving PLMN if one exists. If no SMSF has been selected for the AMF already, then the AMF shall perform SMSF selection. This could occur e.g. in the following cases:

- First time registration to SMS service

- after inter-PLMN mobility, if no SMSF is already registered in the target PLMN

- after the SMSF has been de-registered

- “Q2: Whether there are other scenarios where the SMSF may be reselected?”

SA2 answer: There is no requirement for re-selecting SMSF to replace an already selected SMSF for the serving PLMN. However, inter-PLMN mobility and SMSF de-registration procedure can lead to situation where there is no selected SMSF in the current serving PLMN at all. In such case, SMSF selection is needed in order to enable SMS service.

SA2 has agreed the attached CR to TS 23.501 to clarify this via referring to already existing requirements.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 to note this answer and align their specifications with these principles.

3. Date of Next SA2 Meetings:

3GPP TSG SA2 #136ah	13th – 17th January 2020	South-Korea

3GPP TSG SA2 #137	24th – 28th February 2020	India