SA WG2 Meeting #136 	S2-1912545

18 – 22 Nov 2019, Reno, NV, USA	 (revision of S2-1912528)

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation of KI #2 conclusions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_5GSAT_ARCH / Rel-16

Abstract of the contribution: conclusions on mobility key issue #2

# Discussion

Clause 7.2 evaluates three mobility solutions as follows:

- Solution #1 is simple for the 3GPP system, but it relies on so called static TA concept.

- Solution #7 works with moving TA concept but it requires additional functionality from UE and AMF.

- Solution #9 is to large extent in the RAN area. SA2 has sent LS to RAN2 asking for their evaluation of the radio related aspects of this solution. SA2 cannot conclude on this candidate solution without the guidance form the RAN experts.

It is proposed to reflect these aspects in the conclusions part of the TR 23.737 as shown below.

## 8.2 	Conclusion on solutions for Key Issue #2

Key Issue #2 studied Mobility Management with moving satellite coverage area. 3GPP system should adapt to being usable over variety of satellite types without putting pre-requisite conditions on the capabilities of the satellites, or the satellite constellation to be used. The system impact on 3GPP system depends on what kind of satellites are used, and therefore, the following conclusions are made.

Solution #1 selected for normative work as it is simple solution to support NR over satellite in those cases where so called static TA can be provided. This solution causes no impact on SA2 specifications, but SA2 is expecting guidance from RAN working groups.

# Solution #9 has got several deeply RAN-dependent aspects for which SA2 has requested RAN2 comment. SA2 can only conclude on this solution after receiving RAN2 guidance.