3GPP TSG-SA2 Meeting #136	S2-1910925

# Reno, Nevada, United States, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1898 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF reporting the time comparison |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UPF needs to be able to report the measured time offset and the cumulative rateRatio separately to SMF. The cumulative rateRatio is the difference between clock frequencies of the 5G clock and the TSN clock. The Periodicity in IEEE parameters is specified with respect to the TSN clock, whereas the Periodicity in TSCAI is with respect to 5G clock, so the latter needs to be corrected by the frequency rate between two clocks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces Session Reporting Rule that can be used to instruct the UPF to measure and report TSC time offset and cumulative rateRatio |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSCAI Periodicity is inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.11.1, 5.8.2.11.X (NEW), 5.8.2.11.Y (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Resubmission of S2-1909783 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

##### 5.8.2.11.1	General

These parameters are used by SMF to control the functionality of the UPF as well as to inform SMF about events occurring at the UPF.

The N4 session management procedures defined in clause 4.4.1 of TS 23.502 [3] will use the relevant parameters in the same way for all N4 reference points: the N4 Session Establishment procedure as well as the N4 Session Modification procedure provide the control parameters to the UPF, the N4 Session Release procedure removes all control parameters related to an N4 session, and the N4 Session Level Reporting procedure informs the SMF about events related to the PDU Session that are detected by the UPF.

The parameters over N4 reference point provided from SMF to UPF comprises an N4 Session ID and may also contain:

-	Packet Detection Rules (PDR) that contain information to classify traffic (PDU(s)) arriving at the UPF;

-	Forwarding Action Rules (FAR) that contain information on whether forwarding, dropping or buffering is to be applied to a traffic identified by PDR(s);

-	Multi-Access Rules (MAR) that contain information on how to handle traffic steering, switching and splitting for a MA PDU Session;

-	Usage Reporting Rules (URR) contains information that defines how traffic identified by PDR(s) shall be accounted as well as how a certain measurement shall be reported;

-	QoS Enforcement Rules (QER), that contain information related to QoS enforcement of traffic identified by PDR(s);

-	Session Reporting Rules (SRR) that contain information how the session level events shall be reported. It can be used to trigger TSN time comparison reports. In this case the Session Reporting Rule indicates how to measure and report the TSN time offset and the cumulative rateRatio. The time synchronization protocol can be identified by the associated PDR(s);

-	Trace Requirements.

The N4 Session ID is assigned by the SMF and uniquely identifies an N4 session.

If the UPF indicated support of Trace, the SMF may activate a trace session during a N4 Session Establishment or a N4 Session Modification procedure. In that case it provides Trace Requirements to the UPF. The SMF may deactivate an on-going trace session using a N4 Session Modification procedure. There shall be at most one trace session activated per N4 Session at a time.

For the MA PDU Session, the SMF may add an additional access tunnel information during an N4 Session Modification procedure by updating MAR with addition of an FAR ID which refers to an FAR containing the additional access tunnel information for the MA PDU session for traffic steering in the UPF.

##### 5.8.2.11.2	N4 Session Context

N4 Session Context is identified by an N4 Session ID. An N4 Session Context is generated by SMF and UPF respectively to store the parameters related to an N4 session, including N4 session ID, all PDRs, URRs, QERs and FARs or MARs used for this N4 session.

## ---Start of the 2nd Change---

##### 5.8.2.11.X	Session Reporting Rule

The following table describes the Session Reporting Rule that can be used to instrcuct the UPF to measure and report TSC time offset and cumulative rateRatio.

Table 5.8.2.11.X-1: Attributes within Session Reporting Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this SRR |  |
| Rule ID | Unique identifier to identify this information. | Used by UPF when reporting the measurement. |
| Reporting triggers | One or multiple events can be activated for the generation and reporting of the Session level events. | Applicable events include: - Time Offset Reporting: indicates a request for reporting the offset between TSN time and 5G time. - Time RateRatio Reporting: indicates a request for reporting the cumulative rateRatio between TSN time and 5G time. NOTE 1. |
| Time offset measurement threshold | Defines the threshold to report the time offset | The offset is reported only when it exceeds the threshold compared to the previous report |
| Cumulative rateRatio measurement threshold | Defines the threshold to report the cumulative rateRatio | The cumulative rateRatio is reported only when it exceeds the threshold compared to the previous report |
| NOTE 1: cumulative rateRatio values to be reported are not the cumulative rateRatio received inside the gPTP message payload, but the new cumulative rateRatio calculated at UPF (i.e. NW-TT) as specified in subclause 5.27.1.2.2. |  |  |

## ---Start of the 3rd Change---

##### 5.8.2.11.Y	Session level Report generated by UPF

The UPF sends the Session level report to inform the SMF about the measurement, e.g. a comparison between TSC time and 5G time. For each SRR, the report may be generated repeatedly, i.e. as long as any one of the valid event triggers applies.

Following attributes can be included in the Session level report:

Table 5.8.2.11.Y-1: Attributes within Session level report

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Uniquely identifies a session. | Identifies the N4 session associated to this Report |
| Rule ID | Session Reporting Rule within a session which triggered the report. | Rule ID of the Session Reporting Rule. |
| Reporting trigger | Identifies the trigger for the Session level report. | Applicable values are: - Time Offset Reporting - Time RateRatio Reporting |

| Measurement information | Defines the content of the measurement in the report | Time Offset Report: Contains the combination of time offset between TSN time and 5G time and the TSN domain number. Several measurements of the same type may be present to report multiple TSN time domains. Cumulative rateRatio Report: Contains the combination of cumulative rateRatio between TSN time and 5G time and the TSN domain number. Several measurements of the same type may be present to report multiple TSN time domains. Details refer to TS 29.244 [65]. |
| --- | --- | --- |

## ---End of Changes---