3GPP TSG-SA WG Meeting # 136	S2-1911201

# Reno, Nevada, USA, 18-22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1829 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | removing requirement that TAC+SV is used to identify UE model in manufacturer assigned ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2019-09-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The assumption manufacturer assigned ID is assigned per TAC+SVN is leading to a huge number of IDs. There can be massive database size savings if this assumption is lisfted, as documented in S2-19xxxxx |  |  |  |  |  |  |  |  |
| Summary of change: |  | the IDs are assigned per UE Vendor. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RACS would be very demanding in terms of size of database and signaling interactions required. The cost of supporting it would be high as the RACS caches in CN and RAN will have to be bigger and the signalling interactions to populate them and resolve IDs when caches misses occorr would be very frequent.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.1a;5.9.10;6.2.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 5.4.4.1a	UE radio capability signalling optimisation (RACS)

With the increase of the size of UE radio capabilities driven e.g. by additional frequency bands and combinations thereof for E-UTRA and NR, an efficient approach to signal UE Radio Access Capability information over the radio interface and other network interfaces is defined with RACS.

In this Release of the specification, RACS does not apply to NB-IOT.

RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called UE Radio Capability ID. A UE Radio Capability ID can be either UE manufacturer assigned or PLMN-assigned, as specified in clause 5.9.10. The UE Radio Capability ID is an alternative to the signalling of the radio capabilities container over the radio interface, within NG-RAN, from NG-RAN to E-UTRAN, from AMF to NG-RAN and between CN nodes supporting RACS.

PLMN-assigned UE Radio Capability ID is assigned to the UE using the UE Configuration Update procedure, or Registration Accept as defined in TS 23.502 [3].

The UCMF (UE radio Capability Management Function) stores all UE Radio Capability ID mappings in a PLMN and is responsible for assigning every PLMN-assigned UE Radio Capability ID in this PLMN, see clause 6.2.21.

In order to be able to interpret the UE Radio Capability ID a Network Function or node may store a local copy of the mapping between the UE Radio Capability ID and its corresponding UE Radio Access Capabilities information i.e. a dictionary entry. When no mapping is available between a UE Radio Capability ID and the corresponding UE Radio Capability information in a Network Function or node, this Network Function or node shall be able to retrieve this mapping and store it.

-	An AMF which supports RACS shall store such UE Radio Capability ID mapping at least for all the UEs that it serves that have a UE Radio Capability ID assigned.

-	The NG-RAN performs local caching of the UE Radio Access Capabilities for the UE Radio Capability IDs for the UEs it is serving, and potentially for other UE Radio Capability IDs according to suitable local policies.

-	When the NG-RAN needs to retrieve the mapping of a UE Radio Capability ID to the corresponding UE Radio Capability information, it queries the AMF using N2 signalling defined in TS 38.413 [34].

-	When the AMF needs to obtain a PLMN-assigned UE Radio Capability ID for a UE from the UCMF, it provides the UE Radio Capabilities Information it has for the current radio configuration of the UE,  the IMEI/ TAC for the UE. . The UCMF stores the association of this IMEI/TACwith this UE Radio Capability ID so that when the PLMN decides to switch to Manufacturer assigned UE radio Capability ID for this TAC, the related PLMN assigned UE radio Capability IDs in the UCMF may be disposed of if these are not also continuing to be used by other UEs identified by other TAC values.

-	When the AMF needs to obtain   the UE Radio Capability Information associated to a UE Radio Capability ID it provides the UE Radio Capability ID  to the UCMF and the UCMF returns a mapping of the UE Radio Capability ID to the corresponding UE Radio Capabilities information.

A network may utilise the PLMN-assigned UE Radio Capability ID, without involving the UE, e.g. for use with legacy UEs.

Mutual detection of the support of the RACS feature happens between NG-RAN nodes at Xn setup and between NG-RAN and AMF at N2 setup time. To allow for a mix of RACS-supporting and non-RACS-supporting RAN nodes over the Xn interfaces, the UE Radio Capability ID should be included in the Path Switch signalling during Xn based handover and Handover Request during N2 based handover between AMF and NG-RAN. In addition, RACS-supporting RAN nodes can be discovered across inter-CN node boundaries e.g. using the Configuration Transfer procedure. The support of RACS by peer AMFs or MMEs is based on configuration in a PLMN or across PLMNs.

A UE that supports WB-EUTRA and/or NR indicates its support for RACS to AMF using UE MM Core Network Capability as defined in clause 5.4.4a.

A UE that supports RACS and stores an applicable UE Radio Capability ID for the current UE Radio Configuration in the PLMN, shall signal the UE Radio Capability ID in the Initial Registration procedure as defined in TS 23.502 [3]. If both PLMN-assigned for the current PLMN and UE manufacturer-assigned UE Radio Capability IDs are stored in the UE and applicable in the PLMN, the UE shall signal the PLMN-assigned UE Radio Capability ID in the Registration Request message.

When a PLMN is operating  based on manufacturer-assigned UE Radio Capability ID for a particular type of UE., the UCMF is provisioned with a dictionary of Manufacturer-assigned UE Radio Capability IDs which include a "Vendor ID" that applies to the manufacturers of these UE, and a list of TACs for which the PLMN has obtained-manufacturer-assigned UE Radio Capability IDs. The UCMF then indicates to the AMFs the TAC values of the UEs for which it has obtained Manufacturer assigned UE radio Capability IDs. Then.:

-	An AMF wich receives these TAC values, stores them in a list of TACs designated to operate by using Manufacturer-assigned UE Radio Capability ID, so it can determine which UEs shall operate in Manufacturer-assigned UE Radio Capability ID.

-	For each UE that registers or is already registered with a TAC which is included in the list of TACs designated to operate by using Manufacturer-assigned UE Radio Capability ID, the AMF indicates to these UEs to delete all the PLMN-assigned UE Radio Capability IDs if they are registering or are registerd using PLMN assigned UE radio capability ID.

-	a UE which receives an indication to delete all the PLMN-assigned UE Radio Capability IDs in the Registration Accept message, or UE Configuration Update command message, shall delete any PLMN-assigned UE Radio Capability IDs for this PLMN. The UE proceeds to register with the UE manufacturer assigned UE Radio Capability ID that is applicable to the current UE Radio configuration.

If for any reason the operator need no longer to keep a dictionary for a UE model identified by a (set of )TAC(s), e.g. because they are obsolete, or due to any other circumstance which leads to stopping the operation based on Manufacturer assigned IDs, the UCMF proceeds to indicate to the AMFs to remove the (set of) TAC(s) from the list of  TACs designated to operate by using Manufacturer-assigned UE Radio Capability ID.

The serving AMF stores the UE Radio Capability ID for a UE in the UE context and provides this UE Radio Capability ID to NG-RAN as part of the UE context information using N2 signalling. During inter PLMN mobility, the new AMF shall delete the UE Radio Capability ID received from the old AMF, unless the operator policy indicates that all UE Radio Capability IDs used in the old PLMN is also valid in the new PLMN.

The UE stores the PLMN-assigned UE Radio Capability ID in non-volatile memory when in RM-DEREGISTERED state and can use it again when it registers in the same PLMN.

NOTE 3:	It is assumed that UE does not need to store the access stratum information (i.e. UE-EUTRA-Capability and UE-NR-Capability specified in TS 36.331 [51] and TS 38.331 [28], respectively) that was indicated by the UE to the network when the PLMN-assigned UE Radio Capability ID was assigned by the network. However, it is assumed that the UE does store the related UE configuration (e.g. whether or not GERAN or UTRAN or MBMS is enabled/disabled).

At any given time at most one UE Radio Capability ID is stored in the UE context in CN and RAN.

The number of PLMN-assigned UE Radio Capability IDs that the UE stores in non-volatile memory is left up to UE implementation. However, to minimise the load (e.g. from radio signalling) on the Uu interface and to provide smoother inter-PLMN mobility (e.g. at land borders) the UE shall be able to store at least the latest 16 PLMN-assigned UE Radio Capability IDs (along with the PLMN that assigned them). This number is independent of the UE-manufacturer-assigned UE Radio Capability ID(s) the UE may store.

It shall be possible for a UE to change, e.g. upon change in its usage settings, the set of UE Radio Capabilities in time and signal the associated UE Radio Capability ID, if available. The UE stores the mapping between the UE Radio Capability ID and the corresponding UE Radio Capability information for every UE Radio Capability ID it stores.

The NG-RAN may apply RRC filtering of UE radio capabilities when it retrieves the UE Radio Capabilities information from the UE as defined in TS 38.331 [28].

NOTE 4:	In a RACS supporting PLMN, the filter of UE Radio Capabilities configured in NG-RAN is preferably as wide in scope as possible (e.g. PLMN-wide). In this case, it corresponds e.g. to the super-set of bands, band-combinations and RATs the PLMN deploys and not only to the specific NG-RAN node or region.

NOTE 5:	If the filter of UE Radio Capabilities configured in two NG-RAN nodes is different, during handover between these two nodes, it is possible that the target NG-RAN node might need to enquire the UE for its UE Radio Capability information again and trigger re-allocation of a PLMN-assigned UE Radio Capability ID leading to extra signalling. Additionally, a narrow filter might reduce the list of candidate target nodes.

If a UE supports both NB-IoT and possibly other RATs the UE handles the RACS procedures as follows:

-	Since there is no support for RACS in NB-IoT, if the UE supports RACS in non-NB-IoT RATs (i.e. for WB-EUTRA and/or NR).

-	NB-IoT specific UE Radio Capability information is handled in UE, RAN and AMF according to clause 5.4.4.1.

-	when the UE is not camping on NB-IoT, the UE provides UE radio capabilities for other RATs but not NB-IoT UE radio capabilities, as per TS 38.331 [28]. As a result the UE Radio Capability ID that is assigned by the network corresponds only to the UE Radio Capabilities of the non-NB-IoT RATs. The UE uses the UE Radio Capability IDs assigned only in Registration Update procedures performed over non-NB-IoT RATs.

Support for RACS in EPS is defined in TS 23.401 [26].

more changes

### 5.9.10	UE Radio Capability ID

The UE Radio Capability ID is a short pointer with format defined in TS 23.003 [19] that is used to uniquely identify a set of UE Radio Capabilities. The UE Radio Capability ID is assigned either by the serving PLMN or by the UE manufacturer, as follows:

-	Manufacturer-assigned: The UE Radio Capability ID may be assigned by the UE manufacturer in which case it it includes a UE manufacturer identification . In this case, the UE Radio Capability ID uniquely identifies a set of UE Radio Capabilities for a UE by this manufacturerin any PLMN.

-	PLMN-assigned: If a manufacturer-assigned UE Radio Capability ID is not used by the UE or the serving network, or it is not recognised by the serving PLMN UCMF, the UCMF may allocate UE Radio Capability IDs for the UE corresponding to different sets of UE Radio capabilities the PLMN may receive from the UE at different times. In this case, the UE Radio Capability IDs the UE receives are applicable to the serving PLMN and uniquely identify the corresponding sets of UE Radio Capabilities in this PLMN.

The type of UE Radio Capability ID (Manufacturer-assigned or PLMN-assigned) is distinguished when a UE Radio Capability ID is signalled.

more changes

### 6.2.21	UE radio Capability Management Function (UCMF)

The UCMF is used for storage of dictionary entries corresponding to either PLMN-assigned or Manufacturer-assigned UE Radio Capability IDs. An AMF may subscribe with the UCMF to obtain from the UCMF new values of UE Radio Capability ID that the UCMF assigns for the purpose of caching them locally.

Provisioning of Manufacturer-assigned UE Radio Capability ID entries in the UCMF is performed from an AF that interacts with the UCMF either directly or via the NEF (or via Network Management) using a procedure defined in TS 23.502 [3].

The UCMF also assigns the PLMN-assigned UE Radio Capability IDs values.

Each PLMN-assigned UE Radio Capability ID is also associated to the TAC of the UE model(s)  that it is related to. When an AMF requests the UCMF to assign a UE Radio Capability ID for a set of UE Radio Capabilities, it indicates the TAC of the UE that the UE Radio Capabilities are related to.

The UCMF may be provisioned with a dictionary of Manufacturer-assigned UE Radio Capability IDs which include a "Vendor ID" that applies to the Manufacturers of these UE, and a list of TACs for which the PLMN has obtained-Manufacturer-assigned UE Radio Capability IDs. The UCMF indicates by a Nucmf_UECapabilityManagement_Notify operation to all AMFs the TAC values which needs to use UE manufacturer-assigned UE Radio Capability IDs.

A PLMN-assigned UE Radio Capability IDs is kept in the UCMF storage as long as it is associated with at least a TAC value. When a TAC value is related to a UE model that is earmarked for operation based on Manufacturer assigned UE radio capability IDs, this TAC value is disassocaited in the UCMF from any PLMN assigned UE Radio Capability IDs.

When a PLMN-assigned UE Radio Capability ID no longer associated to any TAC is removed from the UCMF storage an indication may be sent to the AMFs by a Nucmf_UECapabilityManagement_Notify operation to delete this PLMN-assigned UE radio capability ID from the cache in the AMFs and the AMFs indicate this action the RAN nodes by N2 signalling.

End of changes