SA WG2 Meeting #136 	S2-1911488

18 - 22 Nov 2019, Reno, Nevada, USA	 (revision of S2-191xxxx)

Source:	CATT

Title:	Solution for ProSe communication

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This paper proposes a solution for ProSe communication to address KI#2 of TR 23.752.

1	Introduction

It was agreed at SA2#135 meeting that, the 5G ProSe communication shall take Rel-16 V2X communication over PC5 reference point as baseline to develop solution, and furthermore in KI#2:

-	For one-to-many direct communication and one-to-one direct communication differences with what is documented in TS 23.287 [5] clause 5.2 will be documented;

On the other hand, the equivalent features supported in LTE based ProSe communication (see TS 23.303) also needs to be considered.

In this contribution, the ProSe communication solution is proposed in which the differences with TS 23.287 clause 5.2 are highlighted.

2	Proposal

It is proposed to capture the following solution into TR 23.752.

FIRST CHANGE

6.X	Solution for Key Issue #2: ProSe communication

6.X.1	General Description

In order to support one-to-many direct communication and one-to-one direct communication, the mechanism defined in TS 23.287 [5] clause 5.2 is reused with the following differences:

-	ProSe communication over LTE based PC5 reference point is defined in TS 23.303 [x] clause 5.4.

-	Non-IP based PC5 communication is not supported over PC5 reference point.

-	Both IPv6 and IPv4 based ProSe communication are supported.

-	IPv4 address allocation is defined in TS 23.303 [x] clause 5.4.5.

-	Broadcast, groupcast and unicast mode communication are supported over both LTE based PC5 reference point and NR based PC5 reference point. Therefore, PC5 RAT selection needs to be performed for all the three modes.

-	The privacy requirement for V2X is not applied to ProSe communication.

6.X.2	Procedures

The "Procedures for V2X communication over PC5 reference point" defined in TS 23.287 [5] clause 6.3 is reused to support ProSe communication, and the differences are highlighted as followings.

For broadcast and groupcast mode ProSe communication:

-	The following data unit types are supported: IP and Address Resolution Protocol (see RFC 826 [y]).

-	The Group IP multicast address is used to send and receive IP data.

For unicast mode ProSe communication:

-	DHCPv4 based IP address allocation is supported.

-	Link identifier update procedure is not applied to ProSe communication.

6.X.3	Impacts on Existing Nodes and Functionality

UE impact:

-	UE is enhanced to support ProSe communication over NR based PC5 reference point.

SECOND CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[y]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

END of CHANGES