3GPP TSG-SA2 Meeting #136	S2-1912192

# Reno(NV), USA, 18th Nov 2019 - 22th Nov 2019	(revision of S2-1911776)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1941 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use of EPS/RAT fallback for VoWiFi session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a UE has an ongoing IMS voice session over non-3GPP access (either via ePDG or N3IWF) and wants to handover the session to 3GPP access, and the selected RAT where the IMS PDU session is initially transferred in NR, there is no mechanism to support EPS / RAT fallback. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that EPS / RAT fallback can be triggered by NG-RAN when a UE hands over the IMS voice session from non-3GPP access to 3GPP access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EPS / RAT fallback is not triggered when a UE hands over IMS voice session from non-3GPP access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: Documents the signalling flow in a new clause instead of modifying clause 4.13.6.1. |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 4.13.6.x	Transfer of PDU session used for IMS voice from non-3GPP access to 5GS

Figure 4.13.6.x-1: Transfer of PDU session used for IMS voice from non-3GPP access to 5GS

When the UE has an ongoing IMS voice session via non-3GPP access using ePDG or N3IWF, and the session is transferred to NG-RAN, depending on the selected RAT in 5GS (NR or E-UTRA), SMF configuration and the support of EPS/inter-RAT fallback in NG-RAN, either the IMS voice session continues over NG-RAN or EPS/inter-RAT fallback is triggered.

Steps 1 and 2 apply to either of the above cases.

1.	UE has ongoing IMS voice session via non-3GPP access using ePDG or N3IWF. UE is triggered to move to 3GPP access and camps in NG-RAN.

2.	UE initiates Handover from EPC/ePDG to 5GS as defined in clause 4.11.4.1 or Handover of a PDU Session procedure from untrusted non-3GPP to 3GPP access as in clause 4.9.2. During the PDU Session establishment to handover the PDU session used for IMS voice service (i.e. the PDU Session that includes the QoS Flow used for signalling and QoS Flow used for IMS voice) from non-3GPP access to 3GPP access. In case the UE requested PDU Session establishment to handover the PDU session used for IMS voice service from non-3GPP access is received, the SMF and/or PCF are configured whether to transfer both the QoS flow used for IMS signalling and the QoS flow used for voice or only the QoS flow used for IMS signalling. If the PDU session used for IMS voice service contains only the QoS flow used for IMS signalling, for the UE at this step the PDU session used for IMS voice service is considered successfully transferred.

NOTE: If only the QoS flow used for IMS signalling is transferred, IMS voice packets can also use this QoS flow for voice after the transfer for the duration of the IMS voice call. The SMF needs to configure the right UL and DL QoS rules and DL N4 rules.

If NG-RAN receives also a PDU Session establishment including to setup QoS flow for IMS voice, and NG-RAN is configured to support EPS fallback for IMS voice then:

3.	If NG-RAN decides to trigger EPS or inter-RAT fallback, taking into account UE capabilities, indication from AMF that "Redirection for EPS or or inter-RAT fallback for voice is possible" (received as part of initial context setup as defined in TS 38.413 [10]), network configuration (e.g. N26 availability configuration) and radio conditions. NG-RAN may initiate measurement report solicitation from the UE including E-UTRA as target.

If NG-RAN does not trigger EPS or inter-RAT fallback, then the procedure stops here and following steps are not executed.

4.	NG-RAN responds indicating rejection to setup the QoS flow for IMS voice  received in step 3 by PDU Session Response message towards the PGW-C+SMF (or H-SMF+P-GW-C via V-SMF, in case of roaming scenario) via AMF with and indication that mobility due to fallback for IMS voice is ongoing.

If EPS fallback is triggered steps 5-7 from clause 4.13.6.1 are executed. If inter-RAT Fallback for IMS voice is triggered steps 5-6 from clause 4.13.6.2 are executed.

* * * * End of Changes * * * *