SA WG2 Meeting #136	S2-1912302

Reno, NV, USA, November 18 – 22, 2019	(revision of S2-1912231)

Source:	China Mobile, China Telecom, Tencent, CATT, ZTE, OPPO, NEC

Title:	New Use Case: NWDAF-assisted Smart City

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution propose a new use case for NWDAF-assisted smart city.

1. Introduction

This use case may be related to the WT#1.1 i.e. Multiple NWDAF Instances and WT#6 i.e. Analytics exposure to applications in the FS_eNA_Ph2 Work Tasks.

Most of the use cases in eNA R16 are traditional operator services or OTT services related but not vertical industry related, such as smart city including intelligent transportation, smart industrial park, smart street lamp, smart parking, etc. As the text adopted from eNA phase2 SID below mentions.

  - New Use Cases/key issues:

    - NWDAF analytics exposure to applications for example, in Smart City applications e.g. intelligent transportation to alleviate urban traffic congestion

For example, as demonstrated in Figure 1, the 5G AI intelligence transportation system could utilize NWDAF to provide AI capabilities for smart transportation applications in a layered/distributed manner.

Figure 1: 5G AI based Intelligent Transportation System

As a 5G network AI Centre, the NWDAF are composed of AI training services and AI inference services. With AI training services, the NWDAF

-	could be deployed in a centralized manner on the cloud to implement AI model training cross-region, cross-subnet, ultra-large computing power required, with common application characteristics, etc;

-	could be also deployed in a distributed manner at the edge to implement localized AI training. In this manner, multiple NWDAF instances could be deployed in different regions, and the NWDAF instance at the edge could interact with the NWDAF instance on the cloud.

With AI Training services, The NWDAF could collect the data from 5G NFs and AFs, e.g., Transportation App Server in Smart Transportation case and 3rd AF. Based on that, data training is implemented.

With AI inference services, the NWDAF could be deployed to send the analytics to the application server of various industries in the smart city e.g. Transportation Application Server for final decision-making and execution.

In summary, to support NWDAF-assisted smart city, multiple NWDAF instances deployment, intra-PLMN/inter-region roaming scenarios, what analytics should be exposure to the applications and so on, should be taken into consideration.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-91.

# * * * * First change * * * *

5.1.X	Use Case #X: NWDAF-assisted Smart City

5.1.X.1	Description

As the text adopted from eNA phase2 SID below mentions.

  - New Use Cases/key issues:

    - NWDAF analytics exposure to applications for example, in Smart City applications e.g. intelligent transportation to alleviate urban traffic congestion

Smart city includes intelligent transportation, smart industrial park, smart street lamp, smart parking, etc. The data analytics by NWDAF could be provided to these applications. In the case of smart transportation system, the NWDAF could collect the network data from 5G NFs, and the service data from AFs, e.g., Transportation Application and even the 3rd Application Functions, which can provide pedestrian or vehicle movement patterns and traffic information (e.g., vehicle queue length). Based on that, the NWDAF can implement AI data model.

With ongoing network data and service data, the NWDAF could send the data analytics to Transportation Application Platform. By using the data analytics, the Transportation Application Platform can perform final decision-making and execution to alleviate urban traffic congestion.

Therefore, this use case may need considerations in this study, e.g., what kind of data needs to be collected, what analytics should be exposed to the AFs, and even any potential analytics framework enhancement.

# * * * * End of changes * * * *