SA WG2 Meeting #136	S2-1912659

18 - 22 November 2019, Reno, US	revision of S2-1912626,12634

Source:	China Telecom, Samsung, Orange, China Mobile, ZTE,CATT, China Unicom,AT&T

Title:	FS_eNA key issue: NWDAF-assisted RFSP policy

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA / Rel-17

Abstract of the contribution: This contribution proposes a new key issue to study solutions to support RAT /frequency selection by adjusting RFSP policy based on assistance analytics information from NWDAF.

Discussion

This new key issue is to address the use case #1 NWDAF-assisted RAT/frequency selection described in TR 23.700-91.

Proposal

Proposal: It is proposed to add a new key issue about supporting RAT/frequency selection by adjusting RFSP policy based on assistance analytics information from NWDAF.

The following changes are proposed for FS_eNA TR 23.700-91.

* * * * Start of Changes * * * *

5.2.X	Key issue #X: < NWDAF-assisted RFSP policy >

5.2.X.1	Description

This key issue is to address the use case #1 NWDAF-assisted RAT/frequency selection described in TR 23.700-91. This key issue describes analytics information that may be provided by NWDAF to support NFs to assist on RAT and frequency selection.

In this key issue, the following mechanisms need to be studied:

- What kind of information is made available to the NWDAF for the data analytics (e.g., the information collected for assisting RFSP index) and from which provided NFs?

- Which kind of information should the NWDAF output to the NWDAF consumer  as the data analytics.

* * * * End of Changes * * * *