3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2001854

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0116 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the monitoring period as an event filter information for data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In R16, there is a requirement that the NWDAF need to collect data on a monitoring period [start time … end time] to generate statistics or predictions matching to the analytics target period. However, there is no explict description either in TS 23.288 or in TS 23.502 that the monitorig period [start time … end time] should be indicated to NF(s) by the NWDAF when to collect data on a time period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the monitoring period as an event filter information when NWDAF using NF event exposure services to collect data on a specific time period. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear that the NWDAF can collect data on a monitoring period [start time … end time]. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.5, 6.7.2.2, 6.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.2.5	Usage of Exposure framework by the NWDAF for Data Collection

The NWDAF shall subscribe (and unsubscribe) to the Event exposure service from NF(s) reusing the framework defined in TS 23.502 [3] clause 4.15. This framework supports the possibility for the NWDAF to indicate / request:

-	Events-ID: one or multiple Event ID(s) defined in TS 23.502 [3] clause 4.15.1

-	Target of Event Reporting defined in TS 23.502 [3] clause 4.15.1: the objects targeted by the Events. Within a subscription, all Event ID(s) are associated with the same target of event reporting. In the case of NWDAF, the objects can be UE(s), UE group(s), any UE.

-	Event Filter Information defined in TS 23.502 [3] clause 4.15.1. This provides Event Parameter Types and Event Parameter Value(s) to be matched against. The NWDAF can include the monitoring period [start time ... end time] as an event filter information, indicating the the NF(s) to monitor the event during the period [start time... end time] and send the notification to the NWDAF.

-	A Notification Target Address and a Notification Correlation ID as defined in TS 23.502 [3] clause 4.15.1, allowing the NWDAF to correlate notifications received from the NF with this subscription.

-	Event Reporting Information described in TS 23.502 [3] Table 4.15.1-1.

-	Expiry time as defined in TS 23.502 [3] clause 4.15.1.

The notifications from NFs/AFs contain on top of the Event being reported (and of dedicated information being reported for this event):

-	the Notification Correlation Information provided by the NWDAF in its request,

-	(when applicable to the event) the Target Id e.g. UE ID (SUPI and if available GPSI), and

-	a time stamp.

# * * * * Third changes * * * *

6.7.2.2	Input Data

The NWDAF supporting data analytics on UE mobility shall be able to collect UE mobility information from OAM, 5GC and AFs. The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is UE location carried in MDT data;

-	Network data related to UE mobility from 5GC is UE location information as defined in the Table 6.7.2.2-1;

Table 6.7.2.2-1: UE Mobility information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters within a monitoring period [start time..end time]. |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar mobility behavior. The UE whose mobility behavior is unlike other UEs with the same TAC may be an abnormal one. |
| Frequent mobility re-registration | AMF | A UE (e.g. a stationary UE) may re-select between neighbour cells due to radio coverage fluctuations. This may lead to multiple re-registrations if the cells belong to different registration areas. The number of re-registrations N within a monitoring period M provided by NWDAF may be an indication for abnormal ping-pong behaviour, where N and M are operator's configurable parameters. |

-	Service data related to UE mobility provided by AFs is defined in the Table 6.7.2.2-2;

Table 6.7.2.2-2: Service Data from AF related to UE mobility

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Application ID | Identifying the application providing this information |
| UE trajectory (1..max) | Timestamped UE positions |
| >UE location | Geographical area that the UE enters |
| >Timestamp | A time stamp when UE enters this area |

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE location information).

# * * * * Forth changes * * * *

6.7.2.4	Procedures

The NWDAF can provide UE mobility related analytics, in the form of statistics or predictions or both, to another NF. If the NF is an AF, and when the AF is untrusted, the AF will request analytics via the NEF, and the NEF will then convey the request to NWDAF.

Figure 6.7.2.4-1: UE mobility analytics provided to an NF

1.	The NF sends a request to the NWDAF for analytics on a specific UE or a group of UEs, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The NF can request statistics or predictions or both. The type of analytics is set to UE mobility information. The NF provides the UE id or Internal Group ID in the Target of Analytics Reporting.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to events with all the serving AMFs for information as defined in table 6.7.2.2-1, including a monitoring period [start time ... end time] as an event filter information in the event exposure subscription. This step may be skipped when e.g. the NWDAF already has the requested analytics available.

NOTE:	The NWDAF determines the AMF serving the UE or the group of UEs as described in clause 6.2.2.1.

3.	The NWDAF derives requested analytics.

4.	The NWDAF provide requested UE mobility analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. The details for UE mobility analytics provided by NWDAF are defined in clause 6.7.2.3.

5-6. If at step 1, the NF has subscribed to receive notifications for UE mobility analytics, after receiving event notification from the AMFs subscribed by NWDAF in step 2, the NWDAF may generate new analytics and provide them to the NF.

# * * * * End of changes * * * *