3GPP TSG-WG SA2 Meeting #137E e-meeting  	S2-2002173

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2138 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routing binding indication correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Since S2#136 meeting the routing binding indication has been introduced to separate the binding indication generated by the NF consumer. So the binding indication in clause 4.17.11 need be changed to the routing binding indication. For the 1st time service request, the routing binding indication has not been received from the NF service producer. However as the NF service consumer has done the service discovery the related binding information can be filled. The only uncertain information is the binding level. As the binding indication can be updated in the response, it is proposed that the based on the implementation the NF service consumer can generate the routing binding indication locally. Thus if the bindng information is unsuitable the NF service producer just update the binding information later. This can assure that if need, the SCP can reselect another possbile NF producer service instance based on the routing binding indication. Also it is clarified that for the following service request, the message still go via the SCP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the binding indication to routing binding indication - Add a Note to clarify that for the 1st time service request the routing binding indication can be generated by the consumer locally. - Clarify that the subsequent message for indirect communication is routed via SCP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Wrong terminology is used for routing. - The SCP need return the error to the NF service consumer even other NF service instance is available. - Unclear how the subsequent message is routed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.1, 5.2.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.17.11	Indirect Communication without delegated discovery Procedure

This clause provides the call flow for indirect communication model without delegated discovery.

Figure 4.17.11-1: Procedure for Indirect Communication without delegated discovery

The NF/NF service discovery procedure is defined in clauses 4.17.4 and 4.17.5. In a successful discovery the NF service consumer gets the NF profile(s) matching the search criteria provided in the Nnrf_NFDiscovery_Request message.

1.	When the NF Service Consumer needs to send a Service Request and has stored results from the Discovery Procedure, the NF Service Consumer selects an appropriate NF Producer / NF Service Producer instance from the list of NF profiles provided by the NRF. The NF Service Consumer considers the NF and NF service parameters (e.g. TAI, S-NSSAI, locality, priority etc) in the NF profiles. The NF Service consumer requests service from the NF Service producer by sending a service request message to the NF service producer via the SCP and the NF Service Consumer may provide a Routing Binding Indication.

NOTE:	Before the Routing Binding Indication received from the NF Service producer, based on the implementation the NF Service Consumer can use the local generated Routing Binding Indication, which can assist the SCP to (re-)select the NF Service producer if needed.

2.	If the Routing Binding Indication is provided by the NF Service Consumer, SCP (re-)selects as specified in Table 6.3.1.0-1 of TS 23.501 [2] and routes the service request to target accordingly. If the Routing Binding Indication is not provided by the NF Service Consumer, then the SCP routes the service request based on routing information available.

3.	The NF Service Producer responds via SCP.

4.	SCP forwards the response.

On a subsequent operation on the created resource, the NF service consumer sends the Service Request via the SCP to the resource address returned by the NF service producer.

# * * * * End of changes * * * *