SA WG2 Meeting #S2-137E	S2-2002287

24 - 27 Feb, 2020, Elbonia 	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.216 | CR | 0367 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SRVCC operation impossible indication for 5G emergency SRVCC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SRVCC |  |  |  |  |  | Date: |  |  | 2020-02-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the last meeting, the 23.216 CR#0385 (S2-2001112) is approved to introduce the 5G SRVCC for IMS emergency session. In the 4G, SRVCC for IMS emergency session can be supported regardless of the subscription data from the HPLMN. If the MME detects IMS emergency session event (e.g., emergency APN and associated QCI-1 bearer setup), MME shall update the E-UTRAN that SRVCC is possible by including "SRVCC operation possible" indication via S1 AP UE CONTEXT MODIFICATION REQUEST, if needed. In the 5GS, it proposes to make alignment with 4G/EPS. AMF set "SRVCC operation possible" after the Emergency PDU session establishment, if needed, even there is no STN-SR in the subscription. After Emergency PDU session is released, the AMF shall restore the SRVCC indication to the same status as prior to the emergency PDU session was established. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AMF set "SRVCC operation possible" after the Emergency PDU session establishment, if needed, even there is no STN-SR in the subscription. After Emergency PDU session is released, the AMF shall restore the SRVCC indication to the same status as prior to the emergency PDU session was established. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If there is no 5G SRVCC subscrition in the HPLMN, the 5G SRVCC for IMS emergency session can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.5.X	Emergency PDU session establishment and release procedure for SRVCC

The Emergency PDU session is established as defined in TS 23.502 [45] with the following additions:

-	AMF includes a "5G-SRVCC operation possible" indication in the N2 Request, if needed.

The Emergency PDU session is released as defined in TS 23.502 [45] with the following additions:

-	AMF shall restore the SRVCC indication to the same status as prior to the emergency PDU session was established..

END OF CHANGES